# The Boundary Scan Test (BST) technology

Approved as an IEEE standard since February of 1990, under the title of “A standard boundary scan architecture and test access port”, the BST technology met an increasing acceptance in a broad spectrum of sectors related to digital electronics design and manufacturing. A wide variety of BST components is now available, together with tools that cover a wide range of activities, from the design of the BST infrastructure to the automatic test vector generation for boards with BST components.

This chapter will start by presenting the reasons that led to the BST technology and proceed to identify its application domain. The BS architecture and test access port (TAP) will then be presented with enough detail to enable the reader to accomplish the objective proposed in the hands-on session recommended for this chapter: The implementation of a BS architecture in a medium-complexity programmable logic device.

## The development of BST and its application domain

Two main reasons led during the 80s to the development of the BST technology:

- The increasing complexity of integrated circuits (ICs) made it exceedingly difficult to develop test programs for the functional test of complex printed circuit boards (PCBs), requiring extensive access to internal control and observation points (functional test equipment relies heavily on access through edge connectors and therefore became increasingly inefficient in these cases)

- The shrinking effect of using small outline surface mount devices and advanced mounting technologies almost disabled physical access to internal PCB nodes (in-circuit test equipment relies on the use of bed-of-nail fixtures to provide direct contact to internal PCB nodes, but is limited in terms of mechanical precision and reliability)

In-circuit and functional automatic test equipment (ATE) were traditionally used in combination to overcome each other’s limitations, but the packaging and mounting technologies that emerged in the late 80s made it clear that the design and manufacturing of advanced digital electronics modules was about to face a major bottleneck. Unless an alternative test technology was developed to overcome the challenge of testing new circuits and electronic assemblies, the complexity of the problems at hand was guaranteed to leave all traditional ad hoc and in-house structured test methodologies way behind.

An international group of companies was set up in 1985 to develop a proposal for a new standard (design for) test technology able to gather a wide support in the design, test and manufacturing worlds. This group was known as JETAG (Joint European Test Action Group) but rapidly incorporated US companies and dropped the E, leading to an IEEE working group that managed to have an approved standard in five years time. Due to the very nature of the problem that led to its development, the BST technology, such as defined in the IEEE 1149.1 std, addresses a very specific domain: the structural test of digital printed circuit boards (note the keywords structural, digital and PCB). These three attributes were sufficiently narrow to enable a quick development of solutions taking place immediately in the early 90s, but at the same time they somehow limited the application domain of a technology which may still live to see a much broader scope.

## The BS architecture and test access port (TAP)

The basic concept underlying the BST technology is shown in figure 2.1 and consists of associating a dedicated test cell to each functional pin. These test cells are the Boundary Scan cells and their main function consists of decoupling the internal logic from the IC pins, allowing the application of the required logic values at the cell outputs, regardless of the values present at their inputs. It therefore becomes possible to:

- Apply any combination to the IC output pins and to the IC internal logic inputs (through the parallel output of the BS cells).

- Capture the values present at the IC input pins and at the internal logic outputs (through the parallel input of the BS cells).

The serial input of each cell is connected to the serial output of the previous cell within each IC, forming an internal chain known as the Boundary Scan Register. Since all BS registers may be daisy-chained at PCB level, as suggested in figure 2.1, direct access to any BS pin is now possible, without requiring physical access to internal PCB nodes.

Figure 2.1: The basic concept underlying the BST technology.

Figure 2.1 also suggests that additional resources are required in each IC to control the operating mode of the BS cells, leading to a dedicated set of test pins known as the Test Access Port (TAP). This TAP includes 4 pins with the following characteristics, as defined in the IEEE 1149.1 std:

- TDI (Test Data Input): The serial data input to the BS register. When floating, this pin produces a response identical to the application of a logic 1.

- TDO (Test Data Output): The serial data output of the BS register. This is the only output pin in the TAP and must be in the high-impedance state except when a scan operation is in progress.

- TCK (Test Clock): The clock signal for the test logic.

- TMS (Test Mode Select): A control input that defines the operating mode required for the test logic. When floating, this pin produces a response identical to the application of a logic 1.

Optionally, a fifth pin may be present for initialisation purposes (the /TRST  Test Reset, active low  pin). The complete BS architecture may in fact be represented as shown in figure 2.2, which brings into evidence the following main features:

- The TDI-TDO path goes through two multiplexers, a n:1 “Data multiplexer” and a 2:1 “Data / Instruction multiplexer”.

- The 2:1 data / instruction multiplexer is controlled by an output of the “TAP controller” block (which is a small finite state machine controlled by the external input TMS) and selects the TDI-TDO path through the “Instruction register” or one of the “Data registers” (one of which is the BS register).

- The n:1 data multiplexer is controlled by the contents of the instruction register (through a decoder block called the “Instruction decoder”) and selects which of the data registers is to be routed to the TDO output, through the data  instruction multiplexer.

Figure 2.2: The BS architecture at IC level.

Access to this BS architecture is controlled by the sequence of logic values applied to the TMS input and may be summarised as follows:

- The TAP controller output connected to the data / instruction multiplexer places the instruction register in the TDI-TDO path.

- The proper bit sequence (instruction) is shifted in, setting the contents of the instruction register with the code that selects the required data register (by controlling the n:1 data multiplexer).

- The TAP controller output connected to the data / instruction multiplexer places the selected data register in the TDI-TDO path.

- Test vectors may now be shifted in and out through the selected test data register.

However, and before a block-level analysis of the architecture shown in figure 2.2 takes place, the basic functional requirements that must be satisfied by the BS cells will be detailed in the following section.

### The basic boundary scan cell

The typical BS cell configuration is represented in figure 2.3 and shows that:

- The output multiplexer (controlled by the Mode signal) enables a transparent mode of operation in which the parallel output is directly connected to the parallel input of the cell.

- The Mode signal controlling the output multiplexer, if set to 1 (Test mode), enables a controllability mode of operation, in which the parallel output takes the value present at the output latch, regardless of the value present at the parallel input.

- The Shift signal controlling the input multiplexer, if set to 0, enables an observability mode of operation, in which the value present at the parallel input is sampled into the left flip-flop and may be shifted out (by setting the Shift signal to 1).

Figure 2.3: Typical BS cell configuration.

These three basic operating modes of the BS cells enable test operations to proceed according to a simple protocol which consists of the following steps:

- Shift in a new test vector (left multiplexer in Shift mode, right multiplexer in either Normal or Test modes)

- Apply the test vector at the parallel outputs of the cells (left multiplexer in either Shift or Capture modes, right multiplexer in Test mode)

- Capture the responses present at the parallel inputs of the cells (left multiplexer in Capture mode, right multiplexer in either Normal or Test modes)

- Shift out the responses (left multiplexer in Shift mode, right multiplexer in either Normal or Test modes)

Notice that the first and last steps described above may, and do in fact, take place at the same time, since the process of shifting in a new test vector may proceed simultaneously as the responses to the previous vector are shifted out. Notice also that the sequence of steps presented above is the same either for testing interconnects at PCB level or for testing the core logic in a BS IC. However, and since the contents of the selected scan chain have to be shifted in / out for each test vector, this test protocol becomes very slow for large numbers of test vectors. Low speed wouldn’t be acceptable for testing the internal logic of an IC, which may require hundreds of thousands of vectors, but it presents no problems as far as the main application domain of BST is concerned: the structural testing of digital PCBs. As we will see later when considering the detection of open and short-circuit faults in PCB interconnects, only a reduced number of test vectors is required to enable complete fault detection.

### The test data registers

Figure 2.2 shows that the n:1 data multiplexer enables the choice of which data register to place between TDI and TDO, according to the instruction currently present in the instruction register. From none to several “User data registers” may exist, bringing the total number of data registers to a minimum of three and to a maximum that depends only on the testability infrastructures which the designers cared to insert. A brief description of each of the data registers shown in figure 2.2 may be presented as follows:

- The BS register comprises the set of BS cells present in the circuit and is mandatory in any BS IC. The main function of this register is to apply test vectors and capture test responses, essentially at PCB-level (and eventually also at IC level, as was previously referred). At least two instructions selecting this register have to be supported (EXTEST and SAMPLE / PRELOAD).

- The bypass register is another mandatory register defined in the IEEE 1149.1 std and its function is to shorten the total length of the serial PCB-level chain, when the scan operation addresses only a subset of the ICs present. This register is 1-bit long and is selected by the BYPASS instruction, which is also always supported.

- The identification register is optional and its function is to provide a 32-bit sequence enabling the test engineer to perform an identity check on each device supporting the IDCODE instruction.

- The user test data registers are also optional and will normally interface additional testability infrastructures introduced by the designers. An example could be the register storing the result of BIST (Built-In Self-Test) operations.

There is a close relationship between the data registers and the instructions supported, since each instruction selects one data register (and a particular operating mode, if more than one instruction select the same register).

### The instruction register

The IEEE 1149.1 std defines mandatory and optional instructions, respectively addressing (or selecting) the mandatory and optional data registers shown in the BS architecture depicted in figure 2.2. The mandatory instructions are those guaranteeing the basic objectives underlying the development of the BS test technology and may be briefly described as follows:

- The EXTEST instruction selects the BS register and imposes the (external) test mode in each BS cell, decoupling the IC core logic from the pins. The configuration shown in figure 2.3 for a BS cell will however enable internal tests to be carried out as well, since the BS cells associated to input pins may be used to apply test vectors and the BS cells associated to output pins used to capture the responses. The EXTEST instruction has a pre-defined code of all-0s.

- The SAMPLE / PRELOAD instruction also selects the BS register, but the BS cells are now in transparent mode. The main objective of this instruction is to enable a first vector to be shifted in before the internal logic is decoupled from the pins (while the BS cell output latches are not loaded with meaningful data, it may be necessary to prevent their initial value to appear on the parallel output of the cell). This instruction does not have a pre-defined code.

- The BYPASS instruction selects the bypass register and enables a 1-bit data path through those ICs which do not play a role in the current test operation. This instruction has an all-1 code and is automatically loaded into the instruction register on reset (the BS register will in this case have its cells in transparent mode, meaning that at power-up all BS ICs will start to perform their normal functions).

A number of optional BS instructions are possible with the objective of supporting other test operations or the optional data registers shown in figure 2.2. The description of these instructions will be present in the device datasheet, but the IEEE 1149.1 std defines a few possible instructions, including the following:

- The IDCODE selects the identification register and enables access to a unique 32-bit code identifying the device manufacturer, the part number and the version number.

- User-programmable components may also support an USERCODE instruction that selects the same 32-bit identification register, now with the objective of enabling access to a user-programmable identification code.

- The RUNBIST instruction is used to enter a self-test mode that leaves the test result in the data register selected by this instruction (a dedicated user test data register will normally be used for this purpose).

None of the optional instructions have pre-defined codes. The optional instructions to support are naturally left to the discretion of the designer and it is possible that a BS component supporting several optional instructions does not support those referred above (such as is the case with the SCOPE octals manufactured by Texas Instruments).

### The TAP controller

The block called TAP controller in figure 2.2 is a small finite state machine (FSM) that generates the control signals defining the operating mode of the existing registers and of the data / instruction multiplexer (the 2:1 multiplexer that selects between the instruction register or a data register). This FSM is controlled by the TAP pin called TMS and enables three main operations on the register placed between TDI and TDO:

- To capture the logic value present at the parallel input of its cells.

- To shift data serially through the register cells.

- To update the cell parallel outputs with the values that were shifted in.

Each of these three operations may take place on a data register or on the instruction register, leading to two parallel and similar paths in the TAP controller state transition diagram, shown in figure 2.4:

Figure 2.4: The TAP controller state transition diagram.

The IR (instruction register) branch is the right one and the DR (data register) branch is at its left. The three operations previously referred correspond respectively to the Capture, Shift and Update states (-DR and -IR). The Test Logic Reset (TLR) and the Run Test / Idle (RTI) are also important states, with the following objectives:

- The TAP controller is forced into the TLR following power-up. In this state the BS logic has no effect at functional level, since all BS cells are in transparent mode.

- RTI is a state where certain test operations defined by optional instructions (such as BIST) will take place.

The Select, Exit1 and Exit2 states (-DR and –IR) are temporary states. The last two, combined with the Pause (-DR and –IR) state, allow shifting of test data to be temporarily halted. The Select (-DR and –IR) states allow the selection of the type of register (-DR and –IR) to be connected between TDI and TDO (since there is only one TMS input, an additional state is required to distinguish if a DR or IR operation is to follow after RTI).

It is important to understand the timing of the actions associated with the TAP controller FSM, namely that:

- State transitions occur with the rising edge in the TCK signal.

- Actions in a TAP controller state (such as capture, shift or update operations) occur with the falling edge of TCK in the state.

These requirements aim at guaranteeing correct operation even with BS chains including thousands of cells. Notice in particular that TDO changes at the falling edge of TCK, while TDI is read at its rising edge (both operations occur during any shift operation in the BS cells).

2.	The Boundary Scan Test (BST) technology

2.1	The development of BST and its application domain	2.1

2.2	The BS architecture and test access port (TAP)	2.2

2.1.1	The basic boundary scan cell	2.5

2.1.2	The test data registers	2.6

2.1.3	The instruction register	2.7

2.1.4	The TAP controller	2.9

Hands-on:	Implementing an 1149.1 BST infrastructure on a Mach-series programmable logic device