# 5.h-o 1. Exhaustive testing of non-BS clusters

This document addresses one of the alternatives available to test non-BS clusters present in partial BS boards. The selected approach consists of using the surrounding BS infrastructure for cluster testing, by shifting in each cluster test vector and analysing each response shifted out.

The cluster to be used as an example is the 74LS139 + 74LS04 group in our demonstration board, where all the BS cells surrounding this cluster belong to the same BS chain. Each test vector will be applied through the BS cells connected to the four inputs of the cluster (A, B, /1G and /2G) and the corresponding response values will be captured at the BS cells connected to its eight outputs (/1Y0, /1Y1, /1Y2, /1Y3, 2Y0, 2Y1, 2Y2 e 2Y3). Instead of trying to find out what will be the minimum test vector set for maximum fault coverage, and considering that the total number of test vectors is very small (only 16), we will simply perform an exhaustive test of this cluster.

The connections between the cluster to be tested and the surrounding BS infrastructure are shown in figure 5h-o1.1:

Figure 5h-o1.1: Cluster under test and surrounding BST infrastructure.

The application of every possible combination to the four cluster inputs will produce at the outputs (of a fault-free cluster) the test vector responses shown in table 5h-o1.1:

| Order of the BST cells in the complete BST chain: |  |  |  |
| --- | --- | --- | --- |
| 10 11 12 13 | 20 21 22 23 24 25 26 27 |  |  |
| A B /1G /2G | /1Y0 /1Y1 /1Y2 /1Y3 2Y0 2Y1 2Y2 2Y3 | Hex. |  |
| 0 0 0 0 | 0 1 1 1 1 0 0 0 | 78 |  |
| 0 0 0 1 | 0 1 1 1 0 0 0 0 | 70 |  |
| 0 0 1 0 | 1 1 1 1 1 0 0 0 | F8 |  |
| 0 0 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 0 1 0 0 | 1 1 0 1 0 0 1 0 | D2 |  |
| 0 1 0 1 | 1 1 0 1 0 0 0 0 | D0 |  |
| 0 1 1 0 | 1 1 1 1 0 0 1 0 | F2 |  |
| 0 1 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 1 0 0 0 | 1 0 1 1 0 1 0 0 | B4 |  |
| 1 0 0 1 | 1 0 1 1 0 0 0 0 | B0 |  |
| 1 0 1 0 | 1 1 1 1 0 1 0 0 | F4 |  |
| 1 0 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 1 1 0 0 | 1 1 1 0 0 0 0 1 | E1 |  |
| 1 1 0 1 | 1 1 1 0 0 0 0 0 | E0 |  |
| 1 1 1 0 | 1 1 1 1 0 0 0 1 | F1 |  |
| 1 1 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |

Table 5h-o1.1: Exhaustive test vector set and expected responses.

Once the information present in figures 5.h-o1.1 and 5.h-o1.2 is obtained, the next step consists of determining the operand of the NSHFCP instructions that will shift in each new test vector and shift out the responses to the previous test vector. In order to implement this step we also need to know what values should be sent to those BS cells which are not directly involved in test vector application and response capturing. This task is simplified if we start by considering their relative position in the chain, as shown in figure 5h-o1.2:

Figure 5h-o1.2: Location of those cells involved in test vector application and response capturing.

In what concerns those BS cells not directly involved in the test of this cluster, we will take the following decisions:

- BS cell 0 will receive a 0, since otherwise those pins connected to cells 10 to 13 would be in high-impedance and test vector application would not take place. BS cell 1 plays the same role for those pins connected to cells 14 to 17; although these pins are not used to test this cluster, cell 1 will also receive a 0.

- The values shifted in to BS cells 2 to 9 are applied to the internal logic of the first BS component (on the left side in figure 5.h-o1.1). Since the internal test of this component is not a requirement, we will simply shift a 0 to each of these cells.

- BS cells 14 to 17, which are also not directly involved in the cluster test operation, will also receive a 0.

- BS cells 18 e 19, as well as cells 28 to 35 (corresponding to the output pins of the second BS component), will for the same reasons also receive a 0.

The remarks presented so far lead us to the conclusion that the procedure to determine the operand of each NSHFCP instruction may be defined as follows (except for the first vector, which does not include the information concerning the responses to the previous vector and therefore has a simpler construction rule):

- The values to be shifted into the BS chain are obtained by inserting each 4-bit combination, corresponding to each test vector to be applied, in those bit positions indicated in figure 5h-o1.3 (e.g. this part of the operand for the second NSHFCP instruction will be $00,..,..,$00,..,..,$40,..,..,$00,..,..,$00,..,..).

Figure 5h-o1.3: Construction rule for the serial test vectors.

- The expected responses are obtained by inserting the expected output combination for the previous vector in those bit positions indicated in figure 5h-o1.4 (e.g. this part of the operand for the second NSHFCP instruction will be ..,$00,..,..,$78,..,..,$00,..,..,$00,..,..,$00,..).

Figure 5h-o1.4: Construction rule to insert the expected response information.

- The comparison masks part of each NSHFCP instruction has a construction rule identical to the one shown in figure 5.h-o1.4, which consists of inserting a 1 in each bit position belonging to the expected responses group (e. g. this part of the operand for the second NSHFCP instruction will be ..,$00,..,..,$FF,..,.., $00,..,.., $00,..,..,$00,..).

The set of NSHFCP instructions for testing the non-BST cluster illustrated in figure 5.h-o1.1 will therefore be represented as follows:

NSHF	$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$78,$FF,$40,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$70,$FF,$80,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F8,$FF,$C0,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$D2,$FF,$40,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$D0,$FF,$80,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F2,$FF,$C0,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$B4,$FF,$40,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$B0,$FF,$80,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F4,$FF,$C0,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$E1,$FF,$40,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$E0,$FF,$80,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F1,$FF,$C0,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$C0,$00,$00,$03,$00,$00,$00,$00,$00

It must be recalled that the TAP controller in each BS component has to be taken to the Shift-DR state before the first shifting operation takes place:

NSHF	$00,$00,$00,$00,$00

TMS1	; go to Update-DR

TMS1	; go to Sel.-DR

TMS0	; go to Capt.-DR

TMS0	; go to Shift-DR

From this point on, the following instruction sequence will be repeated for each new vector:

NHSFCP	$00,$00,$00,$00,$78,$FF,$40,$00,$00,$00,$00,$00,$00,$00,$00,

TMS1	; go to Update-DR

TMS1	; go to Sel.-DR

TMS0	; go to Capt.-DR

TMS0	; go to Shift-DR

.. etc.

As a concluding exercise, the reader is invited to try to find out if some of the faults associated with this cluster may escape detection in the case of the (exhaustive) test vector set presented and if so what measures could be taken to improve fault detection.