# 2.h-o	Implementing an 1149.1 BS architecture on a Mach-series programmable logic device

This document describes the implementation of a basic BS architecture in a medium-complexity programmable logic device. This basic architecture will only support the mandatory instructions (EXTEST, SAMPLE / PRELOAD and BYPASS) defined in the IEEE 1149.1 standard and should be seen as a starting point, from where it is possible to implement optional instructions leading to additional modes of operation. The required BS architecture is represented in figure 2.h-o.1 and it is assumed that all flip-flops receive the same clock signal (a free-running clock).

Figure 2.h-o.1: BS architecture to be implemented.

The three mandatory BS instructions and their respective opcodes are shown in table 2.h-o.1:

| IR[1] | IR[0] | Instruction |
| --- | --- | --- |
| 0 | 0 | EXTEST |
| 0 | 1 | SAMPLE / PRELOAD |
| 1 | 0 | - |
| 1 | 1 | BYPASS |

Table 2.h-o.1: BS instructions and opcodes.

The first step towards the implementation of this architecture is to build the corresponding PALASM specification. This specification (a file that we may call BST.PDS) will comprise three main sections, starting with the declaration section which may be described as follows:

;PALASM Design Description

;---------------------------------- Declaration Segment ------------

TITLE    BST.PDS

PATTERN  BST infrastructure for experimental purposes

REVISION 1

AUTHOR   JMF

COMPANY  FEUP/DEEC

DATE     Outubro 1997

CHIP  BST MACH210

PIN 35 TCK                          ; CLOCK INPUT

PIN ? RESET                         ; INPUT - INTERNAL RESET

NODE 1 GLOBAL

PIN ? TDI

PIN ? TDO

PIN ? TMS

PIN ? BSIN[1,0]

PIN ? BSOUT[3,2]

NODE ? INTRST

NODE ? BSOUT[1,0]

NODE ? BSIN[3,2]

NODE ? BSFF1[3..0]

NODE ? BSFF2[3..0]

NODE ? BSSHF

NODE ? BSCAP

NODE ? BSUPD

NODE ? MODE

NODE ? IRFF1[1,0]

NODE ? IRFF2[1,0]

NODE ? IRSHF

NODE ? IRCAP

NODE ? IRUPD

NODE ? BPSHF

NODE ? BPCAP

NODE ? BPOUT

NODE ? DTMUXCTR

NODE ? MUXCTR

NODE ? DROUT

NODE ? DRSHF

NODE ? DRCAP

NODE ? TDOACT

The TAP controller is a 16-state finite state machine (a Moore-type FSM) and will be the first block to be implemented. This block has one external input (TMS, an input pin) and nine (internal) outputs (these outputs are internal control signals to the remaining BS architecture blocks). Recall that the combinational block of this TAP controller FSM still has four additional inputs and four additional outputs, corresponding to the four state flip-flops. The name and function of each of the nine outputs may be presented as follows:

- BSUPD: Boundary scan update, to update the output stage of the BS cells.

- INTRST: Internal reset, to force a synchronous reset of all register flip-flops, when the TAP controller is in the Test Logic Reset state.

- IRSHF: Instruction register shift and …

- IRCAP: Instruction register capture. These two signals indicate when the shift stage in each instruction register cell is to hold the present state, to shift or to capture (the two least significant bits must capture 01 as defined in the IEEE 1149.1 standard).

- IRUPD: Instruction register update, to update the output stage of the instruction register cells.

- DRSHF: Data register shift and …

- DRCAP: Data register capture. These two signals indicate when the shift stage in each BS register cell is to hold the present state, to shift or to capture (the shift and capture control signals to each of the two data registers  BS and BP  will take place according to the instruction stored in the IR).

- MUXCTR: Controls the data / instruction register multiplexer (selects if the internal TDI-TDO path goes through the instruction register or through a data register).

- TDOACT: Controls the tri-state output buffer in the TDO pin (TDO will be in high-impedance in all but the SHIFT-IR and the SHIFT-DR states).

The PALASM description of the TAP controller may be presented as follows:

;---------------------------------- State Segment ------------

STATE

MOORE_MACHINE

START_UP := POWER_UP -> TLR

; TRANSITION EQUATIONS

TLR     := TMS1 -> TLR

+-> RTI

RTI     := TMS1 -> SELDR

+-> RTI

SELDR   := TMS1 -> SELIR

+-> CAPDR

CAPDR   := TMS1 -> EX1DR

+-> SHFDR

SHFDR   := TMS1 -> EX1DR

+-> SHFDR

EX1DR   := TMS1 -> UPDDR

+-> PAUDR

PAUDR   := TMS1 -> EX2DR

+-> PAUDR

EX2DR   := TMS1 -> UPDDR

+-> SHFDR

UPDDR   := TMS1 -> SELDR

+-> RTI

SELIR   := TMS1 -> TLR

+-> CAPIR

CAPIR   := TMS1 -> EX1IR

+-> SHFIR

SHFIR   := TMS1 -> EX1IR

+-> SHFIR

EX1IR   := TMS1 -> UPDIR

+-> PAUIR

PAUIR   := TMS1 -> EX2IR

+-> PAUIR

EX2IR   := TMS1 -> UPDIR

+-> SHFIR

UPDIR   := TMS1 -> SELDR

+-> RTI

;OUTPUT EQUATIONS

TLR.OUTF     = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT* INTRST

RTI.OUTF     = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

SELDR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

CAPDR.OUTF   = /DRSHF* DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

SHFDR.OUTF   =  DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR* TDOACT*/INTRST

EX1DR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

PAUDR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

EX2DR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

UPDDR.OUTF   = /DRSHF*/DRCAP* BSUPD*/IRSHF*/IRCAP*/IRUPD*/MUXCTR*/TDOACT*/INTRST

SELIR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD* MUXCTR*/TDOACT*/INTRST

CAPIR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF* IRCAP*/IRUPD* MUXCTR*/TDOACT*/INTRST

SHFIR.OUTF   = /DRSHF*/DRCAP*/BSUPD* IRSHF*/IRCAP*/IRUPD* MUXCTR* TDOACT*/INTRST

EX1IR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD* MUXCTR*/TDOACT*/INTRST

PAUIR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD* MUXCTR*/TDOACT*/INTRST

EX2IR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP*/IRUPD* MUXCTR*/TDOACT*/INTRST

UPDIR.OUTF   = /DRSHF*/DRCAP*/BSUPD*/IRSHF*/IRCAP* IRUPD* MUXCTR*/TDOACT*/INTRST

CONDITIONS

TMS1 = TMS

Once the PALASM description for the TAP controller is complete, we are ready to consider the remaining blocks shown in figure 2.h-o.1. Recalling that all flip-flops share a common clock signal, the schematic diagram for a BS cell may be presented as shown in figure 2.h-o.2:

Figure 2.h-o.2: BS cell able to support the three mandatory instructions.

The PALASM description of the BS register (which contains four cells as shown in figure 2.h-o.2) may be presented as follows:

;---------------------------------- Equations Segment ------------

EQUATIONS

GLOBAL.RSTF = RESET

; Registo BST

BSFF1[0] := (/BSSHF*/BSCAP)*BSFF1[0] + ( BSSHF* BSCAP)*BSFF1[0]

+ (/BSSHF* BSCAP)*BSIN[0]

+ ( BSSHF*/BSCAP)*TDI

BSFF1[1] := (/BSSHF*/BSCAP)*BSFF1[1] + ( BSSHF* BSCAP)*BSFF1[1]

+ (/BSSHF* BSCAP)*BSIN[1]

+ ( BSSHF*/BSCAP)*BSFF1[0]

BSFF1[2] := (/BSSHF*/BSCAP)*BSFF1[2] + ( BSSHF* BSCAP)*BSFF1[2]

+ (/BSSHF* BSCAP)*BSIN[2]

+ ( BSSHF*/BSCAP)*BSFF1[1]

BSFF1[3] := (/BSSHF*/BSCAP)*BSFF1[3] + ( BSSHF* BSCAP)*BSFF1[3]

+ (/BSSHF* BSCAP)*BSIN[3]

+ ( BSSHF*/BSCAP)*BSFF1[2]

BSFF2[0] := /BSUPD*BSFF2[0] + BSUPD*BSFF1[0]

BSFF2[1] := /BSUPD*BSFF2[1] + BSUPD*BSFF1[1]

BSFF2[2] := /BSUPD*BSFF2[2] + BSUPD*BSFF1[2]

BSFF2[3] := /BSUPD*BSFF2[3] + BSUPD*BSFF1[3]

BSOUT[0] = /MODE*BSIN[0] + MODE*BSFF2[0]

BSOUT[1] = /MODE*BSIN[1] + MODE*BSFF2[1]

BSOUT[2] = /MODE*BSIN[2] + MODE*BSFF2[2]

BSOUT[3] = /MODE*BSIN[3] + MODE*BSFF2[3]

The instruction register (IR) cell schematic is shown in figure 2.h-o.3 and is very similar to the BS cell shown in figure 2.h-o.2, the only difference being that the output multiplexer is not required.

Figure 2.h-o.3: Schematic diagram for each IR cell.

The PALASM description for the IR may therefore be presented as shown:

; Instruction register

IRFF1[0] := /INTRST*(/IRSHF*/IRCAP)*IRFF1[0] + /INTRST*( IRSHF* IRCAP)*IRFF1[0]

+ /INTRST*(/IRSHF* IRCAP)

+ /INTRST*( IRSHF*/IRCAP)*IRFF1[1]

IRFF1[1] := /INTRST*(/IRSHF*/IRCAP)*IRFF1[1] + /INTRST*( IRSHF* IRCAP)*IRFF1[1]

+ /INTRST*( IRSHF*/IRCAP)*TDI

IRFF2[0] := /IRUPD*IRFF2[0] + IRUPD*IRFF1[0] + RESET

IRFF2[1] := /IRUPD*IRFF2[1] + IRUPD*IRFF1[1] + RESET

The instruction decoder is a combinational block required to generate the control signals shown in figure 2.h-o.1. The IEEE 11491.1 standard defines when each of these control signals will have to be in the active state, leading to the following PALASM description for this block:

; Instruction decoder

BSSHF = DRSHF*/IRFF2[1]

BSCAP = DRCAP*/IRFF2[1]

BPSHF = DRSHF*IRFF2[1]*IRFF2[0]

BPCAP = DRCAP*IRFF2[1]*IRFF2[0]

MODE = /IRFF2[1]*/IRFF2[0]

DTMUXCTR = IRFF2[1]*IRFF2[0]  ; will be 1 when the instruction is BP

The bypass register is the least complex of the three registers considered thus far and can be represented as shown in figure 2.h-o.4:

Figure 2.h-o.4: Bypass register schematic diagram.

The PALASM description of the bypass register may be represented as follows:

; Bypass register

BPOUT    := (/BPSHF*/BPCAP)*BPOUT + ( BPSHF* BPCAP)*BPOUT

+ ( BPSHF*/BPCAP)*TDI

The two multiplexers preceding the TDO output are still to be described and may be represented by the following PALASM sequence:

; Output of the first mux

DROUT = /DTMUXCTR*BSFF1[3] + DTMUXCTR*BPOUT

; Output of the second mux (TDO)

TDO = /MUXCTR*DROUT + MUXCTR*IRFF1[0]

TDO.TRST = TDOACT

To conclude the description of the architecture shown in figure 2.h-o.1, the only remaining task is to build the PALASM specification of the internal circuit logic, which in the case considered is a simple identity function and may be represented as follows:

; Internal logic

BSIN[2] = BSOUT[1]

BSIN[3] = BSOUT[0]

Once the “internal logic” and the BST infrastructure are described, we are able to build the last section of the BST.PDS file: the simulation section. This section contains the simulation commands which will enable design verification and should therefore be carefully considered. In our case-study, and only to avoid a lengthy description, we will restrict the simulation to capture, shift and update operations in the two main paths of the TAP controller state diagram: data and instruction branches. The simulation section may in this case be represented as follows:

;---------------------------------- Simulation Segment ------------

SIMULATION

TRACE_ON        TCK TMS TDI TDO DRSHF DRCAP BSUPD IRSHF IRCAP IRUPD

;                BSSHF BSCAP BPSHF BPCAP MUXCTR DTMUXCTR TDOACT

;                IRFF1[1,0] IRFF2[1,0]

SETF RESET

SETF /RESET

SETF /TMS /TDI

CLOCKF TCK      ; from TLR to RTI

SETF TMS

CLOCKF TCK      ; to SELDR

SETF /TMS

CLOCKF TCK      ; to CAPDR

CLOCKF TCK      ; to SHFDR

SETF TMS

CLOCKF TCK      ; to EX1DR, shift 0

CLOCKF TCK      ; to UPDDR

CLOCKF TCK      ; to SELDR

CLOCKF TCK      ; to SELIR

SETF /TMS

CLOCKF TCK      ; to CAPIR

CLOCKF TCK      ; to SHFIR

CLOCKF TCK      ; SHFIR, shift 0

CLOCKF TCK      ; SHFIR, shift 0

CLOCKF TCK      ; SHFIR, shift 0

SETF TMS

CLOCKF TCK      ; to EX1IR, shift 0

CLOCKF TCK      ; to UPDIR

CLOCKF TCK      ; to SELDR

SETF /TMS

CLOCKF TCK      ; to CAPDR

CLOCKF TCK      ; to SHFDR

SETF TDI

CLOCKF TCK      ; SHFDR, shift 1

SETF /TDI

CLOCKF TCK      ; SHFDR, shift 0

SETF TDI

CLOCKF TCK      ; SHFDR, shift 1

SETF /TDI

CLOCKF TCK      ; SHFDR, shift 0

SETF TDI

CLOCKF TCK      ; SHFDR, shift 1

SETF /TDI

CLOCKF TCK      ; SHFDR, shift 0

TRACE_OFF

The waveforms resulting from the stimuli applied to the circuit inputs, as generated by the PALASM simulator and waveform visualisation, are partially shown in figure 2.h-o.5 and confirm correct circuit operation:

Figure 2.h-o.5: Waveforms for the simulation section considered (partially shown).

As a final exercise, the reader should identify the two aspects in which the proposed implementation violates the IEEE 1149.1 standard and discuss the implications resulting from those violations.