# Basic test concepts

Testing is one of the final stages in the production cycle and is of fundamental importance to assure that customers will get working units. No product can be manufactured without an efficient test strategy, guaranteeing that the percentage of defective products passing undetected is sufficiently low to be acceptable (this threshold level is not the same for all products / manufacturers / customers, but its range is normally measured in parts per million). The definition of the test strategy has indeed to start early in the design stage, since any decisions capable of simplifying the production test phase will normally lead to lower cost and more reliable products.

Being such an important stage in terms of influencing customer satisfaction, test has over the years taken a very prominent role in the overall design and test cycle. We will therefore start by considering the basic test concepts, which have to be clearly understood in order to perceive the importance of the test infrastructures defined in the IEEE 1149.X suite of test standards and proposed standards. Our roadmap in this route starts with the concepts of fault models and controllability / observability issues, in order to prepare the reader for the elementary procedures leading to automatic test vector generation, and finally to testability improvement alternatives made possible by ad hoc and structured test methods.

## Fault modeling and ss@ faults

Our first challenge when trying to classify or enumerate the reasons for incorrect circuit operation is how to cope with the very large number of its possible causes. Short or open circuits, as well as defective or incorrectly placed components, are just a few among several other common reasons for circuit malfunction. Considering that short circuits alone grow exponentially with the number of nodes in the circuit, it becomes immediately clear that no feasible test strategy can be developed unless the complexity of the problem is first reduced to acceptable levels.

Confining the boundaries of the test problem, in terms of complexity, will then have to start by reducing the number of situations leading to circuit malfunction. This was the reason why the development of methods to describe physical defects at logic level started as soon as the early 60s, basically when it started to become clear that testing an integrated circuit (IC) could not continue to be done by checking correct operation for all input combinations. Fault models were the result of this effort and enable us to develop test vector generation algorithms without having to consider the unfeasible universe of physical defects, but rather a much smaller group of their “representatives” at logic level. It should however be stressed that there is frequently no “equivalence” relationship between defects and faults, since otherwise the complexity of the problem would basically be the same. The attributes that qualify a good fault model are mainly two, namely:

- It has to be sufficiently simple to allow efficient test vector generation procedures.

- It has to be sufficiently representative in order to guarantee that the percentage of defective components passing undetected (by a test vector set able to detect 100% of the faults) is very small.

Among several fault models that were proposed over the years, the single stuck-at (ss@) fault model was one of the first and still is one of the most popular (due to its properties concerning the two attributes defined above). This fault model defines only two types of faults, s@1 and s@0, and considers that only one fault at a time is present in the circuit. Notice that this restriction guarantees that the number of faulty conditions now grows linearly with the number of nodes, instead of exponentially, which enables the development of practical computer programs for automatic test vector generation.

Even without providing a close representation of real manufacturing defects (there is no equivalence between short-circuited nodes and stuck-at nodes), experience has shown that test vector sets approaching 100% fault coverage of ss@ faults are very effective in preventing defective components to pass undetected.

## Controllability, observability and testability

The basic procedures for the generation of test vectors to detect ss@ faults will be easier to understand if we start by considering three essential concepts in this area: controllability, observability and testability.

Controllability gives us a measure of how easy or difficult it is to force the logic level in a node to either 1 or 0.  If we consider the simple circuit shown in figure 1.1, it is easy to verify that only 2 out of 8 possible input combinations enable us to drive node Y to 1. On the other hand, driving X to 0 can be done with half of the possible input combinations, leading us to conclude that a better controllability exists in the second case.

| A | B | C | Contr. Y=1 |  | A | B | C | Contr. X=0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | X | - |  | X | X | 0 | - |
| 0 | 1 | X |  |  | X | X | 1 |  |
| 1 | 0 | X | - |  |  |  |  |  |
| 1 | 1 | X | - |  |  |  |  |  |

Figure 1.1: Controllability of nodes Y=1 and X=0.

The example above suggests that in a circuit with low controllability we will frequently find it difficult to drive a node to a required logic value, which certainly does not simplify test vector generation, since:

- As we will see later, our first step to detect a given s@ fault in a node consists of driving it to a logic value that is the opposite of the fault.

- In an IC, the value at any node can only be controlled from the input pins (the primary inputs of the circuit).

The same reasoning leads us to conclude that a given fault can only be detected if the error signal (the difference between the value that we tried to force into the node and the actual value due to the s@ condition) can be propagated to one IC output (to a primary output of the circuit). The observability of a node is a parameter that gives us a measure of how easy or difficult it is to propagate the logic level in a node to a directly observable output.  If we again consider the same circuit, now shown in figure 1.2, it is easy to verify that 6 out of 8 possible input combinations allow us to observe the value of Y at output F. On the other hand, observing the value of X can be done with half of the possible input combinations, leading us now to conclude that the observability of Y is better than that of X.

| A | B | C | Observ. of Y |  | A | B | C | Observ. of X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | 0 | - |  | 0 | 0 | X |  |
| X | X | 1 |  |  | 0 | 1 | X | - |
|  |  |  |  |  | 1 | 0 | X |  |
|  |  |  |  |  | 1 | 1 | X |  |

Figure 1.2: Observability of nodes Y and X.

The reader may have perceived by now that the test vector generation task will be much easier for circuits with high levels of controllability and observability. In fact, it is useless to have high controllability in a node if we can not propagate its value to a circuit output; and it is just as likely useless to have high observability of a node if can not force its logic level from the circuit inputs.

To conclude this section, we will present the concept of testability as a measure of the circuit controllability and observability values in all its nodes. Obviously enough, high testability makes test vector generation much easier and will in most cases lead to better test effectiveness.

## Test vector generation for combinational circuits

Many of the current test vector generation algorithms can be traced back to a basic procedure developed in the mid-60s by Paul Roth and known as the D-algorithm (because of the D-notation that was used to describe the algorithm). This procedure was based in the ss@ fault model and considered that each node could be in one of four possible states: 0, 1, D (1/0, meaning that we tried to drive the node to 1 but it was s@0) and /D (0/1), as described in table 1.1:

| Condition at the node | D- notation | Description |
| --- | --- | --- |
| 0/0 | 0 | Fault-free node at 0 |
| 0/1 | /D | s@1 node which we are trying to drive to 0 |
| 1/0 | D | s@0 node which we are trying to drive to 1 |
| 1/1 | 1 | Fault-free node at 1 |

Table 1.1: The D-notation.

Based on these four possible node conditions, the basic procedure to generate a test vector according to the D-algorithm can now be described by the following three-step sequence, for each fault at each node:

- Drive the node to a logic value that is the opposite of the fault (0 if s@1 and 1 if s@0).

- Propagate the error signal (D or /D) to a primary output.

- Justify (backwards) the values that enable the propagation path, until a necessary combination at the primary inputs (a test vector) is found.

When 100% of the circuit ss@ faults have been detected, or otherwise the acceptable / feasible upper limit for each case has been reached, we will have available a small (when compared to all possible input combinations) and reliable test vector set to be used in a production test environment.

Due to the type of faults considered (s@ connections) and to the hardware description level used (schematic diagram), this type of test is frequently referred to as structural test. In order to maximise test effectiveness, structural test vectors are frequently complemented with functional test vectors in the final production test strategy for each product.

Figure 1.3 illustrates the application of the three step D-algorithm to the same example circuit considered before, in this case to detect a s@0 fault in node Y:

| (a) Node Y s@0. | (b) Fault activation (step 1). |
| --- | --- |
| (c) Fault propagation (step 2). | (d) Fault justification (step 3). |

Figure 1.3: Test vector generation according to the D-algorithm.

Only the test vector (A,B,C)=(0,1,1) is able to detect the fault s@0 in node Y, since no other combination at the circuit inputs will simultaneously drive the opposite value at the s@ node and propagate the error signal to the circuit output. It is however common that more than one vector exists to detect a given fault, such as would happen if we considered the fault s@1 in this same node Y. For this case, and considering that setting one of the AND inputs to 0 will drive Y to 0 (if it is not s@1) and that C=1 guarantees propagation of the error signal, we have three vectors able to detect the fault: (A,B,C) = { (0,0,1) | (1,0,1) | (1,1,1)}.

An opposite situation may also take place, when there are no vectors able to detect a given fault.  Figure 1.4 shows such an example for the same case of Y s@0 (but now with a slightly different circuit with only two inputs), where it is not possible to drive the faulty node to 1 and at the same time to propagate the resulting error signal to the circuit output (the A input should simultaneously be 0 and 1).

| (a) Node Y s@0. | (b) Opposite value of s@0 applied in node Y. |
| --- | --- |
| (c) Propagation (tentative). | (d) Justification (not possible). |

Figure 1.4: Example of an undetectable fault.

There are however some cases where a test vector may still be found even if we momentarily come to a conflicting situation, provided that it is possible to backtrack to a previous decision point and choose a different solution. In the example shown in figure 1.5, and when the first choice in (b) is seen to be impossible, the alternative solution tried in (c) leads to the required test vector.

| (a) Node Y s@0. | (b) First tentative (not possible). |
| --- | --- |
| (c) Alternative choice. | (d) Propagation and justification. |

Figure 1.5: Example of backtracking to generate a test vector.

Test vector generation is in practice done by tools that accept a description of the circuit (such as a netlist) and search for the smallest set of test vectors able to achieve maximum fault coverage.

## Testability and test generation in sequential circuits

The ss@ fault model and several improvements over the basic D-algorithm described in the previous section led to several tools that are very efficient (in terms of execution time and fault coverage) for test pattern generation, as far as combinational circuits are concerned. Sequential circuits are however much more difficult to handle in this respect, for reasons which are better explained through a simple example such as the one shown in figure 1.6.

The application of the basic D-algorithm procedure to the case of fault s@0 in node X rapidly leads us to the test vector required at the inputs of the combinational block (the two ANDs and one OR), but these are not the circuit primary inputs. In fact, our circuit is seen from the outside as having only one primary input (A) and one primary output (F), besides the clock signal.

Figure 1.6: Simple sequential circuit with a s@0 fault.

The problem in this case is that the inputs of the combinational block are connected to the outputs of the flip-flops (the state variables) and not to the circuit primary inputs. What we got as a result of the D-algorithm procedure was therefore the state combination that is required to detect X s@0, but the sequence in A that will bring the circuit to this state is still to be found. In the example of figure 1.6 this is not really a problem, since two clock cycles with A=1 will bring the two flip-flop outputs to 1 (and their complements to 0), as shown in figure 1.7.

|  | (a) This is the initial circuit state, where we assume that both flip-flops are at 0. Fault detection is not possible because neither fault activation nor fault propagation take place. |
| --- | --- |
|  | (b) The first clock cycle is applied with A=1, leading the circuit to a new state which is still not capable of activating the fault. Fault propagation would however be possible, since the other OR input is now at 0. |
|  | (c) The second clock cycle is applied with A=1 and guarantees fault detection, because the circuit is now brought to a state where fault activation and fault propagation are simultaneously possible. |

Figure 1.7: Circuit conditions from initial state until fault detection.

The following remarks will help us to understand why test generation for sequential circuits is generally a very complex problem (and why it was not so for the case considered in figure 1.6):

- The application of the D-algorithm gives us the combination required at the inputs of the combinational block, which includes the flip-flop outputs, but the sequence required at the primary inputs to reach this state is still to be found and requires knowledge about the circuit state transition diagram. This was not a problem in the example of figure 1.6 because the two flip-flops are connected as a shift-register and therefore it is very straightforward to load any required state at their outputs.

- A similar situation takes place if the output where the fault is detectable is not a circuit primary output, but is instead a next state output.

- Even when the fault-free state transition diagram is known beforehand, it may well happen that a ss@ fault affects the next state outputs of the combinational block. In this case the new state transition diagram would have to be computed before the sequence required at the primary inputs can be found. Again this was not the case in the example of figure 1.6, where the fault at node X has no such effect.

Considering a fault that modifies the state transition diagram will give us a better understanding of how difficult it will become to find a test vector, even for the very simple circuit shown in figure 1.6. The case of node Y s@0 is one of such faults, since we can no longer in this case continue to consider the fault-free state transition diagram. Figure 1.8 shows the two diagrams for this case, assuming that Q1 and Q0 are respectively the outputs of the upper and lower flip-flops.

| (a) Fault-free operation. | (b) Node Y s@0. |
| --- | --- |

Figure 1.8: State transition diagram for the circuit of figure 1.6, with and without Y s@0.

As we already know, setting A=1 during two clock cycles takes the circuit to state 3 and enables the detection of X s@0 (Q1,Q0=11 as shown in figure 1.8.a). However, if Y is s@0, the new steps required to detect this fault will now have to be as follows:

- The D-algorithm will give us the required test vector at the inputs of the combinational block (same as for X s@0).

- The new state diagram for the faulty circuit, shown in figure 1.8.b, will now have to be derived (this step was not necessary for the case of X s@0).

- Analysing the two state diagrams, we are now finally able to find out the required sequence at primary input A.

It is interesting to notice that the same sequence in A (1 during two clock cycles) will also detect Y s@0, as becomes evident by considering the state diagrams shown in figure 1.8. The circuit will be taken to state 3 if the fault is not present (F=1) and will otherwise remain in state 0 (F=0) if Y is s@0.

## Testability improvement via ad hoc solutions

Ad hoc measures to improve testability, as the name suggests, consist of design rules or amendments introduced to avoid or minimise test vector generation problems. These measures are specific to each design and therefore have two major drawbacks:

- Are not always reusable, since each design has its specific requirements and testability problems.

- Are not able to guarantee high testability levels regardless of the circuit.

A large number of examples might be used to illustrate ad hoc design for testability (DfT) rules, but it is largely preferable that structured DfT rules (considered in the following section) are used instead. As such, we will only refer the following common ad hoc solutions:

- In order to avoid having to apply a large number of clock cycles to counters with a large number of bits, it is common to provide a way to split an n-bit counter into k (n/k)-bit counters for test purposes, which could then be taken to any n-bit combination with a much smaller number of clock cycles.

- The provision of reset or preset lines, synchronous or asynchronous, are also common solutions to force the circuit to one or more given states with few or even no clock cycles required.

- In order to enable better observability, some internal nodes may be brought to primary output nodes, either by increasing the number of primary outputs or by multiplexing existing primary outputs with the internal nodes to be observed. A similar solution is possible concerning controllability of internal nodes, which may be connected to additional primary inputs multiplexed with the internal signals controlling those nodes.

These and other measures are however not systematic enough as to enable a uniform approach to testable circuit design, a reason which led since the mid-70s to the development of design methodologies that can always guarantee high levels of testability.

## Structured approaches to design for testability

The basic concept underlying most structured design for testability (DfT) methodologies consists of providing a systematic way of driving the circuit to any given state in a fixed (and short) number of clock cycles, regardless of the current state and of the fault present in its combinational circuitry (even if affecting the next-state generation logic). The expression structured DfT implies that the same design approach can always be used and assure good testability levels, regardless of the circuit function. There is however a price to pay, which normally consists of accepting that some design rules (rather a design style) are enforced and that additional silicon area, more pins and higher propagation delays, are tolerated.

The test vector generation problem can be greatly simplified for any synchronous sequential circuit built with D flip-flops (D-FF), if each D-FF input is preceded by a 2:1 multiplexer, with all multiplexers sharing a common control signal. This scheme is shown in figure 1.9, where we can see that driving the Test Mode control signal to 1 will rearrange all D-FFs in a standard shift-register and will enable any state combination to be shifted in using exactly N clock cycles (with N being the number of D-FFs).

Figure 1.9: Scan design principle.

When each D-FF in the circuit is preceded by this 2:1 multiplexer (creating what is commonly called a scan flip-flop) we have a case of full-scan design. This structured DfT methodology started to gain wide acceptance in the late 70s, when it became evident that test vector generation algorithms for sequential circuits would not be able to cope with the increasing complexity of emerging VLSI designs. Scan design techniques overcome this difficulty by providing a simple solution to the two main problems that make test vector generation for sequential circuits so highly complex:

- First problem: Part of the combinational circuitry inputs are not directly controllable, since they come from the D-FF outputs (these nodes define the circuit present state).

Solution: The scan flip-flops shown in figure 1.9 enable direct controllability of the D-FF outputs through a simple procedure with a fixed number of clock cycles:

- Set the Test Mode control signal to 1

- Shift in the required state combination (three clock cycles)

- Set the Test Mode control signal to 0

Figure 1.10 shows the example of taking the circuit to state 110, assuming that the current state was 100 and that the next state inputs were 001 (notice again that this sequence is always possible, regardless of whatever function the circuit performs or wherever the s@ fault is located):

|  | - Initial circuit conditions: Present state 100, next state 001. Recall that the present state nodes are (internal) inputs of the combinational logic block defining the global sequential circuit function and that the next state nodes are (also internal) outputs of this same block. Since the Test Mode control signal is at 0, each clock cycle will transfer to the D-FFs outputs the values present at the next state nodes. |
| --- | --- |
|  | - The Test Mode control signal is now at 1 and the D-FFs are connected as a shift-register. The first clock cycle applied will shift the D-FFs one bit position “up” and transfer the value present at the Scan In input to the output of the first D-FF in the chain. The values shown at each node are those following the application of the first clock cycle with Scan In at 1. Notice that the next state values will now be different from the initial conditions, since the inputs of the combinational block now have different values as well. |
|  | - The values shown at each node are now those following the application of the second clock cycle, keeping Scan In at 1. Notice that the initial present state values are being shifted out simultaneously as the new present state is being shifted in. |
|  | - The last clock cycle has now been applied with the Scan In input set at 0, so the new present state is 110 as requested. Three clock cycles were therefore necessary to take the circuit to its new present state, the general rule being that the number of clock cycles required is equal to the number of D-FFs. For larger circuits this rule guarantees a much faster transition to any given state (in the extreme case, for a circuit with N D-FFs, 2N-1 clock cycles are replaced by N clock cycles). |
|  | - The requested present state has been loaded and the Test Mode control signal has been taken back to 0. Each next state node is now connected to the respective D-FF input and is defined by the combinational logic block according to present state 110 and to the current values at the (external) circuit primary inputs. |

Figure 1.10: Controllability of the present state nodes.

- Second problem: Part of the combinational circuitry outputs are not directly observable, since they go to the D-FF inputs (these nodes define the circuit next state).

Solution: The scan flip-flops shown in figure 1.9 enable direct observability of the D-FF inputs through a similar procedure:

- Capture the values present at the D-FF inputs (the outputs of the combinational circuitry).

- Set the Test Mode control signal to 1.

- Shift out the current state combination (three clock cycles).

- Set the Test Mode control signal to 0.

Figure 1.11 considers the same initial conditions as in figure 1.10.a and shows how to observe the next state defined by the combinational circuitry outputs (again whatever the circuit function, wherever the fault may be):

|  | - The initial circuit conditions are the same as in the previous example, the present state being 100. The 2:1 multiplexers have their Test Mode control signal at 0 and are therefore in “transparent” mode (apart from the larger propagation delay, the circuit behaves as if no DfT techniques were present). |
| --- | --- |
|  | - The values shown at each node are those following the application of the first clock cycle. Since the Test Mode control signal was kept at 0, the values transferred to the D-FF outputs were those present in the next state nodes, defined by the (internal) outputs of the combinational logic block. Notice that the value present at the Scan Out output is now the first bit to be shifted out. As was seen in the previous example, the next state node values were modified as well. |
|  | - The Test Mode control signal was now set to 1 and the values shown at each node are those following the application of the second clock cycle. Since the D-FFs are now connected as a shift-register, the present state nodes were shifted one bit position “up” and the second bit was shifted out. Notice that the first D-FF in the chain loads the value present at the Scan In input, even if we are not interested in controlling the values to be left at the present state nodes. |
|  | - The values now shown are those following the application of the third (and last) clock cycle. The last bit (the rightmost bit in the initial 001 next state) was now shifted out. Notice that only two clock cycles were required after the Test Mode control signal was set to 1, since the first bit is immediately present at the Scan Out output. |
|  | - Setting the Test Mode control signal to 0 brings the circuit back to the normal operation mode. The present state was modified as defined by the values shifted in and the next state values were modified accordingly. Notice however that the initial present state might have been kept, if Scan Out and Scan In were directly connected and a fourth clock cycle was applied before setting the Test Mode control signal back to 0. |

Figure 1.11: Observability of the next state nodes.

The main virtue of scan design techniques is therefore their ability to eliminate the sequential nature of the circuit, which is seen by the test vector generation tool as a pure combinational circuit. The remarks initially made concerning the cost of DfT should be clear by now:

- The presence of the extra 2:1 multiplexers lead to larger propagation times and require extra silicon area, eventually increasing the circuit cost. Notice however that we can not be positive about the cost issue, since it might well happen that our circuit had spare pins and was not core-limited, in which case the circuit might actually become cheaper to produce (since test is now so much easier).

- The designer is not free to use whatever solution he might find more appropriate, since the controllability and observability of all combinational inputs and outputs can only be achieved if the synchronous scan flip-flop structures defined by full-scan techniques are accepted for the whole circuit (for example, no asynchronous structures are allowed).

In those cases where full-scan can not be accepted, it is still possible to select only those nodes which present higher testability problems and restrict the use of scan flip-flops to those circuit areas.

1.	Basic test concepts

1.1	Fault modeling and ss@ faults	1.1

1.2	Controllability, observability and testability	1.2

1.3	Test vector generation for combinational circuits	1.4

1.4	Testability and test generation in sequential circuits	1.7

1.5	Testability improvement via ad hoc solutions	1.11

1.6	Structured approaches to design for testability	1.12