# 5.h-o 2. Non-BS cluster testing with PRPG and SA

Following the approach selected in part 1 of this text, we will now consider another alternative for testing the same non-BS cluster. The previous approach consisted of performing an exhaustive test of this cluster, for which 16 test vectors were required (since this cluster has only 4 inputs). By analysing the respective test program, we conclude that a total of 680 TCK cycles were required solely for test vector / responses shift in and out (additional TCK cycles are required for the necessary transitions across the TAP controller state diagram).

The approach to be considered in the second part of this text consists of using the facilities available in the Texas Instruments SCOPE octals (which surround the cluster to be tested) for pseudo-random pattern generation (PRPG) and response compression by signature analyses (SA). This alternative has the additional advantage of highlighting some aspects which are frequently found in the area of BIST (Built-In Self-Test), where these methods for test vector generation and response compression are widely used.

For the case now being considered, we will set the left octal to work in PRPG mode and the right octal in SA mode. The information present in the Texas Instruments data sheet for these components shows that the sequence of actions to carry out is as follows:

- Shift into the instruction register (IR) the code of BOUNDARY READ (e.g. X0001010).

- Shift in the initial seed value for the BS register.

- Shift into the IR the code of TEST DATA REGISTER SCAN (e.g. X0001110).

- Shift into the Test Data Register the code corresponding to the required operating mode (01 for 16-bit mode PRPG; 10 for 16-bit mode SA).

- Shift into the IR the code of BOUNDARY RUN TEST (X0001001).

- Bring the TAP controller to the Run Test / Idle state, where the necessary number of TCK clock cycles will be applied.

The application of TCK cycles while in the Run Test / Idle state will then initiate the test operation. Recall that the test vector responses do not have to be shifted out individually in this case, since the SA operation is in progress at the inputs of the second SCOPE octal. When the necessary number of TCK cycles has taken place, the contents of the BS register of the second octal are shifted out and compared with the expected signature.

The application of this method requires that an answer is previously found for the two following questions:

- How should we proceed to determine the expected (correct) signature? (recall that error detection through the resulting signature will be done using the NSHFCP instruction, which will include the correct signature information as part of its operand).

- How many test vectors will have to be applied to conclude the test operation? (we will use the 16-bit mode PRPG, but our 4-input cluster does not require  neither would it be acceptable, for reasons of test speed  that all possible combinations in the octal outputs are applied).

These questions will now be considered in detail in the following sections.

## Determining the correct signature

The expected contents in the BS register of the second octal, after the test operation is concluded, may be determined by one of the following methods:

- By logic simulation of the three blocks: PRPG, non-BS cluster and SA.

- By reading the contents of the BS register of the second octal, after a complete test run takes place with a circuit which is known to be fault-free.

The first method is clearly the most time consuming and requires more sophisticated resources, since we will have to build simulation models for the three blocks and proceed to simulate their operation during a complete test run. We might for example use Altera’s tools (version 7.21, distributed through the Altera University Program, is free) and use the schematic capture and logic simulation modules for this purpose. The schematic diagrams of the three blocks to be simulated are shown in figures 5.h-o2.1 to 5.h-o2.3.

The complete circuit to be simulated would then be defined in a top-level schematic description including these three blocks, as shown in figure 5.h-o2.4. Logic simulation could now be carried out and the required result read in the SA block outputs after the last TCK cycle has been applied. The expected signature determined using this procedure will naturally be the same that is obtained by setting up the same operating conditions in practice, provided that we are careful enough to avoid errors during the schematic capture phase.

Figure 5.h-o2.1: BS register in 16-bit PRPG mode (74BCT8244).

Figure 5.h-o2.2: Non-BS cluster.

The second method is however much faster and we recommend that the reader considers it as the preferred alternative (the test program should be run in a fault free circuit and suspended before the last NSHFCP instruction is executed; shifting out the contents of the boundary scan register of the second octal will yield the required signature).

Figure 5.h-o2.3: BS register in 16-bit SA mode (74BCT8244).

Figure 5.h-o2.4: Top-level representation of the circuit to be simulated.

## The required number of PRPG test vectors

The last question to consider is how to determine when is it possible to conclude the application of TCK cycles (each TCK cycle produces a new PRPG test vector), by considering that all the possible combinations in the 4 cluster inputs have already been generated (although we are still applying an exhaustive test vector set, we are not using a deterministic test procedure). The answer to this question may be obtained by one of the two following alternatives:

- Once again through logic simulation (for example with the Altera tools), this time involving only the PRPG block in figure 5.h-o2.1, we are able to check the outputs of this block and determine what is the number of TCK cycles required to obtain the 16 possible combinations in the 4 cluster inputs.

- Simulation of the PRPG operation in the SCOPE octal outputs could also be done using the Scan Educator application which is available from Texas Instruments web server (www.ti.com).

Figure 5.h-o2.5: The Scan Educator in the required mode (control of a single SCOPE octal).

This second method is in fact much faster and may be started by selecting the option 4 in the main menu (“scan control mode testing  single device”) followed by option 3 (“on your own”). This sequence will take us to the situation represented in figure 5.h-o2.5, where the six steps described in the first page can be carried out (to shift in instructions and initialise the data registers). The application of TCK cycles will then start until the necessary 16 combinations in the 4 cluster inputs have already taken place (the required number of TCK cycles depends on the seed value shifted into the boundary scan register).

We conclude this second part on testing partial BS boards by inviting the reader to write the test program corresponding to this second strategy and to verify its operation in practice. The required number of TCK cycles for this second approach should be compared with the 680 cycles necessary for shifting in / out test data in the previous approach.