# A Windows BS test controller application

This Windows test controller application, called Tapper, is compatible with the test controller model described in a previous chapter and was developed by Luís Humberto Silvestre to support practical hands-on sessions with BS boards comprising one or two BS chains. Tapper will be described in this chapter, starting with the required test set up, followed by a short description of the functions supported and of its user interface.

## Test set up

Tapper enables the user to write and execute test programs for boards with one or two BS chains, accessing the respective TAPs through the PC parallel port. The parallel port I/O pins were mapped to the test interface signals as presented in table 6.1.

| Test signal | D25 connector | I/O | Pin name | Centronics |
| --- | --- | --- | --- | --- |
| TDI 0 | 2 | Output | D0 | 2 |
| TMS 0 | 3 | Output | D1 | 3 |
| TCK 0 | 4 | Output | D2 | 4 |
| TRST 0 | 5 | Output | D3 | 5 |
| TDI 1 | 6 | Output | D4 | 6 |
| TMS 1 | 7 | Output | D5 | 7 |
| TCK 1 | 8 | Output | D6 | 8 |
| TRST 1 | 9 | Output | D7 | 9 |
| NC | 1 | Output | STROBE | 1 |
| NC | 14 | Output | AUTO FEED | 14 |
| SOA | 16 | Output | INIT | 31 |
| SOB | 17 | Output | SELECT IN | 36 |
| NC | 10 | Input | ACK | 10 |
| TDO 0 | 11 | Input | BUSY | 11 |
| TDO 1 | 12 | Input | PE | 12 |
| SIA | 13 | Input | SELECT | 13 |
| SIB | 15 | Input | ERROR | 32 |
| GND | 18-25 | - | GND | 19-30 |

Table 6.1: I/O to test interface pin mapping.

Notice that provisions were made to support two synchronisation channels (A and B), through sync inputs (SIA and SIB) and outputs (SOA and SOB). However, and due to the unavailability of pins in the PC parallel port to apply test stimuli and capture responses, the sync instructions using these two pairs of pins are currently not implemented.

The test set up is shown in figure 6.1, where a flat-cable connects the PC parallel port to the test connector of the demonstration board described in the previous chapter.

Figure 6.1: The test set up, showing the demonstration board and the flat-cable connection to the PC.

## The test controller application

The Tapper window comprises a test program control bar, status information area, three pop-down menus and a general purpose editing area, as shown in figure 6.2. Notice that the small test program segment shown checks the integrity of a scan chain that includes a single IC placed in TAP 1 (question: what would in this case be the operating mode selected for the BS architecture and the TAP controller state, following the execution of the test program shown?).

Figure 6.2: The Tapper user interface.

### Test program control bar

The test program control bar includes the five control buttons shown in figure 6.2. From left to right, the function of each button may be described as follows:

- : The Run button initiates program execution.

- : The Pause button temporarily suspends program execution.

- : The Stop button stops the execution of the test program.

- : The Step button executes the test program in single-step mode.

- : The Breakpoint button is used to insert a breakpoint in the current line.

Any number of breakpoints may be inserted, forcing the program to suspend execution when a line with a breakpoint is reached. Pressing either the Run or Single-step buttons will reactivate program execution.

### Status information

The status information area includes information about the TAP currently selected, the contents of the internal counters, the error flag currently selected and the state of the available error flags. A brief description of each of these fields, as they appear following the execution the small test program represented in figure 6.2, may be presented as follows:

- : Indicates the currently selected TAP.

- : The Tapper has two internal counters of 16 and 24 bits and supports the LD CNT,N instruction (any of the two counters may be used internally in this case), a LD C16,N instruction (uses the 16-bit counter) and a LD C24,N instruction (uses the 24-bit counter).

- : Indicates the currently selected error flag and the state of each of the eight error flags available for each BS chain. Since the test program shown in figure 6.2 was executed with no board connected to the PC parallel port, the selected error flag was set during program execution.

### Pop-down menus

The Tapper application includes three standard pop-down menus: File, Edit and Help. The options available in these menus are the following:

- The File menu has the standard options of New, Open, Save, Save as and Exit.

- The Edit menu includes the standard options of Undo, Cut, Copy, Paste and Delete, followed by a last Clean all breakpoints option that removes all breakpoints inserted in the test program.

- The Help menu has the standard options of Contents, Index and find, and About Tapper. Additionally, it includes a TAP controller option that shows the state transition diagram of the TAP controller FSM.

### Editing area

The editing area is used to write or modify test programs. The syntax of each line is as follows:

Label:	instruction	operand	; comment

Notice that a colon must always follow a label and that any text after a semi-colon is considered as a comment.

## Application set up

A set up diskette is included at the end of this document, containing an executable file called Setup.exe. To install the Tapper application, run this executable file.

6.	A Windows BS test controller application

6.1	Test set up	6.1

6.2	The test controller application	6.2

6.2.1	Test program control bar	6.3

6.2.2	Status information	6.4

6.2.3	Pop-down menus	6.4

6.2.4	Editing area	6.4

6.3	Application set up	6.5