# A BS test controller model

The operation of the BS architecture at IC-level is by now well understood, as well as the test protocol that enables the detection of several types of faults in a BS board. However, and before a test program may be written, we still need to develop a formal specification “language” that allows us to code the necessary test protocol steps. The BS test controller model presented in this chapter is the tool that will enable us to specify test programs for real case studies.

## Basic test operations

Analysing the basic test operations required to implement the three main phases of the test protocol for BS boards (integrity check of the BS architecture, interconnects and clusters test, components test) leads us to conclude that three main types of basic test operations are required. These operations will be detailed in the next sections and their objective is to control the BS infrastructure, to synchronise this infrastructure with external test resources and to control internal resources and test program flow.

### Basic test operations to control the BS infrastructure

Used extensively in any of the three main test protocol steps, these basic test operations are the following:

- Apply an active-low pulse in /TRST.

When the optional /TRST pin exists, applying a logic 0 in this pin in all BS ICs provides a faster alternative to bring their TAP controller FSM to the TLR state, than holding TMS at 1 and applying 5 TCK cycles.

- Apply a TCK cycle while TMS is fixed at a pre-defined (0 or 1) logic value.

This basic test operation is required to step through the TAP controlled state transition diagram.

- Shift a bit stream into the selected scan chain (no comparison required on the bits shifted out). The TMS line must be held at 0 except in the last TCK cycle, when it must be held at 1.

There are several occasions when the bit stream shifted out does not carry any information of interest, such as when the first vector for interconnect test is shifted in (the values previously captured by the BS cells are meaningless, since no test vector was previously applied). Regardless of whether or not the bit stream shifted out is to be compared, the last TCK cycle (shifting the last bit) has to be applied with TMS held at 1, so that the TAP controller leaves the Shift state with the last bit and proceeds into the Exit1 state.

- Shift a bit stream into the selected scan chain while the bit stream shifted out is checked against expected values in pre-defined bit locations. Again, the TMS line must be held at 0 except in the last TCK cycle, when it must be held at 1.

During the application of a set of test vectors, and while each new vector is shifted in (with the exception of the first), the responses to the previous vector are shifted out. It is therefore necessary to compare the responses being shifted out with their expected values, but only some of the bits shifted out carry meaningful data. In fact, and although every BS cell captures the value present in its parallel input, only a few of those cells are associated with BS input pins (capturing the logic values present in the PCB interconnects) or to BS output pins where the expected value at the output of the internal IC logic is known. For each bit to shift into the scan chain, two additional bits are therefore required: a comparison mask (to enable or disable the comparison) and the expected value (when known).

- Apply N TCK cycles while TMS is held at 0.

Running BIST operations accessible through the BS architecture requires that the TAP controller FSM is taken to the RTI state and kept there (by maintaining TMS at 0) during a number of TCK cycles not lower than the value specified in the manufacturer data sheet.

- Select the BS chain in which the previous test operations are to take place.

This basic test operation is required in the case of multiple BS chain boards, if we assume that the test controller is able to control only one chain at a time.

### Basic test operations to synchronise the BS infrastructure with external test resources

The test protocol for full-BS interconnects and non-BS clusters, described in the previous chapter, made it clear that testing interconnects or clusters associated with cells from different BS chains, or with primary I/O pins of the PCB, require a co-ordinated procedure to complete test vector application from all driving pins, before the responses start to be captured. A synchronisation protocol is required when co-ordinating the BS infrastructure and the external test resources, which may be implemented by a simple handshake procedure for test vector application and response capturing on both sides, based on a sync output and a sync input and leading to the following basic test operations:

- Set the sync output to a pre-defined logic value (0 or 1).

The BS test controller will set the sync output to notify the external test equipment that the application of the test vector through the BS infrastructure is finalised and will reset it to notify that all test responses were captured through the BS infrastructure.

- Wait until the sync input is set to a pre-defined logic value (0 or 1).

Following the application of each test vector through the BS infrastructure and its notification through the sync output, the BS test controller will wait for notification from the external test equipment (that the external test stimuli were applied as well) before capturing any responses. Similarly, after concluding the response capturing phase, the BS test controller will wait for notification from the external test equipment (that response capturing was finalised there as well), before initiating the application of the next test vector.

### Basic test operations to control internal resources and test program flow

The BS test controller model has to include a small number of internal resources and test program flow control mechanisms, which leads to the last set of basic test operations:

- Load an internal counter with the required number of TCK cycles.

Operations like BIST or shifting a bit stream into the scan chain require the application of a pre-specified number of TCK cycles. Among other possible alternatives for this same purpose, the BS test controller model presented here assumes the existence of an internal counter that will be set to the required number of TCK cycles.

- Select the error flag to be set if an expected value is not found.

Fault detection alone would not require any error flags, but their existence enables a limited fault diagnosis capability. If a new error flag is selected before the set of vectors for each step of the test protocol starts to be applied, it becomes possible to identify what type of faults were detected (even if not with what test vectors or in which cells). Assuming that this limited fault diagnosis capability is of interest, a basic test operation to select the required error flag becomes necessary.

- Perform a conditional jump in the test program flow according to the value of the selected error flag.

Some types of faults might be potentially destructive, such as short-circuit faults, in which case it is recommendable to stop the test at once. This possibility requires the existence of a basic test operation controlling the test program flow, to enable corrective action with minimum delay.

- Conclude the test program.

The simplest of all basic test operations, yet still indispensable.

## The test instruction set

According to the basic test operations that were previously identified, this section presents the instruction set of our BS test controller. Formal specification of any test program becomes possible with this instruction set, as will be seen when a case study with two BS chains and non-BS clusters is presented in a later chapter. The BS test controller instruction set is presented in three parts, each addressing the basic test operation types that were described earlier.

### Control of the BS infrastructure

The instructions available to control the BS infrastructure are those presented in table 4.1, assuming that the board under test may have one or two BS chains. The mnemonic and a simplified description of its operation are presented for each instruction.

| Mnemonic | Operation |
| --- | --- |
| TRST | Applies an active-low pulse in the /TRST line |
| TMS0, TMS1 | Applies a TCK cycle with TMS in 0 (TMS0) or 1 (TMS1) |
| NSHF <bit_stream> | Shifts a bit stream into the selected scan chain (no comparison required on the bits shifted out) |
| NSHFCP <bit_stream> | Shifts a bit stream into the selected scan chain while the bit stream shifted out is checked against expected values in pre-defined bit locations |
| NTCK | Applies N TCK cycles while TMS is held at 0 |
| SELTAP0, SELTAP1 | Select BS chain 0 (SELTAP0) or 1 (SELTAP1) |

Table 4.1: Instructions available to control the BS infrastructure.

Timing specifications will not be presented here, since these details depend on each possible implementation of the BS test controller model. At this stage we are only interested in presenting a functional model of the test controller, which enables us to code the test protocol steps regardless of the physical test controller being a dedicated hardware chip or a software application.

In order to enable the construction of the bit stream in the NSHFCP instruction, we will assume that it is organised in 8-bit segments, interleaved as described below:

- The first 8 bits are those to be shifted into the board TDI pin (the TDI pin in the TAP of the first BS IC in chain).

- The second 8 bits contain the values expected at the board TDO, as each of the above 8 bits is shifted in (the board TDO is the TDO pin in the TAP of the last BS IC in the chain).

- The third 8 bits contain the comparison masks that identify which bit locations have known expected values.

- In each 8-bit group, the rightmost bit is the first to be considered (shifted in or compared).

The small example shown in figure 4.1 may help to clarify the proposed rule to build the bit stream of the NSHFCP instruction, by assuming that it is required to:

- Shift in a new test vector that forces the last 8 cells (8..15) to 01010101 and all the first 8 cells (0..7) to 0.

- Check that cells (0..3) capture 0101 and cells (8..11) capture 0011.

Figure 4.1: A simple 16-bit BS chain.

The NSHFCP instruction bit stream would in this case be as follows:

NSHFCP $55,$30,$F0,$00,$50,$F0

(where the $ sign indicates an hexadecimal notation). The $55 8-bit group is the data to be shifted in (starting by the rightmost bit, which at the 16th TCK cycle will arrive in cell 15), $30 represents expected values (the contents of cells 8..11 arrive at TDO between the 4th and the 7th TCK cycles), $F0 indicates that the first four bits that will be shifted out have unknown expected values, $00 are the following 8 bits to shift in (to cells 0..7), $50 contains the expected values of cells 0..3 (the value captured in cell 0 will be the last to arrive in TDO) and finally $F0 indicates that expected values are known only for the last four TCK cycles).

### Synchronisation with external test resources

Table 4.2 presents the instructions for synchronisation with external test resources, assuming the availability of two synchronisation channels (A and B).

| Mnemonic | Operation |
| --- | --- |
| SSA0, SSA1, SSB0, SSB1 | Sets sync output A or B to 0 (SSA0, SSB0) or 1 (SSA1, SSB1) |
| WSA0, WSA1, WSB0, WSB1 | Waits for sync input A or B to be set (WSA1, WSB1) or reset (WSA0, WSB0) |

Table 4.2: Instructions available for synchronisation with external test resources.

### Control of internal resources and test program flow

The instructions available for controlling internal resources and test program flow are shown in table 4.3, assuming the existence of eight error flags per BS chain.

| Mnemonic | Operation |
| --- | --- |
| LD CNT,N | Loads an internal counter with the required number of TCK cycles |
| SERFLG0, ..., SERFLG7 | Selects the error flag to be set upon fault detection |
| JPE Address, JPNE Address | Jumps to the specified address if the selected error flag is set (JPE) or reset (JPNE) |
| HALT | Stops test program execution |

Table 4.3: Instructions available to control internal resources and test program flow.

Our test controller model assumes that the LD CNT,N instruction has to be included before the following instructions: NTCK, NSHF and NSHFCP. Naturally, there would be alternative ways to specify the number of TCK cycles required (and we might even argue that NSHF and NSHFCP should not require to be preceded by LD CNT,N because the number of TCK cycles required is simply the number of bits in the respective bit streams). However, we must keep in mind our objective of presenting a test controller model that makes no assumptions restricting the possible implementation alternatives, which in the case of a dedicated test controller chip would have the bit streams following the NSHF / NSHFCP opcodes stored in memory and could not therefore know how many TCK cycles were required.

## Test program generation

The BS test controller model presented in this section comprises a total number of 28 instructions, belonging to the three types described. This instruction set enables us to specify test programs for any board with one or two BS chains, with or without non-BS clusters, provided that no more than two synchronisation channels are required to apply / capture test vectors / responses to the board primary I/O pins or cluster I/O pins.

The instruction set proposed has a very low abstraction level, meaning that test program generation is a tedious and error-prone task if done manually. The ideal situation would naturally be that an automatic test program generation tool is used to produce the test program for each case, from a set of input files containing the board structural information (netlist), the description of the BS architecture in each BS IC and the external test vector sets for non-BS clusters testing.

However, we must recall that the proposed test controller model has a pedagogical objective that would not be achieved if the small details of controlling the BS chain at bit-level were omitted.

4.	A BS test controller model

4.1	Basic test operations	4.1

4.1.1	Basic test operations to control the BS infrastructure	4.1

4.1.2	Basic test operations to synchronise the BS infrastructure with external test resources	4.2

4.1.3	Basic test operations to control internal resources and test program flow	4.3

4.2	The test instruction set	4.4

4.2.1	Control of the BS infrastructure	4.4

4.2.2	Synchronisation with external test resources	4.6

4.2.3	Control of internal resources and test program flow	4.7

4.3	Test program generation	4.7