# An introduction to Built-In Self-Test (BIST)

Following our analysis of design for testability and boundary-scan in general, we are now ready to consider the extension of these resources in order to allow the realisation of self-test functions at IC level. The complexity of this topic, as well as the general nature of this text, recommend however that we restrict our study to an introductory level, which therefore becomes the objective of this chapter.

The existence of built-in self-test (BIST) functions is nowadays a fundamental pre-requisite for manufacturing complex circuits, since their production test cost would be prohibitive if all the test operations had to be performed externally. It is therefore necessary to find a suitable trade-off solution for the inclusion of testability infrastructures and BIST functions, since the test overhead (namely additional silicon area and degradation of circuit performance) increases with the achieved fault coverage, increasing cost and eventually decreasing the probability of success as a commercial product.

We will start by an overview of the BIST architecture at IC level and proceed from there to consider several alternatives for the implementation of BIST functions in combinational circuits, sequential circuits and macro-cells. Following the analysis of the main BIST structures, we will consider their interface with the BST infrastructure, namely through the RUNBIST instruction, and their contribution to hierarchical self-test strategies. To conclude this chapter, we will consider the design for testability and BIST features in a real example: the Intel Pentium Pro processor.

## BIST architecture at IC level

In general terms, BIST is present whenever all the resources required for test execution – namely for test vector generation / application and response capturing / evaluation – are internal to the circuit. This definition provides ample room for different implementation alternatives, which at IC level will essentially take place in hardware (this being the scope of this chapter). Firmware is another implementation alternative at this level, even if more common at board level (mainly in the case of microprocessor / microcontroller -based designs), and software is a common implementation technology for BIST at system level.

The main issues to consider when specifying BIST functions at IC level may be summarised as follows:

- What fault coverage should be achieved through the BIST functions?

- What is the acceptable test overhead? (as regards silicon area and degradation of circuit performance)

These two questions pave the way to a cost / benefit analysis that is responsible for defining the amount of BIST functions to include in each circuit, taking into account that self-test resources significantly influence test overhead and have substantial implications in the design phase (time, people and tools). These cost factors should however be considered in parallel with the corresponding benefits, be it in terms of a much faster transition from prototype to production (mainly if a test and debug infrastructure is present), of smaller production test times (the main reason of BIST) and eventually of simplified maintenance and diagnosis operations.

Figure 7.1: BIST architecture at IC level (general overview).

In general terms, the BIST architecture at IC level may be represented as shown in figure 7.1, where the internal circuit under test is surrounded by three types of blocks:

- A block associated with the circuit primary inputs, which is able to operate in transparent mode or in test vector (or TP, test pattern) generation / application mode

- A block associated with the circuit primary outputs, which is able to operate in transparent mode or in response capture / evaluation mode

- Finally, a block that controls the self-test schedule and determines the operating mode of the two blocks referred previously

### Test vector generation and application

There are three main alternatives for implementing the TP generation / application block:

- The smallest number of test vectors able to achieve the required fault coverage may be generated by an automatic test pattern generator (ATPG) tool and stored in an on-chip ROM.

This alternative leads to the smallest test application time, since we use the smallest number of test vectors, but has an evident drawback: the silicon area required for storing the test vectors in ROM leads to a test overhead that is unacceptable in the majority of cases.

- Exhaustive testing, where all possible input combinations are applied to the circuit, or pseudo-exhaustive testing, where all the input combinations are applied only to sub-blocks (where the outputs are only dependent upon the sub-block inputs).

Contrary to what happens in the previous case, where the number of test vectors was smallest, this alternative leads to very high test application times and for this reason becomes unfeasible except in very small circuits (what would be the time required – measured in years – to apply all input combinations to a 32-bit ALU, assuming a 1 GHz test vector rate?). Pseudo-exhaustive testing would enable faster test times, but the complexity of circuit partitioning (required to determine the disjoint sub-blocks) is a highly complex problem, for which there are no algorithms known to execute in polynomial time.

- The third alternative is by far the most common approach and consists of applying to the circuit under test a set of pseudo-random test vectors (PRPG, pseudo-random pattern generation).

Pseudo-random testing is based on the mathematical properties of pseudo-random sequences, which show higher fault detection capability than the binary sequences normally associated with exhaustive testing (for the same number of test vectors). The hardware structures used to generate pseudo-random sequences are also simpler than those used to generate binary sequences, which constitutes an additional argument in favour of PRPG. The same fault coverage is therefore achieved with a smaller number of test vectors, associating low test overhead (in silicon area) and high speed (small number of test vectors).

### Response capture and evaluation

Response capture / evaluation can be implemented essentially through two main alternatives, as follows:

- The expected responses may be stored in ROM and compared with the captured responses on a vector-by-vector basis.

This procedure may be used with any of the test vector generation alternatives referred previously, but again shows an evident drawback: the silicon area required for ROM storage. Mainly when it is required that every response be stored in ROM, this solution becomes unfeasible for the majority of cases.

- Response compaction through one of several alternatives available, e.g. transition counting, 1s counting, signature analysis.

Besides being compatible with any of the test vector generation alternatives described earlier, this procedure shows the additional advantage of enabling fault detection by analysing a compressed representation of the responses captured for each of test vectors applied. Response compaction also eliminates the problem of silicon area overhead, while minimising the test application time, since the comparison operation takes place only once by test session (and not once for each test vector). The main problem associated with response compaction consists of the possible occurrence of fault masking (aliasing), since the presence of a fault does not necessarily lead to a compressed response different from the fault-free situation.

### BIST controller

This block is responsible for controlling the different phases comprising the self-test sequence (according to the type of sub-circuits to test) and by scheduling the operations that are required to take place in each phase. Considering for example the case of pseudo-random test vector generation and response compaction by signature analysis, the BIST controller will initialise the test blocks involved, switch their operating mode from transparent to test, apply the required number of test clock cycles (for test vector generation and response compaction) and finally close the self-test procedure, making the test result available to the external test controller. The interface with external test resources is in fact one of the main functions provided by the internal BIST controller, due to its importance in terms of the overall system level test strategy.

The implementation alternatives for each of the three blocks represented in figure 7.1 will now be analysed in the following sections, first for the case of combinational and sequential circuits, and then for macro-cells including sub-circuits of these two types.

## BIST of combinational circuits

The class of combinational circuits is where the problem of testability assumes its lowest complexity, since these circuits don’t have “memory” (internal state information) and are frequently designed in the form of regular structures with well-defined properties. These characteristics enable the application of all the alternatives previously considered, be it for test vector generation / application or for response capture / evaluation. The advantages and drawbacks described for each alternative explain why pseudo-random test vector generation and response compaction by signature analysis are the most popular solutions, which for this reason will be considered in greater detail in the following sections.

### Pseudo-random test vector generation

Pseudo-random test vectors are normally generated by a structure known as a linear feedback shift register (LFSR), comprising flip-flops (FF) and XOR gates interconnected in a way similar to a shift register, where the feedback path leads to a cyclic behaviour, through a pre-defined set of states. The XOR gates may be placed in the feedback path or in-between some of the FF, but not all possible configurations lead to a maximum-length cyclic behaviour, which corresponds to 2(N+1) states (as we will see later, an all-0 combination in the FF outputs locks the LFSR permanently in this state).

Pseudo-random test vector generation may be analysed in detail by considering the simple example of a circuit whose internal logic consists of a 2:1 multiplexer, to be tested by PRPG carried out by a 4-bit LFSR, as shown in figure 7.2.

Figure 7.2: A 4-bit LFSR for pseudo-random test vector generation.

Any solution of the type shown above requires consideration of the following set of questions:

- What connections should be made between the circuit under test and the LFSR?

Considering that the LFSR shown is of the primitive polynomial type (i.e. its cyclic behaviour has the maximal length of 2(N+1)), and that the number of LFSR outputs is larger than the number of inputs in the circuit under test, we are free to decide which connections to establish between (Q0,Q1,Q2,Q3) and (A,B,C). Notice however that the number of test vectors to apply in order to reach a pre-defined fault coverage will depend upon the connections between the LFSR and the circuit under test, and it may be necessary to perform fault simulation, as referred in the following paragraph.

- What is the required number of test clock cycles?

Since each test clock pulse applied to the LFSR leads to a new test vector in its outputs, it becomes easy to conclude that 15 clock cycles are sufficient to guarantee that all possible combinations are applied to the inputs of the circuit under test. This solution would in fact perform an exhaustive testing of the multiplexer, which would eliminate one of the advantages of pseudo-random testing: a small number of test vectors. In fact, the main reason supporting pseudo-random testing consists of achieving high fault coverage with a simple hardware structure (the LFSR), in a reduced test time (small number of test clock cycles). Although the minimum number of test vectors required with PRPG is larger than what would correspond to a deterministic test (since the deterministic test vectors are optimised by the ATPG tool), it is still far smaller than the number of all possible combinations at the circuit inputs. A fault simulation session is therefore required to determine the minimum number of LFSR clock pulses, i.e. the minimum number of pseudo-random test vectors necessary to achieve the required fault coverage. To conclude, it should be referred that the initial value of the LFSR outputs is not relevant – provided that it is not locked in the all-0 state –, since the sequence of test vectors generated is always the same (the initial value only determines the starting point in this sequence).

- Would it be possible to use a 3-bit LFSR?

It would in principle be possible to use a 3-bit LFSR implementing a primitive polynomial (able to generate seven test vectors), unless we need to achieve 100% fault coverage and there is at least one fault which can only be detected by test vector 000 (notice that an LFSR with more than 3 bits enables us to apply the all-0 combination at the circuit inputs).

- How do the pseudo-random and exhaustive test vector generation alternatives compare in terms of test overhead?

In general terms, test overhead is assessed according to three main factors: the test preparation time, the importance of the additional resources required for its implementation and the test application time. With respect to the first of these factors, pseudo-random testing requires access to fault simulation if we wish to determine the smallest number of test vectors (this being the common situation). Fault simulation is obviously not required for exhaustive testing, but we should also recall that this requisite relates solely to the design time. Concerning the additional resources required for self-test implementation, and their implications over circuit performance (e.g. additional silicon area, test dedicated pins, higher propagation delays, etc.), these two alternatives are essentially similar. An LFSR structure is in fact simpler than a binary counter, but we should notice that an LFSR might also be used for exhaustive test vector generation (and the difference in total silicon area is possibly not relevant). Finally, and in relation to the test application time (which is important to determine the cost of a component), the much smaller number of test vectors that is traditionally associated with pseudo-random testing constitutes an important advantage recommending this alternative for most situations.

### Response compaction by signature analysis

An LFSR may also be used to implement a response compaction block, producing a signature that corresponds to the LFSR contents (the FF outputs), when the test is completed. Figure 7.3 shows an example where the internal circuit under test is again the 2:1 multiplexer and the signature register is implemented by an LFSR in SISR (single-input signature register) configuration.

Figure 7.3: A 3-bit LFSR for response compaction by signature analysis.

Once again, we will start from the example shown and discuss the main questions to be taken into account:

- What connections should be made between the circuit under test and the LFSR?

It is evident in this case that the multiplexer output (F) should be connected to the LFSR input (D), since we have a single-output circuit under test and a single-input compactor. Notice however that there are also multiple-input signature registers (MISR), in which case the question considered has more than one answer.

- What is the required number of test clock cycles?

Since the LFSR contents are updated with each clock cycle, according to the value present at its input and the state of each FF, the number of test clock cycles is not imposed by the response compactor, but instead by the test vector generator (in the case of pseudo-random testing, as we have already seen, this number is determined through fault simulation).

- What should be the initial value in the LFSR outputs?

The starting value in the LFSR outputs is not pre-determined, since different initial contents will simply lead to different end signatures, according to the expected sequence at the outputs of the circuit under test. The initial value must however be known, so that we are able to determine the expected signature in the fault-free case, either mathematically or by logic simulation.

- Will the probability of fault masking vary with the number of bits in the signature?

The mathematical theory underlying response compaction by signature analysis shows that the signature register performs a polynomial division, the final signature being the remainder of this arithmetic operation. Intuitively, and if we consider that all the signatures are equally probable, it becomes easy to conclude that the number of faults which produce the correct (fault-free) signature will decrease as the number of bits in the signature increases (parity detection, which is in fact the case of a single-bit SISR, allows the occurrence of masking in a relatively large number of situations).

Other response compaction methods are possible besides signature analysis, such as transition counting and syndrome counting (counting the number of 1 in the circuit outputs). The reader is invited to determine the results of response compaction through each of the three methods referred, considering the 2:1 multiplexer and the application of an exhaustive test to its inputs (in which case will the probability of aliasing be higher?).

## BIST of sequential circuits

For reasons which are already known, sequential circuits are far more complex to test than combinational circuits, this being the reason the led to the development of design for testability techniques (including BIST). The same reasons explain why not all of the techniques presented so far for self-test can be reused with sequential circuits. Due to the existence of internal state information, we may have different output sequences for the same input sequences, precluding the use of such approaches as pseudo-random test vector generation. Deterministic test vectors stored in ROM may be used in this case, but the drawbacks (in terms of test overhead) that were previously referred prevent this solution as well, even if an efficient ATPG tool was available for this type of circuits. Notice however that the methods described for response capture and evaluation may be reused with sequential circuits, since the alternatives referred are valid regardless of the type of circuit under test.

The problem of BIST for sequential circuits is however not so complex as we might initially anticipate, since manufacturability issues will in the majority of cases dictate that design for testability techniques are enforced from early in the design phase, and namely scan design, which for test purposes eliminates the sequential nature of the circuit. Under these circumstances, all the methods presented so far are equally useful for sequential circuits, where the existing state FF may in fact be reused for pseudo-random test vector generation and signature analysis.

## BIST of macro-cells

The introduction of a macro-cell type of block, which is not restricted to one of the two types of circuits considered previously, enables us to reuse complex circuit blocks, internally comprising networks of combinational and / or sequential circuits and eventually including their BIST structures as well. This situation is relatively common for ASIC designers, who normally use libraries of circuit blocks widely variable in terms of functionality and complexity. Once the macro-cell BIST structures are known (either specified by the designer, according to the techniques previously referred, or because they are already part of the macro-cell specifications), what remains to be done is to decide on how to integrate them into the overall BIST strategy. This problem is in fact an instance of the wider issue of hierarchical test, which normally departs from the macro-cell level and extends upwards as far as the system level (set of printed circuit boards). We have stated from the beginning that the scope of our study is essentially restricted to the IC level, but it is nevertheless important to situate this problem in the global context and to discriminate between the two situations that will be referred in the following sections.

### Low / medium complexity macro-cells

Low / medium complexity macro-cells generally require relatively homogeneous BIST resources and as such there are well known solutions for most of the  circuits in this class. Memory blocks are a typical example in the medium complexity range, for which the inclusion of BIST resources normally does not create relevant problems:

- ROM BIST is frequently done by scanning through all the addresses and compacting the corresponding contents into a signature, which is in fact a case of exhaustive test vector generation and response compaction through signature analysis.

- BIST of RAM blocks presents a case of somewhat higher complexity, due to the variety of fault models to be considered, but a large number of test algorithms is described in the literature for this purpose.

Integrating BIST of low / medium complexity macro-cells into the overall system test strategy does not therefore raise significant problems, since the required test methods and resources are normally available.

### BIST of megacells: the IEEE P1500 proposed standard

BIST of higher complexity macro-cells, e.g. a microcontroller, including several types of circuits and eventually low / medium complexity macro-cells as well, requires the definition of more powerful solutions to facilitate their integration into the overall system test strategy. The fact that the BIST solutions used within these blocks are normally widely heterogeneous, as well as the need to protect intellectual property rights, led to an IEEE standard for this purpose which is currently under development: the IEEE P1500 proposed Standard for Embedded Core Test.

The IEEE P1500 proposed standard will develop a “standard test method for Integrated Circuits containing embedded cores, i.e. reusable megacells. This method will be independent of the underlying functionality of the Integrated Circuit or its individual embedded cores. The method will create the necessary testability requirements for detection and diagnosis of such Integrated Circuits, while allowing for ease of interoperability of cores originated from distinct sources. This method will be usable for all classes of digital cores including hierarchical ones.” A more in-depth description of the objectives and contents of this proposed standard goes well beyond the objectives of this document, but the interested reader may find out more about IEEE P1500 at URL http://grouper.ieee.org/groups/1500.

## Interface between BIST and the BST infrastructure

The BST infrastructure constitutes an excellent vehicle to access self-test functions, BIST dedicated pins being in principle no longer necessary in a BST component. Besides enabling all BIST-related operations to be triggered through the TAP, the IEEE 1149.1 standard defines an optional RUNBIST instruction which provides a template to specify all the necessary details concerning access and execution of BIST functions (independently of the circuit manufacturer and functionality).

### Architecture of a BST component with BIST

The architecture of a BST component with BIST may be represented in general terms as illustrated in figure 7.4, which shows several internal logic blocks and their surrounding BIST resources, a global IC BIST controller and the register which retains the self-test result (the optional register designated as “BIST data” in figure 7.4).

Figure 7.4: General overview of the test infrastructure in a BST component with BIST.

The global IC BIST controller is required to schedule the operation of the corresponding structures present in each block, which may or may not take place simultaneously (some of the BIST resources may indeed be common to more than one block). Notice also that the BST infrastructure itself may be responsible for part of the BIST operations, be it in terms of the BIST controller or in the generation / application of test vectors and response capture / evaluation (e.g. by using the BST register for pseudo-random test vector generation and response compaction by signature analysis).

### The RUNBIST instruction revisited

The main specifications of the IEEE 1149.1 standard concerning the optional RUNBIST instruction may be summarised as follows:

- The self-test modes of operation shall execute only when the TAP controller is in the Run Test / Idle (RT/I) state

- A duration shall be specified for the self-test (required number of test clock cycles)

- The test data register selected by RUNBIST shall contain the self-test result after the specified number of test clock cycles were applied, in which case the result may be read by moving from RT/I to Shift-DR (S-DR), and scanning out the register contents

BIST execution in a BST component therefore proceeds according to a protocol which can be summarised as follows: i) scan any required seed values into the registers requiring this initialisation procedure; ii) scan the RUNBIST code into the instruction register; iii) move the TAP controller to the RT/I state and apply the number of test clock (TCK) pulses specified by the manufacturer; iv) move the TAP controller to the S-DR state and scan out the self-test results.

### Hierarchical BIST

The existence of BST components with BIST facilitates the implementation of a hierarchical test structure, where the system level test functions (normally implemented in software or in firmware) co-ordinate the operation of the same functions at board level (now probably implemented in firmware), which in turn interface the BIST structures present at IC level (mainly implemented in hardware, according to the techniques described). The common solution would in this case consist of placing a BIST controller IC in each board, eventually in the form of a dedicated processor, which would be responsible for controlling the board level BST infrastructure and the component level BIST functions.

The existence of the optional RUNBIST instruction enables the testability infrastructure used at board level (BST) to trigger component level BIST functions, therefore pre-defining a hierarchical test strategy at board level and below (printed circuit board  component  (and eventually) macro / mega cell, mainly when the IEEE P1500 proposed standard becomes formally approved). The existence of BST at board level raises the possibility of extending this test infrastructure also to system level, which would enable the implementation of a hierarchical test strategy with minimal test overhead. The restrictions faced by this possibility are essentially related to the fact that BST was developed to facilitate the structural testing of digital printed circuit boards, meaning that the organisation of system level scan chains will have some limitations to overcome (the reader is invited to identify some of these limitations).

Extending the testability infrastructure to the upper level (printed circuit board  system) is not a trivial task, even considering that an IEEE standard for this purpose has been available for some years now: the IEEE 1149.5 Standard for Test and Maintenance Bus (MTM-bus), which defines a system level test bus comprising 4+1 lines: MMD (Master Data), MSD (Slave Data), MCTL (Control), MCLK (Clock) and MPR (Pause Request). The testability infrastructure defined by the MTM-bus differs from BST, and this has been an obstacle to its wide acceptance so far, since a hierarchical test solution employing the same standard would optimise the investment in test infrastructures. As such, several proposals with this objective have been presented in recent years and there are some BST components commercially available that enable the implementation of global self-test strategies, from system level down to the macro / mega cell level.

## Design for testability and BIST in the Pentium Pro processor

To close our study of issues related to BIST, we will conclude this chapter with a brief analysis of the design for testability and BIST solutions employed in a real example: the Intel Pentium Pro processor. The description of the test infrastructures present in this component is available from easily accessible publications (see for example A. Carbine and D. Feltham, “Pentium Pro Processor Design for Test and Debug,” IEEE Design and Test of Computers, July-September 1998, pp. 77-82) and has the advantage of  going beyond the technical aspects, having in fact the objective of presenting a global perspective of the advantages and limitations which have to be taken into account for similar design cases.

The Pentium Pro design team had a very clear objective of finding the best trade-off solution between the silicon area reserved for functional purposes and for test purposes, a concern which brings into evidence the fact that each new microprocessor generation is fundamentally bound in terms of the resources required for the implementation of the underlying architectural decisions. Excluding design for testability is obviously out of the question, but it is important to stress that the silicon area used for this purpose could instead be used to improve circuit performance, which would lead to higher probability of commercial success. The main principles that guided the definition and design of the Pentium Pro design for testability infrastructures were the following:

- “Have zero performance impact”

- “Have minimal die area impact”

- “Be multiuse features wherever possible (supporting component debug, production test, and so on)”

- “Be designed in from the start (that is, coded and validated in the register-transfer-level model)”

Starting from here, the design for testability infrastructures present in the Pentium Pro processor are summarised in table 7.1, extracted from the paper referred previously (silicon area figures refer to the processor version that includes a 256 Kbytes cache memory, using a total of approximately 21 million transistors).

Table 7.1: Summary of the testability features present in the Pentium Pro processor.

It is worth to stress the following aspects, concerning the information present in this table:

- The Pentium Pro BST infrastructure supports seven public instructions and “many more private instructions” used for test and debug operations

- The Scan-out mode of operation employs scan design techniques to enable observation of more than 2.000 internal nodes, either in snapshot (the internal state is sampled and scanned out) or signature (continuous sampling and compaction into a signature) modes

- The Debug mode stops the processor and enables interactive test operations, following which the processor may return to the normal execution mode

- The Internal breakpoint mechanism can be set up for several internal events and for the subsequent debugging actions (taking place after a programmable time delay)

- Microcode update enables the temporary modification of microcode sections to facilitate test and debug in early silicon versions

- The IDDQ mode uses a private BST instruction to disable all devices that draw static current and enable IDDQ tests

Specifically concerning the BIST features supported in the Pentium Pro processor, the following issues are worth mentioning explicitly:

- There is a self-test routine that is “primarily targeted at achieving high toggle coverage for burn-in testing”

- Most BIST functionality was implemented in microcode, but LFSR blocks were used to implement BIST structures for “several inaccessible programmable logic arrays”

- The L2 cache memory (256 Kbytes in the version considered here) has power-on self-test structures which provide an “effective early filter in the production test flow.” Additionally, an L2 PBIST (programmable BIST) block was implemented which “provides an on-chip, fully programmable array test pattern generator, including a small vector memory and a programming language” (this being “the main mechanism through which the L2 cache die is tested in production”).

To conclude, it is important to stress that production test effectiveness is ultimately measured by the number of defective components that are shipped to customers (in parts per million), and not by fault coverage figures. The Pentium Pro processor used 4% of the CPU area and 6% of the L2 cache memory area for design for testability and BIST purposes, with “no negative impact on processor performance, either in clock frequency or instructions per clock.”

## Hands-on

The main objective of this exercise consists of applying the basic principles underlying BIST through pseudo-random test vector generation and signature analysis. The simple BST infrastructure that was previously described in chapter 2 should in this case surround an internal core logic consisting of a 2:1 multiplexer, as shown in figure 7.5 below. The blocks used for pseudo-random test vector generation and signature analysis should be those illustrated in figures 7.6 and 7.7.

Figure 7.5: Internal circuit logic.

Figure 7.6: Block recommended for pseudo-random test vector generation.

Figure 7.7: Block recommended for response compaction by signature analysis.

The following recommendations should be taken into account:

- The BST register should be used to implement the BIST blocks shown in figures 7.6 and 7.7.

- The 10 code (which was unused) should be allocated to the RUNBIST instruction.

- The new BST register should have seven cells, four of them based on the common BS cell configuration, enhanced to support the required PRPG / SA operating modes. The remaining cells should be as simple as possible, in order to complete the seven stages that are required for PRPG and SISR. The use of multiplexers in the BST register should enable the mandatory EXTEST and SAMPLE / PRELOAD instructions to exclude the extra cells from the scan path, which will only be present when the optional instruction RUNBIST is loaded.

7.	An introduction to Built-In Self-Test (BIST)	1

7.1	BIST architecture at IC level	1

7.1.1	Test vector generation and application	3

7.1.2	Response capture and evaluation	4

7.1.3	BIST controller	4

7.2	BIST of combinational circuits	5

7.2.1	Pseudo-random test vector generation	5

7.2.2	Response compaction by signature analysis	8

7.3	BIST of sequential circuits	10

7.4	BIST of macro-cells	10

7.4.1	Low / medium complexity macro-cells	11

7.4.2	BIST of megacells: the IEEE P1500 proposed standard	11

7.5	Interface between BIST and the BST infrastructure	12

7.5.1	Architecture of a BST component with BIST	12

7.5.2	The RUNBIST instruction revisited	13

7.5.3	Hierarchical BIST	14

7.6	Design for testability and BIST in the Pentium Pro processor	15

7.7	Hands-on	18

Arithmetic and Logic Unit.

Application-Specific Integrated Circuit.