# Test protocol for BS boards

The process of testing a BS board takes place in a sequence of steps that starts by checking the integrity of the BS infrastructure itself, followed by two main types of tests: interconnects and components. These two types of test operations are frequently subdivided into further steps, namely:

- Full BS interconnects test (for open and short-circuit faults) and non-BS cluster interconnects test

- Components with and without BIST

The expression full-BS interconnects refers to those interconnects that do not include pins of non-BS ICs (primary input / output pins may however be present, as well as Ground or VCC connections, besides the BS pins).

Each of the steps referred above will now be detailed in the following sections.

## Fault detection in the BS infrastructure

Two main types of faults may affect the integrity of the BS infrastructure: Faulty TAP interconnects (open or short faults in the TDI, TDO, TMS, TCK or /TRST pins) or faulty / misplaced components. The sequence of operations usually carried out to detect these faults starts with the initialisation of the BS logic in each component (the TLR TAP controller state can be reached from any other state by the application of a 1 in TMS during not more than 5 TCK cycles), which is followed by:

- A capture-and-scan operation through the instruction registers (IRs) in all BS components.

- A capture-and-scan operation through the identification registers of those BS components supporting this optional instruction.

The IEEE 1149.1 std states that the result of a capture operation in the IR (taking place in the Capture-IR TAP controller state) must be X…X01, where X represents a “don’t care” condition and the rightmost bit is closest to TDO. This feature was defined with the objective of enabling the simple integrity check operation shown in figure 3.1, that consists of bringing the TAP controller FSM to the Capture-IR state and then on to Shift-IR and to scan out the IR contents in all BS ICs.

Figure 3.1: Integrity checking of the BS chain by capture-and-scan through the IRs.

Since the two least significant bits in each IR are 01, every TDO-TDI connection will be forced to 0 and to 1, which enables the detection of a large spectrum of faults in the TAP pins:

- Any stuck-at TAP pin will disrupt the 01 sequence shifted out and be detectable at TDO of the last BS IC (the “board-level TDO”).

- Most short-circuit faults involving TAP pins will have a similar effect and therefore be detectable in the same way.

- Open interconnects among TAP pins are generally easy to detect, since TDI and TMS are defined in the standard as capturing a logic 1 when floating, which again disrupts the sequence and is easily detectable.

Some specific faulty conditions might remain undetected by this simple test, but these situations would normally prevent correct operation in the following steps of the test protocol and as such be rapidly detected (alternatively, a more sophisticated infrastructure test may be specified). An example of one of these specific faults might be a short circuit between TDO and TDI of the same TAP, in a chain where all ICs have the same specification for the IRs. The reader is invited to explain why this particular fault would not be detected by this test and to suggest a way to detect it.

Boards with more than one BS chain may have their BS infrastructures checked independently, but again there are a few specific faults which will only be detected if the integrity check is co-ordinated in all chains. An example of one of these faults might be a short circuit between two TDO pins in different chains, which will be in high-impedance except when scanning is in progress. If one BS chain is in TLR state while the other is being checked, any short-circuit between two TDO pins in different chains has no effect, since only one of them will be active. Again the reader is invited to specify a co-ordinated integrity check procedure that is able to detect the situation referred above for the case of multiple BS chains.

If the capture-and-scan test runs without problems there is a high probability that no faults are present in the BS chain. However, and if there are components supporting the optional IDCODE or USERCODE instructions, it is recommended that these instructions are used to check that the proper components were placed where they are expected to be.

## Open fault detection in full-BS interconnects

Once a simple integrity check is concluded with success, the next step consists of checking that open or short circuit faults are not present in the remaining board interconnects. It should be recalled that power has to be applied to the board while testing through the BS chain, meaning that potentially destructive short-circuit currents should be detected as early as possible. In fact, the only two types of tests carried out before looking for short-circuit faults are the BS chain integrity check and the application of test vectors for the detection of open faults, in both cases because of the small number of test vectors involved.

Proceeding to the open fault detection phase is done by shifting into the IRs the SAMPLE / PRELOAD instruction, which selects the BS register in each IC and enables the first vector to be shifted in. Once each output latch in the BS cells has been loaded with the first logic value to be applied to each pin, the EXTEST instruction is shifted into all IRs, forcing the mode multiplexer to decouple the internal logic from the pins and leading to the application of the first vector.

The generation of test vectors to detect open faults in full-BST interconnects is based on the practical experience of dealing with floating inputs (TTL inputs will normally capture a 1 while CMOS inputs are not predictable). Open faults may therefore be detected by driving each interconnect to 0 and checking the value captured at the corresponding input pins, followed by the same operation, but now driving each interconnect to 1. In interconnects with multiple driving pins, as shown in figure 3.2, detection of all open faults will only be guaranteed if the previous procedure is repeated through each driving pin in turn (the remaining driving pins in the same interconnect will meanwhile be kept in the high-impedance state).

Figure 3.2: Testing for open faults in interconnects with multiple driving pins.

The number of test patterns required for open fault detection is therefore equal to the double of the maximum number of driving pins existing in a single interconnect, meaning that the number of test vectors required to provide complete open fault coverage is normally very low.

## Short-circuit fault detection among full-BS interconnects

As soon as the open fault detection phase is concluded, and since the EXTEST instruction is already present in the IRs, the first vector for short-circuit fault detection may be shifted in. Since all input pins belonging to two short-circuited interconnects will see the same voltage level, it is reasonable to expect that their BS cells will capture the same logic value. Test vector generation for detection of short-circuit faults is therefore based on the principle of driving opposite logic values to the interconnects under test. For interconnects with multiple driving pins (and since the previous step has shown that no open faults are present), only one of them will be selected to drive the interconnect.

Short-circuit faults are harder to deal with, not only because of their potentially destructive effect, but also because the number of possible faults is much larger than open or stuck-at faults. The number of possible short-circuit faults among N nodes is given by 2N – (N+1), growing exponentially with the dimension of the circuit. It is true that some short-circuit faults will be less likely than others (a short-circuit among all interconnects is highly improbable), but this fact is hardly relevant for test vector generation purposes. On the other hand, complete short-circuit fault detection will be possible if opposite logic values are applied to all possible interconnect pairs, since any short-circuits involving 3 or more interconnects will be detected as well. This fact brings our attention to the distinction between fault detection and fault diagnosis, the former being much simpler to achieve.

A large number of papers have been published describing test vector generation algorithms for detection of short-circuit faults, trading off number of vectors for diagnostic resolution. The first of these algorithms was proposed in 1974 by W. H. Kautz (not for boundary scan boards, which at the time did not exist) and uses the binary partition principle to guarantee complete fault detection with a minimum number of test vectors (and with minimum diagnostic resolution, as will be expected).

Binary partition consists of starting with a test vector that applies a logic 0 to half the interconnects and a 1 to the other half, so that any short circuit between interconnects in different halves will be detected. However, and since short-circuits among interconnects in the same half will escape detection, the procedure is again repeated, but now further dividing each half in two halves, and so on until the last partition, which leads to single interconnects. The application of this algorithm to the very simple case of a circuit with 8 interconnects is shown in figure 3.3.

Figure 3.3: The binary partition algorithm for short-circuit detection.

The binary partition algorithm guarantees complete short-circuit fault detection with the smallest number of test vectors, given by log2(N), where N is the number of interconnects and X (ceiling of X) represents the smallest integer not lower than X (10 test vectors will therefore be able to provide 100% short-circuit fault detection in a board with 1.000 interconnects).

## Full-BST interconnect testing in boards with multiple BS chains

Open fault detection in boards with multiple BS chains can be done sequentially or simultaneously in each chain, since each open fault does not affect more than one interconnect. A different situation takes place when considering short-circuit faults, since their detection requires the two shorted interconnects to be active (and with opposite logic values applied) at the same time. The binary partition algorithm presented in the previous section guarantees this condition, but further attention has to be given to the case of boards with multiple BS chains, since it is not possible to ignore the case of two interconnects belonging to different chains.

The binary partition algorithm will generate the test vectors for the whole group of interconnects in the board, regardless of how many BS chains exist or of what particular BS chain(s) each interconnect belongs to. However, the application of test vectors in each chain has to be done in a co-ordinated manner, through a protocol that may be described as follows for each vector:

- For the first BS chain, shift the new vector in through the BS register cells and proceed to the update-DR and select-DR TAP controller states after the last bit is shifted in.

- Repeat the previous operation with all remaining BS chains. The necessary conditions to detect all short-circuit faults detectable with this test vector will only be set when all BS chains have gone through the update-DR state.

- Again in the first BS chain, capture the test vector responses present at the BS cell parallel inputs. Since the TAP controller of each IC was left in the Select-DR state, applying one TCK cycle with TMS=0 brings the TAP controller to Capture-DR, where the falling edge of TCK will trigger the capture operation.

- Repeat the previous operation will all remaining BS chains. Capturing the test vector responses will only be complete when this step is concluded for the last BS chain.

This test protocol for boards with multiple BS chains guarantees correct application of the test vectors generated by the binary partition or by any other test vector generation algorithm.

## Fault detection in non-BS clusters

Fault detection in non-BS clusters (groups of non-BS components that are present in boards with BS) might be done exclusively through the surrounding BS components, but smaller test times and higher fault coverage may be achieved if external test resources are available as well. The BS infrastructure and external test channels may be combined mainly as represented in the three cases shown in figure 3.4.

| (a) Full access. | (b) Peripheral access. | (c) Primary I/O access only. |
| --- | --- | --- |

Figure 3.4: Different BS / physical test channel access combinations for testing non-BS clusters.

The symbol  is used in figure 3.4 to represent an external test channel with direct physical access to the PCB node. The three situations shown correspond to different trade off solutions in terms of test speed (and eventually fault coverage as well) versus cost (in resources of test equipment and physical access provisions), which may be summarised as follows:

- Full access (figure 3.4.a) means that the non-BS cluster may be tested independently of the surrounding BS chain, since all interconnects (internal or in the periphery) have an in-circuit test channel. This is the fastest solution (no scan operations involved), but requires extensive test resources and non-restricted physical access, which in many cases will not be possible.

- Physical access to those interconnects in the periphery of the non-BS cluster (figure 3.4.b) is an alternative which requires less test resources, due to the much lower number of physical access points. The external in-circuit test channels enable a functional test of the cluster, which will normally result in lower test speeds, but which still does not require any scan operations.

- When physical access is not at all possible except to the primary inputs and outputs of the board (figure 3.4.c), the only alternative left is to use the surrounding BS infrastructure to test the cluster. While requiring minimal test resources, this alternative is also the slowest, since (if a deterministic set of test vectors is to be used) a complete scan operation has to take place for each test vector.

Test vector generation for the non-BS cluster is done by an automatic test pattern generation tool according to a specific fault model and to the test resources that are assumed to be available (functional test of the cluster or in-circuit access to internal cluster nodes). For the case shown in figure 3.4.c, post-processing of the functional test vector set is then employed to serialise and insert the test stimuli in the proper locations in the BS chain(s).

As complex packaging and mounting technologies become more and more common, the situation shown in figure 3.4.c becomes the preferred choice. It should be pointed out that the need to scan in / out each test vector / responses may lead to unacceptably low test speed, for those cases when the BS chain(s) include hundreds / thousands of cells and the number of test vectors is very large. For certain types of circuits (for example, most combinational logic clusters), alternative test techniques may however be applied, such as pseudo-random test generation and response compaction by signature analysis, which enable test vector application at the rate of one test vector per clock cycle.

## Testing non-BS clusters in boards with multiple BS chains

The test methods illustrated in figures 3.4.a and 3.4.b are independent of how many BS chains exist in the board, since there are no scan operations involved. As for the situation represented in figure 3.4.c, where the BS infrastructure is used to test the cluster, the same protocol described for testing full-BS interconnects in boards with multiple BS chains should be applied.

## Faulty components detection

Recalling that BS was developed to become an effective structural test technology for digital PCBs, it comes as no surprise that there are limited facilities available for components testing. Since the exhaustive testing approach of successively shifting in and out test stimuli and responses is unfeasible in most cases, the two alternatives remaining are the following:

- When the optional RUNBIST instruction is supported (which is essentially the case of complex VLSI devices where the overhead of BIST structures is acceptable), this instruction is used to test the component:

- The RUNBIST instruction is loaded into the instruction register.

- The minimum number of TCK cycles specified by the manufacturer should be applied while the TAP controller is kept in the RTI state.

- The BIST result is now available externally by reading the contents of the selected (by the BIST instruction) data register.

- BS components which do not support a RUNBIST instruction can only be checked “alive or dead” by the application of a reduced number of test vectors through the BS register. However, the slowness of scanning in / out each test vector / responses prevents a meaningful functional test of the internal logic except in very simple components (which in most cases will not have a BS infrastructure, except specific testability components such as TI’s SCOPE octals).

It should also be referred that the IEEE 1149.1 std describes an INTEST optional instruction, meant to be used in the second situation described above, but which provides exactly the same result of the EXTEST instruction, when the BS cells are of type presented in chapter 2 (figure 2.3). Recall also that those components belonging to non-BS clusters will already have been tested during the cluster test phase, since those test vectors generated for this purpose will normally address components test as well.

3.	Test protocol for BST boards

3.1	Fault detection in the BS infrastructure	3.1

3.2	Open fault detection in full-BST interconnections	3.3

3.3	Short-circuit fault detection among full-BST interconnections	3.4

3.4	Full-BST interconnect testing in boards with multiple BS chains	3.6

3.5	Fault detection in non-BS clusters	3.7

3.6	Testing non-BS clusters in boards with multiple BS chains	3.8

3.7	Faulty components detection	3.8