# Glossary

Combinational circuit: Logic block where the outputs depend only on the current value applied to its inputs (each input combination corresponds to only one output combination).

Controllability: Parameter quantifying how simple / difficult it is to control the logic level in a given node, assuming access only to the circuit primary inputs (when referring to a circuit, it represents the average value of  the controllability of its nodes).

Defect: Physical condition that is capable of preventing the proper operation of a circuit.

Design for testability: A specific design methodology which enforces the adoption of rules aiming at improving the testability of a circuit. Ad hoc design for testability uses bespoken rules (according to each particular circuit), while structured design for testability methodologies use pre-defined rules of a general nature.

Design verification: Test operation that uses a circuit model to assess its compliance with the specification that guided its development.

Detection: Test operation which aims solely at identifying the presence / absence of faults / defects in a circuit (produces a binary result: yes / no).

Diagnostic: Test operation that aims at identifying the types of faults / defects present in a circuit (normally produces as much detail as possible, with the objective of facilitating repair operations or improvements in the manufacturing process).

Fault: Logic condition that prevents the proper operation of a node, or set of nodes, in the circuit model (a fault does not necessarily have an equivalent defect). A fault will be detectable if it is able to prevent the circuit model from producing the correct output, for at least one test vector applied to its inputs.

Fault coverage: Parameter quantifying the percentage of faults (according to the adopted fault model) which is detected by a given test vector set. It is important to stress that defective products can still escape detection, even when the fault coverage of the test vector set is 100% (depending on the quality of the adopted fault model).

Fault model: Abstract representation used to describe a set of logical conditions that are capable of leading the circuit model to produce responses different from expected. A good fault model possesses two main attributes, of fundamental importance for test vector generation purposes: simplicity (to facilitate the test vector generation process) and representativeness (to enable the generated test vectors to detect as many defects as possible).

Fault simulation: Operation to determine the expected response of a circuit model, in the presence of faults, when a given set of test vectors is applied to its inputs. Fault simulation plays a pivotal role for test vector generation purposes.

Full BST interconnects: Board-level interconnections where all the pins are either BST pins (which have associated one or more BST cells) or primary input / output pins of the printed circuit board.

Functional test: Test operation where the test channels are connected only to the primary input / output connectors of the printed circuit board under test (no physical / direct contact exists with internal nodes). The same expression is also used outside the automatic test equipment domain, to refer to a test operation where the logic function of a circuit is checked against its specification (as a complement to parametric testing, which checks for voltage and current levels, timing parameters, etc.).

Hierarchical test: Test operation which takes place at more than one level (macro-cell, integrated circuit, printed circuit board, system).

In-circuit test: Test operation that uses a bed-of-nails fixture to establish a physical / direct connection between the test channels of the automatic test equipment and the internal nodes of the printed circuit board under test (integrated circuit pins or passive component pins).

Logic simulation: Operation to determine the expected response of a circuit model, in the absence of faults, when a given set of test vectors is applied to its inputs. Logic simulation plays a pivotal role for design verification purposes.

Mixed-signal testing: Test operation dealing with mixed-signal circuits (including an analog component and a digital component).

Model: Abstract representation used to describe the main features of a physical entity. When applied to a digital circuit (logic modelling), a model corresponds to a textual or graphical description which enables us to anticipate its behaviour in response to any combination of logic values applied to its inputs.

Observability: Parameter quantifying how simple / difficult it is to observe the logic level in a given node, assuming access only to the circuit primary outputs (when referring to a circuit, it represents the average value of the observability of its nodes).

Partial BST board: Printed circuit board where only part of the components support BST (Boundary Scan Test).

Primary inputs: External circuit inputs (logic block, integrated circuit, printed circuit board, or even system).

Primary outputs: External circuit outputs (logic block, integrated circuit, printed circuit board, or even system).

Prototype validation: Test operation that uses a circuit prototype to assess its compliance with the user / customer requirements.

Scan design: Technique underlying the main structured design for testability methodologies, in which the flip-flop inputs come from a 2:1 multiplexer, enabling their interconnection as a shift-register. We may have full scan design or partial scan design, according to whether all or only part of the flip-flops present possess this feature.

Self-test: Test operation taking place inside the circuit, using internal blocks for test vector generation / application and response capture / evaluation (also referred as built-in self-test). Self-test may exist at several hierarchical levels, from macro-cell to system level.

Sequential circuit: Logic block where the outputs depend on the sequence of values applied to its inputs (each input combination may correspond to more than one output combination).

ss@ (single stuck-at) fault model: Fault model which considers only one fault at a time (single), causing the faulty node to be in one of the two following situations: always at 1 (stuck-at-1) or always at 0 (stuck-at-0).

Test: Operation to determine if a circuit, or a model of the circuit, deviate from its expected characteristics (functional, timing, parametric, etc.).

Test overhead: Parameter quantifying the additional resources required to implement specific features aiming at improving the testability of a circuit (test overhead is frequently expressed in terms of silicon area, but it should in fact relate to additional pins and to the implications over circuit performance as well).

Test vector: Combination of logic values that should be applied to the inputs of a circuit, to assess if it deviates from a given specification (functional, timing, parametric, etc.).

Testability: Parameter quantifying how simple / difficult it is to generate a test vector to detect the presence of a fault in a given circuit node (when referring to a circuit, it represents the average value of the testability of its nodes).

Undetectable fault: A fault is undetectable (or not detectable) when its presence is not able to modify the (correct) values at the outputs of the circuit model, whatever the combination applied to its inputs.