# A Windows BS test controller application

This Windows test controller application, called Tapper, is compatible with the test controller model described in a previous chapter and was developed by Luís Humberto Silvestre to support practical hands-on sessions with BS boards comprising one or two BS chains. Tapper will be described in this chapter, starting with the required test set up, followed by a short description of the functions supported and of its user interface.

## Test set up

Tapper enables the user to write and execute test programs for boards with one or two BS chains, accessing the respective TAPs through the PC parallel port. The parallel port I/O pins were mapped as presented in table 6.1.

| Test signal | D25 connector | I/O | Pin name | Centronics |
| --- | --- | --- | --- | --- |
| TDI 0 | 2 | Output | D0 | 2 |
| TMS 0 | 3 | Output | D1 | 3 |
| TCK 0 | 4 | Output | D2 | 4 |
| TRST 0 | 5 | Output | D3 | 5 |
| TDI 1 | 6 | Output | D4 | 6 |
| TMS 1 | 7 | Output | D5 | 7 |
| TCK 1 | 8 | Output | D6 | 8 |
| TRST 1 | 9 | Output | D7 | 9 |
| NC | 1 | Output | STROBE | 1 |
| NC | 14 | Output | AUTO FEED | 14 |
| SOA | 16 | Output | INIT | 31 |
| SOB | 17 | Output | SELECT IN | 36 |
| NC | 10 | Input | ACK | 10 |
| TDO 0 | 11 | Input | BUSY | 11 |
| TDO 1 | 12 | Input | PE | 12 |
| SIA | 13 | Input | SELECT | 13 |
| SIB | 15 | Input | ERROR | 32 |
| GND | 18-25 | - | GND | 19-30 |

Table 6.1: I/O to test interface pin mapping.

Notice that provisions were made to support two synchronisation channels (A and B), through sync inputs (SIA and SIB) and outputs (SOA and SOB). However, and due to the unavailability of pins in the PC parallel port to apply test stimuli and capture responses, the sync instructions using these two pairs of pins are currently not implemented.

The test set up is shown in figure 6.1, where a flat-cable connects the PC parallel port to the test connector of the demonstration board described in the previous chapter.

Figure 6.1: The test set up, showing the demonstration board and the flat-cable connection to the PC.

## The test controller application

The Tapper window comprises a test program control bar, status information area, three pop-down menus and a general purpose editing area, as shown in figure 6.2. Notice that the small test program segment shown checks the integrity of a scan chain that includes a single IC placed in TAP 1 (question: what would in this case be the operating mode selected for the BS architecture and the TAP controller state, following the execution of the test program shown?).

Figure 6.2: The Tapper user interface.

### Test program control bar

The test program control bar includes the five control buttons shown in figure 6.2. From left to right, the function of each button may be described as follows:

- : The Run button initiates program execution.

- : The Pause button temporarily suspends program execution.

- : The Stop button stops the execution of the test program.

- : The Step button executes the test program in single-step mode.

- : The Breakpoint button is used to insert a breakpoint in the current line.

Any number of breakpoints may be inserted, forcing the program to suspend execution when a line with a breakpoint is reached. Pressing either the Run or Single-step buttons will reactivate program execution.

### Status information

The status information area includes information about the TAP currently selected, the contents of the internal counters, the error flag currently selected and the state of the available error flags. A brief description of each of these fields, as they appear following the execution the small test program represented in figure 6.2, may be presented as follows:

- : Indicates the currently selected TAP.

- : The Tapper has two internal counters of 16 and 24 bits and supports the LD CNT,N instruction (any of the two counters may be used internally in this case), a LD C16,N instruction (uses the 16-bit counter) and a LD C24,N instruction (uses the 24-bit counter).

- : Indicates the currently selected error flag and the state of each of the eight error flags available for each BS chain. Since the test program shown in figure 6.2 was executed with no board connected to the PC parallel port, the selected error flag was set during program execution.

### Pop-down menus

The Tapper application includes three standard pop-down menus: File, Edit and Help. The options available in these menus are the following:

- The File menu has the standard options of New, Open, Save, Save as and Exit.

- The Edit menu includes the standard options of Undo, Cut, Copy, Paste and Delete, followed by a last Clean all breakpoints option that removes all breakpoints inserted in the test program.

- The Help menu has the standard options of Contents, Index and find, and About Tapper. Additionally, it includes a TAP controller option that shows the state transition diagram of the TAP controller FSM.

### Editing area

The editing area is used to write or modify test programs. The syntax of each line is as follows:

Label:	instruction	operand	; comment

Notice that a colon must always follow a label and that any text after a semi-colon is considered as a comment.

## Exhaustive testing of non-BS clusters

This section uses the Tapper application to address one of the alternatives available to test non-BS clusters present in partial BS boards. The selected approach consists of using the surrounding BS infrastructure for cluster testing, by shifting in each cluster test vector and analysing each response shifted out.

The cluster to be used as an example is the 74LS139 + 74LS04 group in our demonstration board, where all the BS cells surrounding this cluster belong to the same BS chain. Each test vector will be applied through the BS cells connected to the four inputs of the cluster (A, B, /1G and /2G) and the corresponding response values will be captured at the BS cells connected to its eight outputs (/1Y0, /1Y1, /1Y2, /1Y3, 2Y0, 2Y1, 2Y2 e 2Y3). Instead of trying to find out what will be the minimum test vector set for maximum fault coverage, and considering that the total number of test vectors is very small (only 16), we will simply perform an exhaustive test of this cluster.

The connections between the cluster to be tested and the surrounding BS infrastructure are shown in figure 6.3:

Figure 6.3: Cluster under test and surrounding BST infrastructure.

The application of every possible combination to the four cluster inputs will produce at the outputs (of a fault-free cluster) the test vector responses shown in table 6.2:

| Order of the BST cells in the complete BST chain: |  |  |  |
| --- | --- | --- | --- |
| 10 11 12 13 | 20 21 22 23 24 25 26 27 |  |  |
| A B /1G /2G | /1Y0 /1Y1 /1Y2 /1Y3 2Y0 2Y1 2Y2 2Y3 | Hex. |  |
| 0 0 0 0 | 0 1 1 1 1 0 0 0 | 78 |  |
| 0 0 0 1 | 0 1 1 1 0 0 0 0 | 70 |  |
| 0 0 1 0 | 1 1 1 1 1 0 0 0 | F8 |  |
| 0 0 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 0 1 0 0 | 1 1 0 1 0 0 1 0 | D2 |  |
| 0 1 0 1 | 1 1 0 1 0 0 0 0 | D0 |  |
| 0 1 1 0 | 1 1 1 1 0 0 1 0 | F2 |  |
| 0 1 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 1 0 0 0 | 1 0 1 1 0 1 0 0 | B4 |  |
| 1 0 0 1 | 1 0 1 1 0 0 0 0 | B0 |  |
| 1 0 1 0 | 1 1 1 1 0 1 0 0 | F4 |  |
| 1 0 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |
| 1 1 0 0 | 1 1 1 0 0 0 0 1 | E1 |  |
| 1 1 0 1 | 1 1 1 0 0 0 0 0 | E0 |  |
| 1 1 1 0 | 1 1 1 1 0 0 0 1 | F1 |  |
| 1 1 1 1 | 1 1 1 1 0 0 0 0 | F0 |  |

Table 6.2: Exhaustive test vector set and expected responses.

Once the information present in figure 6.3 and table 6.2 is obtained, the next step consists of determining the operand of the NSHFCP instructions that will shift in each new test vector and shift out the responses to the previous test vector. In order to implement this step we also need to know what values should be sent to those BS cells which are not directly involved in test vector application and response capturing. This task is simplified if we start by considering their relative position in the chain, as shown in figure 6.4:

Figure 6.4: Location of those cells involved in test vector application and response capturing.

In what concerns those BS cells not directly involved in the test of this cluster, we will take the following decisions:

- BS cell 0 will receive a 0, since otherwise those pins connected to cells 10 to 13 would be in high-impedance and test vector application would not take place. BS cell 1 plays the same role for those pins connected to cells 14 to 17; although these pins are not used to test this cluster, cell 1 will also receive a 0.

- The values shifted in to BS cells 2 to 9 are applied to the internal logic of the first BS component (on the left side in figure 6.3). Since the internal test of this component is not a requirement, we will simply shift a 0 to each of these cells.

- BS cells 14 to 17, which are also not directly involved in the cluster test operation, will also receive a 0.

- BS cells 18 e 19, as well as cells 28 to 35 (corresponding to the output pins of the second BS component), will for the same reasons also receive a 0.

The remarks presented so far lead us to the conclusion that the procedure to determine the operand of each NSHFCP instruction may be defined as follows (except for the first vector, which does not include the information concerning the responses to the previous vector and therefore has a simpler construction rule):

- The values to be shifted into the BS chain are obtained by inserting each 4-bit combination, corresponding to each test vector to be applied, in those bit positions indicated in figure 6.5 (e.g. this part of the operand for the second NSHFCP instruction will be $00,..,..,$00,..,..,$40,..,..,$00,..,..,$00,..,..).

Figure 6.5: Construction rule for the serial test vectors.

- The expected responses are obtained by inserting the expected output combination for the previous vector in those bit positions indicated in figure 6.6 (e.g. this part of the operand for the second NSHFCP instruction will be ..,$00,..,..,$78,..,..,$00,..,..,$00,..,..,$00,..).

Figure 6.6: Construction rule to insert the expected response information.

- The comparison masks part of each NSHFCP instruction has a construction rule identical to the one shown in figure 6.6, which consists of inserting a 1 in each bit position belonging to the expected responses group (e. g. this part of the operand for the second NSHFCP instruction will be ..,$00,..,..,$FF,..,.., $00,..,.., $00,..,..,$00,..).

The set of NSHFCP instructions for testing the non-BST cluster illustrated in figure 6.3 will therefore be represented as follows:

NSHF	$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$78,$FF,$40,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$70,$FF,$80,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F8,$FF,$C0,$00,$00,$00,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$D2,$FF,$40,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$D0,$FF,$80,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F2,$FF,$C0,$00,$00,$01,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$B4,$FF,$40,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$B0,$FF,$80,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F4,$FF,$C0,$00,$00,$02,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$00,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$E1,$FF,$40,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$E0,$FF,$80,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F1,$FF,$C0,$00,$00,$03,$00,$00,$00,$00,$00

NHSFCP	$00,$00,$00,$00,$F0,$FF,$C0,$00,$00,$03,$00,$00,$00,$00,$00

It must be recalled that the TAP controller in each BS component has to be taken to the Shift-DR state before the first shifting operation takes place:

NSHF	$00,$00,$00,$00,$00

TMS1	; go to Update-DR

TMS1	; go to Sel.-DR

TMS0	; go to Capt.-DR

TMS0	; go to Shift-DR

From this point on, the following instruction sequence will be repeated for each new vector:

NHSFCP	$00,$00,$00,$00,$78,$FF,$40,$00,$00,$00,$00,$00,$00,$00,$00,

TMS1	; go to Update-DR

TMS1	; go to Sel.-DR

TMS0	; go to Capt.-DR

TMS0	; go to Shift-DR

.. etc.

As a concluding exercise, the reader is invited to try to find out if some of the faults associated with this cluster may escape detection in the case of the (exhaustive) test vector set presented and if so what measures could be taken to improve fault detection.

## Non-BS cluster testing with PRPG and SA

Following the approach selected in the previous section, we will now consider another alternative for testing the same non-BS cluster. The previous approach consisted of performing an exhaustive test of this cluster, for which 16 test vectors were required (since this cluster has only 4 inputs). By analysing the respective test program, we conclude that a total of 680 TCK cycles were required solely for test vector / responses shift in and out (additional TCK cycles are required for the necessary transitions across the TAP controller state diagram).

The approach to be considered in the second part of this text consists of using the facilities available in the Texas Instruments SCOPE octals (which surround the cluster to be tested) for pseudo-random pattern generation (PRPG) and response compression by signature analyses (SA). This alternative has the additional advantage of highlighting some aspects which are frequently found in the area of BIST (Built-In Self-Test), where these methods for test vector generation and response compression are widely used.

For the case now being considered, we will set the left octal to work in PRPG mode and the right octal in SA mode. The information present in the Texas Instruments data sheet for these components shows that the sequence of actions to carry out is as follows:

- Shift into the instruction register (IR) the code of BOUNDARY READ (e.g. X0001010).

- Shift in the initial seed value for the BS register.

- Shift into the IR the code of TEST DATA REGISTER SCAN (e.g. X0001110).

- Shift into the Test Data Register the code corresponding to the required operating mode (01 for 16-bit mode PRPG; 10 for 16-bit mode SA).

- Shift into the IR the code of BOUNDARY RUN TEST (X0001001).

- Bring the TAP controller to the Run Test / Idle state, where the necessary number of TCK clock cycles will be applied.

The application of TCK cycles while in the Run Test / Idle state will then initiate the test operation. Recall that the test vector responses do not have to be shifted out individually in this case, since the SA operation is in progress at the inputs of the second SCOPE octal. When the necessary number of TCK cycles has taken place, the contents of the BS register of the second octal are shifted out and compared with the expected signature.

The application of this method requires that an answer is previously found for the two following questions:

- How should we proceed to determine the expected (correct) signature? (recall that error detection through the resulting signature will be done using the NSHFCP instruction, which will include the correct signature information as part of its operand).

- How many test vectors will have to be applied to conclude the test operation? (we will use the 16-bit mode PRPG, but our 4-input cluster does not require  neither would it be acceptable, for reasons of test speed  that all possible combinations in the octal outputs are applied).

These questions will now be considered in detail in the following sections.

### Determining the correct signature

The expected contents in the BS register of the second octal, after the test operation is concluded, may be determined by one of the following methods:

- By logic simulation of the three blocks: PRPG, non-BS cluster and SA.

- By reading the contents of the BS register of the second octal, after a complete test run takes place with a circuit which is known to be fault-free.

The first method is clearly the most time consuming and requires more sophisticated resources, since we will have to build simulation models for the three blocks and proceed to simulate their operation during a complete test run. We might for example use Altera’s tools (the MAX+PLUS II BASELINE software is free) and use the schematic capture and logic simulation modules for this purpose. The schematic diagrams of the three blocks to be simulated are shown in figures 6.7 to 6.9.

The complete circuit to be simulated would then be defined in a top-level schematic description including these three blocks, as shown in figure 6.10. Logic simulation could now be carried out and the required result read in the SA block outputs after the last TCK cycle has been applied. The expected signature determined using this procedure will naturally be the same that is obtained by setting up the same operating conditions in practice, provided that we are careful enough to avoid errors during the schematic capture phase.

Figure 6.7: BS register in 16-bit PRPG mode (74BCT8244).

Figure 6.8: Non-BS cluster.

The second method is however much faster and we recommend that the reader considers it as the preferred alternative (the test program should be run in a fault free circuit and suspended before the last NSHFCP instruction is executed; shifting out the contents of the boundary scan register of the second octal will yield the required signature).

Figure 6.9: BS register in 16-bit SA mode (74BCT8244).

Figure 6.10: Top-level representation of the circuit to be simulated.

### The required number of PRPG test vectors

The last question to consider is how to determine when is it possible to conclude the application of TCK cycles (each TCK cycle produces a new PRPG test vector), by considering that all the possible combinations in the 4 cluster inputs have already been generated (although we are still applying an exhaustive test vector set, we are not using a deterministic test procedure). The answer to this question may be obtained by one of the two following alternatives:

- Once again through logic simulation (for example with the Altera tools), this time involving only the PRPG block in figure 6.7, we are able to check the outputs of this block and determine what is the number of TCK cycles required to obtain the 16 possible combinations in the 4 cluster inputs.

- Simulation of the PRPG operation in the SCOPE octal outputs could also be done using the Scan Educator application which is available from Texas Instruments web server (www.ti.com).

Figure 6.11: The Scan Educator in the required mode (control of a single SCOPE octal).

This second method is in fact much faster and may be started by selecting the option 4 in the main menu (“scan control mode testing  single device”) followed by option 3 (“on your own”). This sequence will take us to the situation represented in figure 6.11, where the six steps described in the first page can be carried out (to shift in instructions and initialise the data registers). The application of TCK cycles will then start until the necessary 16 combinations in the 4 cluster inputs have already taken place (the required number of TCK cycles depends on the seed value shifted into the boundary scan register).

We conclude this second part on testing partial BS boards by inviting the reader to write the test program corresponding to this second strategy and to verify its operation in practice. The required number of TCK cycles for this second approach should be compared with the 680 cycles necessary for shifting in / out test data in the previous approach.

## Hands-on

Two exercises are proposed to consolidate the lecture notes referring to BST at board-level, both of them based on the demonstration board that was presented in chapter 5 (the corresponding schematic diagram is included as an annex to chapter 5, as well as the data sheets of the components present): the first exercise refers to fault detection using deterministic test vectors and the second exercise refers to fault detection using pseudo-random test generation methods.

### Fault detection through deterministic test vectors

Consider the following set of faults (by increasing complexity): i) short-circuit X16; ii) short-circuit X17; open circuit X4; iv) short-circuit X9; v) short-circuit X2; vi) short circuit X3. For each of these faults:

- Determine the required test vectors for fault detection, including expected responses and mask data

- Serialise all test vectors and provide the bit sequence to be shifted into the scan chain

- Write the corresponding test program, using for this purpose the set of instructions supported by the Tapper application

- Validate the test program by practical experimentation, setting up each fault and verifying that it is detected as expected (use the demonstration board and the Tapper Windows application).

### Fault detection through pseudo-random test vectors

The exercise proposed in this section consists of generating the test program to detect short circuit X2, which is an internal fault to the non-BS cluster on the left. The following recommendations will simplify the work to be done:

- Determine the number of clock pulses in TCK required to guarantee that all input combinations are applied to the non-BS cluster, assuming that its surrounding BS cells operate in PRPG (pseudo-random pattern generation) mode.

- Determine the expected response for the fault-free circuit, present in the BS cells connected to the cluster outputs, assuming that these cells operate in signature analysis mode.

- According to the information obtained in the two previous steps, produce the test program that applies all input combinations to the cluster and checks the resulting signature (use for this purpose the set of instructions supported by the Tapper application).

- Validate the test program by practical experimentation, forcing the presence of short circuit X2 and verifying that it is detected as expected (use the demonstration board and the Tapper Windows application).

6.	A Windows BS test controller application 	1

6.1	Test set up	1

6.2	The test controller application	2

6.2.1	Test program control bar	3

6.2.2	Status information	4

6.2.3	Pop-down menus	4

6.2.4	Editing area	4

6.3	Exhaustive testing of non-BS clusters	5

6.4	Non-BS cluster testing with PRPG and SA	9

6.4.1	Determining the correct signature	10

6.4.2	The required number of PRPG test vectors	12

6.5	Hands-on	14

6.5.1	Fault detection through deterministic test vectors	14

6.5.2	Fault detection through pseudo-random test vectors	14

A set up diskette is included at the end of this document, containing an executable file called Setup.exe. To install the Tapper application, run this executable file.