# A case study of test program generation

The objective of this chapter is to show the practical application of all the concepts described so far, using as a demonstration vehicle a very simple board with BS. The complexity of this demonstration board is low for pedagogical reasons, but it includes the same test cases that are to be found in most real life test situations: multiple (2) BS chains, non-BS clusters (2) and primary I/O pins. The board will be described in first place, followed by the analysis of the information required for test vector generation and, in the last section, by the test program generated for all steps in the test protocol, assuming the BS test controller model presented in the previous chapter.

## The demonstration board

The demonstration board used in this chapter was previously developed as part of an ESPRIT project (EP2478, Research into Boundary Scan Test Implementation), at the time with the objective of validating a set of test vector generation methodologies. The main characteristics of this board are the following:

- There are two BS chains: The first chain (BS chain 0) includes the BS components IC3 and IC4 (both of them the SN74BCT8244) and the second chain (BS chain 1) includes the BS component IC5 (again the SN74BCT8244).

- There are two non-BS clusters: The first cluster comprises the non-BS components IC1 and IC2 (respectively the 74LS139 dual 2:4 decoder and the 74LS04 hex inverter) and is completely surrounded by BS chain 0. The second is a single-IC cluster that comprises the non-BS component IC6 (74LS157 quad 2:1 mux) and is surrounded by BS cells of both chains.

- A total of 17 jumpers enable the insertion of open or short-circuit faults for test program validation purposes.

- There are 8 primary inputs (I[0..7]) and 8 primary outputs (O[0..7]).

The schematic diagram of this board is included as an annex at the end of this chapter. However, and to make it easier to understand this case study from the testing point of view, an alternative representation emphasising the two BS chains and the two non-BS clusters is shown in figure 5.1.

Figure 5.1: BS chains and non-BS clusters in the demonstration board.

The SN74BCT8244 data sheet (also available from TI’s web site at www.ti.com) includes all the information concerning the BS architecture present in this component, which in fact includes several optional instructions specific of TI. However, and since the basic steps of the test protocol presented in an earlier chapter did not make use of optional instructions (which were only referred as a vehicle for optional steps in this protocol), the complete test program will only be based on the three mandatory instructions described in the IEEE 1149.1 std.

## The information required for test program generation

The three main types of information required for test program generation are the description of the BS architecture present in each BS component, the structural information of the PCB (netlist) and the description of test vectors and responses to be used with non-BS clusters present in the board under test. These three types of input information will be analysed as we proceed through the main steps of the test protocol, in the following sections.

### Integrity check of the BS infrastructure

The test program for checking the integrity of the BS chains requires a description of the BS architecture present in these devices, obtainable from the SN74BCT8244 data sheet, and information concerning the location of the BS components in the board-level BS chain. These two types of information are merged in table 5.1 below.

| Parameter | Component 0 | Component 1 | Component 2 |
| --- | --- | --- | --- |
| Component belongs to BS chain | 0 | 0 | 1 |
| Location of the component in the chain | 0 | 1 | 0 |
| Length of the Instruction Register (IR) | 8 | 8 | 8 |
| Contents of the IR following capture | XXXXXX01 | XXXXXX01 | XXXXXX01 |
| Is an Identification Register present? | No | No | No |
| Sample / Preload code | 00000010 | 00000010 | 00000010 |

Table 5.1: Information required for checking the integrity of the BS chains.

Notice that only that information which is not pre-defined is required, such as the code of the S/P mandatory instruction, or the length of the instruction register and its contents following a Capture-IR operation (the two bits closest to TDO are pre-defined, but the remaining bits might be used to capture relevant information).

### Full-BS interconnects test

This demonstration board includes 24 full-BS interconnects †, two of which (nets 10 and 11 in figure 5.1) share their BS cells between the two BS chains. The information required for test program generation is once more obtainable from the SN74BCT8244 data sheet and from the board structural information (netlist), and comprises the following fields for each interconnect:

- Number and identification of the BS chains that have cells present in this interconnect

- Concerning the output pins present:

- Number of output pins and (for each one) location, in the BS chain, of the output cell, the control cell (if any) and the logic value that drives the pin to the high-impedance state.

- Concerning the input pins present:

- Number of input pins and (for each one) location, in the BS chain, of the input cell.

- Concerning the bidirectional pins present:

- Number of bidirectional pins and (for each one) location, in the BS chain, of the output cell, the input cell, the control cell and the logic value that drives the pin to the high-impedance state.

- Concerning the primary input pins present (it is assumed  that each primary input pin is driven by an external test channel that enables the primary input to be set to the high-impedance state):

- Number of primary input pins and (for each one) an identification number and the control value that drives the respective test channel to the high-impedance state.

- Concerning the primary output pins present:

- Number of primary output pins and (for each one) an identification number.

- Finally, and considering the possible case of interconnects that tie unused input pins to ground or VCC:

- Is the interconnect at a fixed logic level and if so at which logic value?

The information required for generating the test program segment concerning the full-BS interconnects, in our demonstration board, is presented in table 5.2.

| Net ident. | # BS chains | BS chain ident. | # of output pins | Output cell | Control cell | High imped. | # of input pins | Input cell | # of bidir. pins | # of prim. inputs | Prim. input | High imped. | # of prim. outputs | Prim. output | Fixed level? | Logic level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Net 0 | 1 | 0 | 0 | - | - | - | 1 | 2 | 0 | 1 | 0 | 0 | 0 | - | No | - |
| Net 1 | 1 | 0 | 0 | - | - | . | 1 | 3 | 0 | 1 | 1 | 0 | 0 | - | No | - |
| Net 2 | 1 | 0 | 0 | - | - | . | 1 | 4 | 0 | 1 | 2 | 0 | 0 | - | No | - |
| Net 3 | 1 | 0 | 0 | - | - | . | 1 | 5 | 0 | 1 | 3 | 0 | 0 | - | No | - |
| Net 4 | 1 | 0 | 0 | - | - | . | 1 | 6 | 0 | 1 | 4 | 0 | 0 | - | No | - |
| Net 5 | 1 | 0 | 0 | - | - | . | 1 | 7 | 0 | 1 | 5 | 0 | 0 | - | No | - |
| Net 6 | 1 | 0 | 0 | - | - | . | 1 | 8 | 0 | 1 | 6 | 0 | 0 | - | No | - |
| Net 7 | 1 | 0 | 0 | - | - | . | 1 | 9 | 0 | 1 | 7 | 0 | 0 | - | No | - |
| Net 8 | 1 | 0 | 0 | - | - | . | 1 | 0 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 9 | 1 | 0 | 0 | - | - | . | 1 | 1 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 10 | 2 | 0 | 1 | 16 | 1 | 1 | 0 | - | 0 | 0 | - | - | 0 | - | No | - |
|  |  | 1 | 0 | - | - | - | 1 | 8 | 0 |  |  |  |  |  |  |  |
| Net 11 | 2 | 0 | 1 | 17 | 1 | 1 | 0 | - | 0 | 0 | - | - | 0 | - | No | - |
|  |  | 1 | 0 | - | - | - | 1 | 9 | 0 |  |  |  |  |  |  |  |
| Net 12 | 1 | 0 | 0 | - | - | - | 1 | 18 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 13 | 1 | 0 | 0 | - | - | - | 1 | 19 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 14 | 1 | 1 | 1 | - | - | - | 1 | 0 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 15 | 1 | 1 | 1 | - | - | - | 1 | 1 | 0 | 0 | - | - | 0 | - | Yes | 0 |
| Net 16 | 1 | 1 | 1 | 10 | 0 | 1 | 0 | - | 0 | 0 | - | - | 1 | 0 | No | - |
| Net 17 | 1 | 1 | 1 | 11 | 0 | 1 | 0 | - | 0 | 0 | - | - | 1 | 1 | No | - |
| Net 18 | 1 | 1 | 1 | 12 | 0 | 1 | 0 | - | 0 | 0 | - | - | 1 | 2 | No | - |
| Net 19 | 1 | 1 | 1 | 13 | 0 | 1 | 0 | - | 0 | 0 | - | - | 1 | 3 | No | - |
| Net 20 | 1 | 1 | 1 | 14 | 1 | 1 | 0 | - | 0 | 0 | - | - | 1 | 4 | No | - |
| Net 21 | 1 | 1 | 1 | 15 | 1 | 1 | 0 | - | 0 | 0 | - | - | 1 | 5 | No | - |
| Net 22 | 1 | 1 | 1 | 16 | 1 | 1 | 0 | - | 0 | 0 | - | - | 1 | 6 | No | - |
| Net 23 | 1 | 1 | 1 | 17 | 1 | 1 | 0 | - | 0 | 0 | - | - | 1 | 7 | No | - |

Table 5.2: Information required to generate the test program for full-BS interconnects.

### Non-BS clusters test

In order to simplify the test set up and to emphasise the capabilities of the BS infrastructure, it is assumed that the two clusters are to be tested through the surrounding BS chains, with no direct connections to external test channels.

The first cluster comprises IC1 and IC2 (74LS139 and 74LS04) and is surrounded by BS cells of chain 0 only, as shown in figure 5.2.

Figure 5.2: The 74LS139 and 74LS04 non-BS cluster.

The information required for generating the test program segment concerning this cluster, besides the test vectors generated externally, comprises the elements presented in table 5.3.

| Parameter | Value |
| --- | --- |
| Number of BS chains surrounding the cluster | 1 (chain 0) |
| Information concerning BS chain 0 |  |
| Number of BS cells | 13 |
| Identification of the BS cells | 0,10,11,12,13,20,21,22,23,24,25,26,27 |
| # of cluster inputs that are also primary inputs | 0 |
| # of cluster outputs that are also primary outputs | 0 |
| Guard values required? | No |

Table 5.3: Description of the first non-BS cluster (except the test vectors).

Notice that there are no primary input or output pins that are also inputs or outputs of the cluster, so all test vectors are to be applied through the BS cells surrounding the cluster. In some cases it might be required to apply guard values to the cluster inputs, during the remaining steps of the test protocol, but this is not the case in our example.

The GenRad HILO test vector generator produced 5 test vectors for this cluster, providing 100% fault coverage of stuck-at pins in both components (IC1 and IC2). These 5 test vectors, their expected responses and masks, are shown in table 5.4, already inserted into the proper places in the stream of 13 cells that surround the cluster (cell 0 at left in each bit stream, then cells 10 to 13, then cells 20 to 27).

| # vectors | BS chain | To shift in | Expected values | Mask data |
| --- | --- | --- | --- | --- |
| 5 | 0 | 0111011111111 0110111111111 0000011111111 0010011111111 0100011111111 | 1111111110001 1111111100000 1111101111000 1111111010010 1111110110100 | 0000011111111 0000011111111 0000011111111 0000011111111 0000011111111 |

Table 5.4: First cluster test vectors, expected responses and mask data.

The leftmost bit in the “shift in” column is the value to be loaded into BS cell 0, which enables the BS pins driving the cluster inputs (and is therefore always 0, otherwise these driving pins would be tristated). The following 4 bits in this same column represent in fact the test vectors to be applied (which will be loaded into BS cells 10 to 13), with the remaining bits in this column representing the values that will be loaded into cells 20 to 27 (any value could be shifted into these cells). In the “expected values” column, only the rightmost 8 bits are meaningful (those that correspond to cells 20 to 27), containing the expected responses to each test vector. Accordingly, only the 8 rightmost bits in the “mask data” column are 1.

The second cluster comprises only a single IC (the 74LS157) and is surrounded by BS cells of both chains, as shown in figure 5.3.

Figure 5.3: The 74LS157 single-IC non-BS cluster.

The information required for generating the test program segment concerning this cluster, besides the test vectors generated externally, comprises the elements presented in table 5.5.

| Parameter | Value |  |
| --- | --- | --- |
| Number of BS chains surrounding the cluster | 2 (chains 0 and 1) |  |
| Information concerning BS chain 0 |  |  |
| Number of BS cells |  | 13 |
| Identification of the BS cells |  | 1,14,15,18,19,28,29,30,31,32,33,34,35 |
| Information concerning BS chain 1 |  |  |
| Number of BS cells |  | 4 |
| Identification of the BS cells |  | 2,3,4,5 |
| # of cluster inputs that are also primary inputs | 0 |  |
| # of cluster outputs that are also primary outputs | 0 |  |
| Guard values required? | No |  |

Table 5.5: Description of the second non-BS cluster (except the test vectors).

Again there are no primary input or output pins that are also inputs or outputs of the cluster, and it is not required to apply guard values to the cluster inputs during the remaining steps of the test protocol.

The GenRad HILO test vector generator produced for this second cluster again 5 test vectors, providing 100% fault coverage of stuck-at pins in the 74LS157. In this case, however, the test vectors will be applied through the BS cells of chain 0 and the test responses will be captured through the BS cells of chain 1. These 5 test vectors, their expected responses and masks, for both chains, are shown in table 5.6, already inserted into the proper places in the stream of cells that surround the cluster (the same rules apply as in the previous case, now with cells 1 and 2 in the leftmost locations for the two BS chains).

| # vectors | BS chain | To shift in | Expected values | Mask data |
| --- | --- | --- | --- | --- |
| 5 | 0 | 0000011111010 0000000010001 0110000110111 0100011010101 0100000100010 | 1111111111111 1111111111111 1111111111111 1111111111111 1111111111111 | 0000000000000 0000000000000 0000000000000 0000000000000 0000000000000 |
|  | 1 | 1111 1111 1111 1111 1111 | 1111 0000 0000 1111 0000 | 1111 1111 1111 1111 1111 |

Table 5.6: Second cluster test vectors, expected responses and mask data.

Notice now that all values in the “shift in” column for chain 0 carry meaningful data, since all these 13 cells play a role in applying each test vector to the cluster inputs. However, for BS chain 0, there are no expected values known and therefore all mask data bit streams contain only 0s. As for BS chain 1, the shift in data is meaningless (since these cells are only used for response capturing), but all bits have known expected values for each vector.

### Components test

The information required to generate a test program segment for component test is summarised in table 5.7.

| Parameter | Component 0 | Component 1 | Component 2 |
| --- | --- | --- | --- |
| Component belongs to BS chain | 0 | 0 | 1 |
| Location of the component in the chain | 0 | 1 | 0 |
| Length of the BS Register | 18 | 18 | 18 |
| The component supports BIST? | No | No | No |
| Number of test vectors for internal test | 2 | 2 | 2 |
| INTEST code | 00000000 | 00000000 | 00000000 |

Table 5.7: Information required for testing the BS components (except the test vectors).

Due to the simplicity of the internal logic, there are no BIST functions available. However, and precisely because the internal logic consists only of identity functions, it is easy to perform a simple “dead or alive” test. Two test vectors are used for this purpose, toggling each input and applying complementary logic values in adjacent pins. Notice that this simple test does not guarantee that there are no faults present (a short-circuit between input pins 0 and 2, for example, would in this case escape detection), but additional test vectors might be used in situations with higher reliability requirements. The two test vectors, their expected responses and mask data are shown in table 5.8.

| Component | To shift in | Expected values | Mask data |
| --- | --- | --- | --- |
| Each component | 110101010111111111 111010101000000000 | 111111111101010101 000000000010101010 | 000000000011111111 000000000011111111 |

Table 5.8: Test vectors for components test.

Recall that the sequence of BS cells in each IC, starting from TDI, are those associated to the 2 tristate control pins, the 8 input pins and the 8 output pins. The 8 bit locations following the 2 leftmost bits in the “shift in” column therefore contain the test vector that is to be applied to the internal logic (01010101 and 10101010 respectively). The responses are captured in the 8 rightmost bits, according to the expected responses and mask data presented in table 5.8.

## The test vectors

This section presents the set of test vectors that were generated to detect all possible short-circuit faults among full-BS interconnects, followed by the complete set of serialised test vectors, for all test protocol steps, including their expected responses and mask data.

### Test vectors for short-circuit detection in full-BS interconnects

The set of test vectors presented in table 5.9 was produced by a modified “self-diagnosis” algorithm † and guarantees complete short-circuit fault detection capability. This algorithm also guarantees that the sequence of logic values captured in any short-circuited interconnect (the so-called STR, sequential test response) is different from the sequence of logic values applied to any interconnect (the so-called Sequential Test Vector, STV). For diagnostic purposes, this is equivalent to say that aliasing syndromes cannot occur, since any faulty SRV never belongs to the set of STVs. The other type of syndrome challenging fault diagnosis occurs when disjoint sets of short-circuited interconnects produce the same SRV. This other type is known as confounding syndrome and its avoidance requires a larger number of test vectors.

ptv[0]:	010101010001000001010101

ptv[1]:	010101010010000010101010

ptv[2]:	011010100001000001011010

ptv[3]:	100110100001000010100101

ptv[4]:	101001100010000001100110

ptv[5]:	101010010010000010011001

Table 5.9: Set of test vectors generated for short-circuit fault detection among full-BS interconnects.

The 6 horizontal test vectors shown in table 5.9 represent the logic values applied simultaneously to the 24 interconnects of this type (each bit stream has 24 bits) and are therefore normally referred to as Parallel Test Vectors (PTVs). The STV applied to each interconnect may be read in each column, starting with the STV applied to net 0 in the leftmost column.

### The complete set of serialised test vectors, expected responses and mask data

The complete set of test vectors, addressing all steps in the test protocol, is presented in this section. Each test vector is presented in serial form, ready to be shifted into the scan chains, and accompanied by its expected responses and mask information.

TOTAL NUMBER OF INFRASTRUCTURE TEST PATTERNS: 4

***************************************************************************

Infrastructure test pattern number 0 (ID or BP register opcodes for IR's):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][0]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 1

Expected result (to be shifted out while this test pattern is shifted in):

ER[0][0]:

0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 0 1 1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[0][0]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1

Boundary scan chain number 1:

Test pattern:

TP[1][0]:

1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 1

Expected result (to be shifted out while this test pattern is shifted in):

ER[1][0]:

0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[1][0]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 1

***************************************************************************

Infrastructure test pattern number 1 (for the selected DRs - ID or BP):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][1]:

1 1

Expected result (to be shifted out while this test pattern is shifted in):

ER[0][1]:

0 0

Mask bits (for this expected result):

MB[0][1]:

1 1

Boundary scan chain number 1:

Test pattern:

TP[1][1]:

1

Expected result (to be shifted out while this test pattern is shifted in):

ER[1][1]:

0

Mask bits (for this expected result):

MB[1][1]:

1

***************************************************************************

Infrastructure test pattern number 2 (Sample/Preload opcode for the IR's):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][2]:

0 0 0 0 0 0 1 0 0 0 0 0 0 0 1 0

Expected result (to be shifted out while this test pattern is shifted in):

ER[0][2]:

1 1 1 1 1 1 0 1 1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[0][2]:

0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1

Boundary scan chain number 1:

Test pattern:

TP[1][2]:

0 0 0 0 0 0 1 0

Expected result (to be shifted out while this test pattern is shifted in):

ER[1][2]:

1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[1][2]:

0 0 0 0 0 0 1 1

***************************************************************************

Infrastructure test pattern number 3 (Extest opcode for the IR's):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][3]:

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result (to be shifted out while this test pattern is shifted in):

ER[0][3]:

1 1 1 1 1 1 0 1 1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[0][3]:

0 0 0 0 0 0 1 1 0 0 0 0 0 0 1 1

Boundary scan chain number 1:

Test pattern:

TP[1][3]:

0 0 0 0 0 0 0 0

Expected result (to be shifted out while this test pattern is shifted in):

ER[1][3]:

1 1 1 1 1 1 0 1

Mask bits (for this expected result):

MB[1][3]:

0 0 0 0 0 0 1 1

TOTAL NUMBER OF INTERCONNECT TEST PATTERNS: 8

***************************************************************************

Interconnect test pattern number 0 (for open fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][0]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][0]:

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][0]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][0]:

0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[1][0]:

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[1][0]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[0]:

0 0 0 0 0 0 0 0

CS[0]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[0]:

0 0 0 0 0 0 0 0

POM[0]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 1 (for open fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][1]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][1]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][1]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][1]:

0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][1]:

0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][1]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[1]:

1 1 1 1 1 1 1 1

CS[1]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[1]:

1 1 1 1 1 1 1 1

POM[1]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 2 (for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][2]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][2]:

0 0 0 1 0 1 0 1 0 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][2]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][2]:

0 0 0 0 0 0 0 0 1 1 0 1 0 1 0 1 0 1

Expected result - shifted out while the next TP is shifted in:

ER[1][2]:

0 0 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0 1

Mask bits (for this expected result):

MB[1][2]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[2]:

0 1 0 1 0 1 0 1

CS[2]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[2]:

0 1 0 1 0 1 0 1

POM[2]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 3 (for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][3]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][3]:

0 0 0 1 0 1 0 1 0 1 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][3]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][3]:

0 0 0 0 0 0 0 0 1 1 1 0 1 0 1 0 1 0

Expected result - shifted out while the next TP is shifted in:

ER[1][3]:

0 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0 1 0

Mask bits (for this expected result):

MB[1][3]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[3]:

0 1 0 1 0 1 0 1

CS[3]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[3]:

1 0 1 0 1 0 1 0

POM[3]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 4 (for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][4]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][4]:

0 0 0 1 1 0 1 0 1 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][4]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][4]:

0 0 0 0 0 0 0 0 1 1 0 1 0 1 1 0 1 0

Expected result - shifted out while the next TP is shifted in:

ER[1][4]:

0 0 0 0 0 0 0 0 0 1 0 1 0 1 1 0 1 0

Mask bits (for this expected result):

MB[1][4]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[4]:

0 1 1 0 1 0 1 0

CS[4]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[4]:

0 1 0 1 1 0 1 0

POM[4]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 5 (for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][5]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][5]:

0 0 1 0 0 1 1 0 1 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][5]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][5]:

0 0 0 0 0 0 0 0 1 1 1 0 1 0 0 1 0 1

Expected result - shifted out while the next TP is shifted in:

ER[1][5]:

0 0 0 0 0 0 0 0 0 1 1 0 1 0 0 1 0 1

Mask bits (for this expected result):

MB[1][5]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[5]:

1 0 0 1 1 0 1 0

CS[5]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[5]:

1 0 1 0 0 1 0 1

POM[5]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 6 (for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][6]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][6]:

0 0 1 0 1 0 0 1 1 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][6]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][6]:

0 0 0 0 0 0 0 0 1 1 0 1 1 0 0 1 1 0

Expected result - shifted out while the next TP is shifted in:

ER[1][6]:

0 0 0 0 0 0 0 0 1 0 0 1 1 0 0 1 1 0

Mask bits (for this expected result):

MB[1][6]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[6]:

1 0 1 0 0 1 1 0

CS[6]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[6]:

0 1 1 0 0 1 1 0

POM[6]:

1 1 1 1 1 1 1 1

***************************************************************************

Interconnect test pattern number 7 (last TP for short fault detection):

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][7]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Expected result - shifted out while a following "dummy" TP is shifted in:

ER[0][7]:

0 0 1 0 1 0 1 0 0 1 0 0 0 0 0 0 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Mask bits (for this expected result):

MB[0][7]:

1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][7]:

0 0 0 0 0 0 0 0 1 1 1 0 0 1 1 0 0 1

Expected result - shifted out while a following "dummy" TP is shifted in:

ER[1][7]:

0 0 0 0 0 0 0 0 1 0 1 0 0 1 1 0 0 1

Mask bits (for this expected result):

MB[1][7]:

1 1 0 0 0 0 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[7]:

1 0 1 0 1 0 0 1

CS[7]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[7]:

1 0 0 1 1 0 0 1

POM[7]:

1 1 1 1 1 1 1 1

TOTAL NUMBER OF CLUSTER TEST PATTERNS: 5

***************************************************************************

Cluster test pattern number 0

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][8]:

0 0 0 0 0 0 0 0 0 0 1 1 1 0 0 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 0

Expected result - shifted out while the next TP is shifted in:

ER[0][8]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[0][8]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][8]:

0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][8]:

0 0 1 1 1 1 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][8]:

1 1 1 1 1 1 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[8]:

1 1 1 1 1 1 1 1

CS[8]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[8]:

1 1 1 1 1 1 1 1

POM[8]:

1 1 1 1 1 1 1 1

***************************************************************************

Cluster test pattern number 1

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][9]:

0 0 0 0 0 0 0 0 0 0 1 1 0 1 0 0 1 1 0 0 1 1 1 1 1 1 1 1 0 0 0 1 0 0 0 1

Expected result - shifted out while the next TP is shifted in:

ER[0][9]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 0 0 0 0 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[0][9]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][9]:

0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][9]:

0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][9]:

1 1 1 1 1 1 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[9]:

1 1 1 1 1 1 1 1

CS[9]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[9]:

1 1 1 1 1 1 1 1

POM[9]:

1 1 1 1 1 1 1 1

***************************************************************************

Cluster test pattern number 2

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][10]:

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 1 1 0 0 1 1 1 1 1 1 1 1 0 0 1 1 0 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[0][10]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 1 1 1 0 0 0 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[0][10]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][10]:

0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][10]:

0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][10]:

1 1 1 1 1 1 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[10]:

1 1 1 1 1 1 1 1

CS[10]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[10]:

1 1 1 1 1 1 1 1

POM[10]:

1 1 1 1 1 1 1 1

***************************************************************************

Cluster test pattern number 3

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][11]:

0 0 0 0 0 0 0 0 0 0 0 1 0 0 1 0 1 1 0 0 1 1 1 1 1 1 1 1 1 1 0 1 0 1 0 1

Expected result - shifted out while the next TP is shifted in:

ER[0][11]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 0 0 1 0 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[0][11]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][11]:

0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][11]:

0 0 1 1 1 1 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][11]:

1 1 1 1 1 1 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[11]:

1 1 1 1 1 1 1 1

CS[11]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[11]:

1 1 1 1 1 1 1 1

POM[11]:

1 1 1 1 1 1 1 1

***************************************************************************

Cluster test pattern number 4

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][12]:

0 0 0 0 0 0 0 0 0 0 1 0 0 0 1 0 1 1 0 0 1 1 1 1 1 1 1 1 0 0 1 0 0 0 1 0

Expected result - shifted out while a following "dummy" TP is shifted in:

ER[0][12]:

1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 0 1 1 0 1 0 0 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[0][12]:

0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1][12]:

0 0 1 1 1 1 0 0 0 0 1 1 1 1 1 1 1 1

Expected result - shifted out while a following "dummy" TP is shifted in:

ER[1][12]:

0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 1 1

Mask bits (for this expected result):

MB[1][12]:

1 1 1 1 1 1 0 0 1 1 0 0 0 0 0 0 0 0

Primary inputs and corresponding control signals:

PI[12]:

1 1 1 1 1 1 1 1

CS[12]:

1 1 1 1 1 1 1 1

Expected values and masks at primary outputs:

PO[12]:

1 1 1 1 1 1 1 1

POM[12]:

1 1 1 1 1 1 1 1

TOTAL NUMBER OF COMPONENT BIST TEST PATTERNS: 0

TOTAL NUMBER OF TEST PATTERNS FOR INTERNAL TEST OF COMPONENTS: 3

***************************************************************************

Internal test of components: Test pattern for shifting the intest/bypass opcodes:

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0]:

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0

Boundary scan chain number 1:

Test pattern:

TP[1]:

0 0 0 0 0 0 0 0

***************************************************************************

Internal test of components: Test pattern number 0.

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][0]:

1 1 0 1 0 1 0 1 0 1 1 1 1 1 1 1 1 1 1 1 0 1 0 1 0 1 0 1 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[0][0]:

1 1 1 1 1 1 1 1 1 1 0 1 0 1 0 1 0 1 1 1 1 1 1 1 1 1 1 1 0 1 0 1 0 1 0 1

Mask bits (for this expected result):

MB[0][0]:

0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1

Boundary scan chain number 1:

Test pattern:

TP[1][0]:

1 1 0 1 0 1 0 1 0 1 1 1 1 1 1 1 1 1

Expected result - shifted out while the next TP is shifted in:

ER[1][0]:

1 1 1 1 1 1 1 1 1 1 0 1 0 1 0 1 0 1

Mask bits (for this expected result):

MB[1][0]:

0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1

***************************************************************************

Internal test of components: Test pattern number 1.

***************************************************************************

Boundary scan chain number 0:

Test pattern:

TP[0][1]:

1 1 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0 1 1 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[0][1]:

0 0 0 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0

Mask bits (for this expected result):

MB[0][1]:

0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1 0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1

Boundary scan chain number 1:

Test pattern:

TP[1][1]:

1 1 1 0 1 0 1 0 1 0 0 0 0 0 0 0 0 0

Expected result - shifted out while the next TP is shifted in:

ER[1][1]:

0 0 0 0 0 0 0 0 0 0 1 0 1 0 1 0 1 0

Mask bits (for this expected result):

MB[1][1]:

0 0 0 0 0 0 0 0 0 0 1 1 1 1 1 1 1 1

## The test program

The test program that applies the set of test vectors presented in the previous section, written in assembly, using the instruction set of our test controller model, is finally presented in this section. This test program was automatically generated and was converted to object code using the table-driven TASM cross-assembler.

0001	0000

0002	0000	;***********************************************************

0003	0000	;	Assembly source code for the demo circuit.

0004	0000	;***********************************************************

0005	0000

0006	0000	start	.org	0h

0007	0000

0008	0000

0009	0000	; Sequence to reset all BST logic.

0010	0000

0011	0000 1A		seltap0	; (switch to) TAP 0

0012	0001 11		trst		; TRST0 output is pulsed low

0013	0002 01		tms1

0014	0003 01		tms1

0015	0004 01		tms1

0016	0005 01		tms1

0017	0006 01		tms1		; soft reset of BST chain

0018	0007 1B		seltap1	; switch to TAP 1

0019	0008 11		trst		; TRST1 output is pulsed low

0020	0009 01		tms1

0021	000A 01		tms1

0022	000B 01		tms1

0023	000C 01		tms1

0024	000D 01		tms1		; soft reset of BST chain 1

0025	000E 1A		seltap0	; switch to TAP 0

0026	000F

0027	000F	; Sequence to test the BST infrastructure

0028	000F

0029	000F 00		tms0		; BST chain 0, >> Run Test / Idle

0030	0010 01		tms1		; BST chain 0, >> Select DR scan

0031	0011 01		tms1		; BST chain 0, >> Select IR scan

0032	0012 00		tms0		; BST chain 0, >> Capture IR

0033	0013 00		tms0		; BST chain 0, >> Shift IR

0034	0014 02 00 1A		ld	cnt,26	; length of IR's + 10, BST chain 0

0035	0017 05		nshfcp	; shift in the ID or BP opcodes

0036	0018 01FD03FCFD03 inf0	.db	$01,$fd,$03,$fc,$fd,$03,$ff,$01,$ff,$03,$00,$03

0037	0024 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0038	0027 01		tms1		; BST chain 0, >> Update IR

0039	0028 01		tms1		; BST chain 0, >> Select DR scan

0040	0029 00		tms0		; BST chain 0, >> Capture DR

0041	002A 00		tms0		; BST chain 0, >> Shift DR

0042	002B 02 00 02		ld	cnt,2	; length of ID (BP, if no ID) registers, BST chain 0

0043	002E 05		nshfcp	; shift out the contents of the ID (BP, if no ID) registers

0044	002F 03 00 03	inf1	.db	$03,$00,$03

0045	0032 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0046	0035 01		tms1		; BST chain 0, >> Update DR

0047	0036 01		tms1		; BST chain 0, >> Select DR scan

0048	0037 01		tms1		; BST chain 0, >> Select IR scan

0049	0038 00		tms0		; BST chain 0, >> Capture IR

0050	0039 00		tms0		; BST chain 0, >> Shift IR

0051	003A 02 00 10		ld	cnt,16	; length of IR's, BST chain 0

0052	003D 05		nshfcp	; shift in the sample/preload opcodes

0053	003E 02FD0302FD03	inf2	.db	$02,$fd,$03,$02,$fd,$03

0054	0044 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0055	0047 01		tms1		; BST chain 0, >> Update IR

0056	0048 01		tms1		; BST chain 0, >> Select DR scan

0057	0049 00		tms0		; BST chain 0, >> Capture DR

0058	004A 01		tms1		; BST chain 0, >> Exit-1 DR

0059	004B 01		tms1		; BST chain 0, >> Update DR

0060	004C 01		tms1		; BST chain 0, >> Select DR scan

0061	004D 01		tms1		; BST chain 0, >> Select IR scan

0062	004E 00		tms0		; BST chain 0, >> Capture IR

0063	004F 00		tms0		; BST chain 0, >> Shift IR

0064	0050 02 00 10		ld	cnt,16	; length of IR's, BST chain 0

0065	0053 05		nshfcp	; shift in the extest opcodes

0066	0054 00FD0300FD03	inf3	.db	$00,$fd,$03,$00,$fd,$03

0067	005A 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0068	005D 01		tms1		; BST chain 0, >> Update IR

0069	005E 01		tms1		; BST chain 0, >> Select DR scan

0070	005F 00		tms0		; BST chain 0, >> Capture DR

0071	0060 00		tms0		; BST chain 0, >> Shift DR

0072	0061 1B		seltap1	; switch to TAP 1

0073	0062 00		tms0		; BST chain 1, >> Run Test / Idle

0074	0063 01		tms1		; BST chain 1, >> Select DR scan

0075	0064 01		tms1		; BST chain 1, >> Select IR scan

0076	0065 00		tms0		; BST chain 1, >> Capture IR

0077	0066 00		tms0		; BST chain 1, >> Shift IR

0078	0067 02 00 12		ld	cnt,18	; length of IR's + 10, BST chain 1

0079	006A 05		nshfcp	; shift in the ID or BP opcodes

0080	006B 01FD03FC01FF	inf4	.db	$01,$fd,$03,$fc,$01,$ff,$03,$00,$03

0081	0074 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0082	0077 01		tms1		; BST chain 1, >> Update DR

0083	0078 01		tms1		; BST chain 1, >> Select DR scan

0084	0079 00		tms0		; BST chain 1, >> Capture DR

0085	007A 00		tms0		; BST chain 1, >> Shift DR

0086	007B 02 00 01		ld	cnt,1	; length of ID (BP, if no ID) registers, BST chain 1

0087	007E 05		nshfcp	; shift out the contents of the ID (BP, if no ID) registers

0088	007F 01 00 01	inf5	.db	$01,$00,$01

0089	0082 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0090	0085 01		tms1		; BST chain 1, >> Update DR

0091	0086 01		tms1		; BST chain 1, >> Select DR scan

0092	0087 01		tms1		; BST chain 1, >> Select IR scan

0093	0088 00		tms0		; BST chain 1, >> Capture IR

0094	0089 00		tms0		; BST chain 1, >> Shift IR

0095	008A 02 00 08		ld	cnt,8	; length of IR's, BST chain 1

0096	008D 05		nshfcp	; shift in the sample/preload opcodes

0097	008E 02 FD 03	inf6	.db	$02,$fd,$03

0098	0091 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0099	0094 01		tms1		; BST chain 1, >> Update IR

0100	0095 01		tms1		; BST chain 1, >> Select DR scan

0101	0096 00		tms0		; BST chain 1, >> Capture DR

0102	0097 01		tms1		; BST chain 1, >> Exit-1 DR

0103	0098 01		tms1		; BST chain 1, >> Update DR

0104	0099 01		tms1		; BST chain 1, >> Select DR scan

0105	009A 01		tms1		; BST chain 1, >> Select IR scan

0106	009B 00		tms0		; BST chain 1, >> Capture IR

0107	009C 00		tms0		; BST chain 1, >> Shift IR

0108	009D 02 00 08		ld	cnt,8	; length of IR's, BST chain 1

0109	00A0 05		nshfcp	; shift in the extest opcodes

0110	00A1 00 FD 03	inf7	.db	$00,$fd,$03

0111	00A4 06 04 01		jpe	theend	; stop the test, if a BST infrastructure fault is found

0112	00A7 01		tms1		; BST chain 1, >> Update DR

0113	00A8 01		tms1		; BST chain 1, >> Select DR scan

0114	00A9 00		tms0		; BST chain 1, >> Capture DR

0115	00AA 00		tms0		; BST chain 1, >> Shift DR

0116	00AB 1A		seltap0	; switch to TAP 0

0117	00AC

0118	00AC	; Sequence for testing the full BST interconnects

0119	00AC

0120	00AC 0B		serflg3	; select error flag 3 (full BST interconnect open faults)

0121	00AD 02 00 24		ld	cnt,36	; length of BST chain 0

0122	00B0 04		nshf		; shift 1st TP (for open testing), no compare

0123	00B1 000000FC03	int0	.db	$00,$00,$00,$fc,$03

0124	00B6 01		tms1		; BST chain 0, >> Update DR

0125	00B7 01		tms1		; BST chain 0, >> Sel. DR scan

0126	00B8 1B		seltap1	; switch to TAP 1

0127	00B9 02 00 12		ld	cnt,18	; length of BST chain 1

0128	00BC 04		nshf		; shift the 1st TP (for open faults) on BST chain 1

0129	00BD 00 03 00	int1	.db	$00,$03,$00

0130	00C0 01		tms1		; BST chain 1, >> Update DR

0131	00C1 01		tms1		; BST chain 1, >> Sel. DR scan

0132	00C2 13		ssa1		; SyncOutA goes high (BST test data applied)

0133	00C3 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0134	00C4 00		tms0		; BST chain 1, >> Capture DR

0135	00C5 00		tms0		; BST chain 1, >> Shift DR

0136	00C6 1A		seltap0	; switch to TAP 0

0137	00C7 00		tms0		; BST chain 0, >> Capture DR

0138	00C8 00		tms0		; BST chain 0, >> Shift DR

0139	00C9 12		ssa0		; SyncOutA goes low (BST test results captured)

0140	00CA 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0141	00CB 02 00 24		ld	cnt,36	; length of BST chain 0

0142	00CE 05		nshfcp	; test pattern for open faults

0143	00CF 000000000000	int2	.db	$00,$00,$00,$00,$00,$00,$0c,$00,$03,$fc,$00,$fc,$03,$00,$0f

0144	00DE 06 04 01		jpe	theend	; stop the test, if a net is found open, or s-a-1

0145	00E1 01		tms1		; BST chain 0, >> Update DR

0146	00E2 01		tms1		; BST chain 0, >> Sel. DR scan

0147	00E3 1B		seltap1	; switch to TAP 1

0148	00E4 02 00 12		ld	cnt,18	; length of BST chain 1

0149	00E7 05		nshfcp	; TP for open faults, BST chain 1

0150	00E8 FF0000030003	int3	.db	$ff,$00,$00,$03,$00,$03,$00,$00,$03

0151	00F1 06 04 01		jpe	theend	; stop the test, if a net is found open, or s-a-1

0152	00F4 01		tms1		; BST chain 1, >> Update DR

0153	00F5 01		tms1		; BST chain 1, >> Sel. DR scan

0154	00F6 13		ssa1		; SyncOutA goes high (BST test data applied)

0155	00F7 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0156	00F8 00		tms0		; BST chain 1, >> Capture DR

0157	00F9 00		tms0		; BST chain 1, >> Shift DR

0158	00FA 1A		seltap0	; switch to TAP 0

0159	00FB 00		tms0		; BST chain 0, >> Capture DR

0160	00FC 00		tms0		; BST chain 0, >> Shift DR

0161	00FD 12		ssa0		; SyncOutA goes low (BST test results captured)

0162	00FE 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0163	00FF 0C		serflg4	; select error flag 4 (full BST interconnect short faults)

0164	0100 02 00 24		ld	cnt,36	; length of BST chain 0

0165	0103 05		nshfcp	; test pattern for short faults

0166	0104 000000000000	int4	.db	$00,$00,$00,$00,$00,$00,$04,$0c,$03,$fc,$fc,$fc,$03,$03,$0f

0167	0113 06 04 01		jpe	theend	; stop the test, if a short fault is found

0168	0116 01		tms1		; BST chain 0, >> Update DR

0169	0117 01		tms1		; BST chain 0, >> Sel. DR scan

0170	0118 1B		seltap1	; switch to TAP 1

0171	0119 02 00 12		ld	cnt,18	; length of BST chain 1

0172	011C 05		nshfcp	; TP for short faults, BST chain 1

0173	011D 55FF00030303	int5	.db	$55,$ff,$00,$03,$03,$03,$00,$00,$03

0174	0126 06 04 01		jpe	theend	; stop the test, if a short fault is found

0175	0129 01		tms1		; BST chain 1, >> Update DR

0176	012A 01		tms1		; BST chain 1, >> Sel. DR scan

0177	012B 13		ssa1		; SyncOutA goes high (BST test data applied)

0178	012C 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0179	012D 00		tms0		; BST chain 1, >> Capture DR

0180	012E 00		tms0		; BST chain 1, >> Shift DR

0181	012F 1A		seltap0	; switch to TAP 0

0182	0130 00		tms0		; BST chain 0, >> Capture DR

0183	0131 00		tms0		; BST chain 0, >> Shift DR

0184	0132 12		ssa0		; SyncOutA goes low (BST test results captured)

0185	0133 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0186	0134 02 00 24		ld	cnt,36	; length of BST chain 0

0187	0137 05		nshfcp	; test pattern for short faults

0188	0138 000000000000	int6	.db	$00,$00,$00,$00,$00,$00,$08,$04,$03,$fc,$54,$fc,$03,$01,$0f

0189	0147 06 04 01		jpe	theend	; stop the test, if a short fault is found

0190	014A 01		tms1		; BST chain 0, >> Update DR

0191	014B 01		tms1		; BST chain 0, >> Sel. DR scan

0192	014C 1B		seltap1	; switch to TAP 1

0193	014D 02 00 12		ld	cnt,18	; length of BST chain 1

0194	0150 05		nshfcp	; TP for short faults, BST chain 1

0195	0151 AA5500030103	int7	.db	$aa,$55,$00,$03,$01,$03,$00,$00,$03

0196	015A 06 04 01		jpe	theend	; stop the test, if a short fault is found

0197	015D 01		tms1		; BST chain 1, >> Update DR

0198	015E 01		tms1		; BST chain 1, >> Sel. DR scan

0199	015F 13		ssa1		; SyncOutA goes high (BST test data applied)

0200	0160 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0201	0161 00		tms0		; BST chain 1, >> Capture DR

0202	0162 00		tms0		; BST chain 1, >> Shift DR

0203	0163 1A		seltap0	; switch to TAP 0

0204	0164 00		tms0		; BST chain 0, >> Capture DR

0205	0165 00		tms0		; BST chain 0, >> Shift DR

0206	0166 12		ssa0		; SyncOutA goes low (BST test results captured)

0207	0167 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0208	0168 02 00 24		ld	cnt,36	; length of BST chain 0

0209	016B 05		nshfcp	; test pattern for short faults

0210	016C 000000000000	int8	.db	$00,$00,$00,$00,$00,$00,$04,$08,$03,$fc,$54,$fc,$03,$01,$0f

0211	017B 06 04 01		jpe	theend	; stop the test, if a short fault is found

0212	017E 01		tms1		; BST chain 0, >> Update DR

0213	017F 01		tms1		; BST chain 0, >> Sel. DR scan

0214	0180 1B		seltap1	; switch to TAP 1

0215	0181 02 00 12		ld	cnt,18	; length of BST chain 1

0216	0184 05		nshfcp	; TP for short faults, BST chain 1

0217	0185 5AAA00030203	int9	.db	$5a,$aa,$00,$03,$02,$03,$00,$00,$03

0218	018E 06 04 01		jpe	theend	; stop the test, if a short fault is found

0219	0191 01		tms1		; BST chain 1, >> Update DR

0220	0192 01		tms1		; BST chain 1, >> Sel. DR scan

0221	0193 13		ssa1		; SyncOutA goes high (BST test data applied)

0222	0194 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0223	0195 00		tms0		; BST chain 1, >> Capture DR

0224	0196 00		tms0		; BST chain 1, >> Shift DR

0225	0197 1A		seltap0	; switch to TAP 0

0226	0198 00		tms0		; BST chain 0, >> Capture DR

0227	0199 00		tms0		; BST chain 0, >> Shift DR

0228	019A 12		ssa0		; SyncOutA goes low (BST test results captured)

0229	019B 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0230	019C 02 00 24		ld	cnt,36	; length of BST chain 0

0231	019F 05		nshfcp	; test pattern for short faults

0232	01A0 000000000000	int10	.db	$00,$00,$00,$00,$00,$00,$04,$04,$03,$fc,$a8,$fc,$03,$01,$0f

0233	01AF 06 04 01		jpe	theend	; stop the test, if a short fault is found

0234	01B2 01		tms1		; BST chain 0, >> Update DR

0235	01B3 01		tms1		; BST chain 0, >> Sel. DR scan

0236	01B4 1B		seltap1	; switch to TAP 1

0237	01B5 02 00 12		ld	cnt,18	; length of BST chain 1

0238	01B8 05		nshfcp	; TP for short faults, BST chain 1

0239	01B9 A55A00030103	int11	.db	$a5,$5a,$00,$03,$01,$03,$00,$00,$03

0240	01C2 06 04 01		jpe	theend	; stop the test, if a short fault is found

0241	01C5 01		tms1		; BST chain 1, >> Update DR

0242	01C6 01		tms1		; BST chain 1, >> Sel. DR scan

0243	01C7 13		ssa1		; SyncOutA goes high (BST test data applied)

0244	01C8 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0245	01C9 00		tms0		; BST chain 1, >> Capture DR

0246	01CA 00		tms0		; BST chain 1, >> Shift DR

0247	01CB 1A		seltap0	; switch to TAP 0

0248	01CC 00		tms0		; BST chain 0, >> Capture DR

0249	01CD 00		tms0		; BST chain 0, >> Shift DR

0250	01CE 12		ssa0		; SyncOutA goes low (BST test results captured)

0251	01CF 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0252	01D0 02 00 24		ld	cnt,36	; length of BST chain 0

0253	01D3 05		nshfcp	; test pattern for short faults

0254	01D4 000000000000	int12	.db	$00,$00,$00,$00,$00,$00,$08,$04,$03,$fc,$68,$fc,$03,$02,$0f

0255	01E3 06 04 01		jpe	theend	; stop the test, if a short fault is found

0256	01E6 01		tms1		; BST chain 0, >> Update DR

0257	01E7 01		tms1		; BST chain 0, >> Sel. DR scan

0258	01E8 1B		seltap1	; switch to TAP 1

0259	01E9 02 00 12		ld	cnt,18	; length of BST chain 1

0260	01EC 05		nshfcp	; TP for short faults, BST chain 1

0261	01ED 66A500030103	int13	.db	$66,$a5,$00,$03,$01,$03,$00,$00,$03

0262	01F6 06 04 01		jpe	theend	; stop the test, if a short fault is found

0263	01F9 01		tms1		; BST chain 1, >> Update DR

0264	01FA 01		tms1		; BST chain 1, >> Sel. DR scan

0265	01FB 13		ssa1		; SyncOutA goes high (BST test data applied)

0266	01FC 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0267	01FD 00		tms0		; BST chain 1, >> Capture DR

0268	01FE 00		tms0		; BST chain 1, >> Shift DR

0269	01FF 1A		seltap0	; switch to TAP 0

0270	0200 00		tms0		; BST chain 0, >> Capture DR

0271	0201 00		tms0		; BST chain 0, >> Shift DR

0272	0202 12		ssa0		; SyncOutA goes low (BST test results captured)

0273	0203 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0274	0204 02 00 24		ld	cnt,36	; length of BST chain 0

0275	0207 05		nshfcp	; test pattern for short faults

0276	0208 000000000000	int14	.db	$00,$00,$00,$00,$00,$00,$08,$08,$03,$fc,$98,$fc,$03,$02,$0f

0277	0217 06 04 01		jpe	theend	; stop the test, if a short fault is found

0278	021A 01		tms1		; BST chain 0, >> Update DR

0279	021B 01		tms1		; BST chain 0, >> Sel. DR scan

0280	021C 1B		seltap1	; switch to TAP 1

0281	021D 02 00 12		ld	cnt,18	; length of BST chain 1

0282	0220 05		nshfcp	; TP for short faults, BST chain 1

0283	0221 996600030203	int15	.db	$99,$66,$00,$03,$02,$03,$00,$00,$03

0284	022A 06 04 01		jpe	theend	; stop the test, if a short fault is found

0285	022D 01		tms1		; BST chain 1, >> Update DR

0286	022E 01		tms1		; BST chain 1, >> Sel. DR scan

0287	022F 13		ssa1		; SyncOutA goes high (BST test data applied)

0288	0230 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0289	0231 00		tms0		; BST chain 1, >> Capture DR

0290	0232 00		tms0		; BST chain 1, >> Shift DR

0291	0233 1A		seltap0	; switch to TAP 0

0292	0234 00		tms0		; BST chain 0, >> Capture DR

0293	0235 00		tms0		; BST chain 0, >> Shift DR

0294	0236 12		ssa0		; SyncOutA goes low (BST test results captured)

0295	0237 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0296	0238 02 00 24		ld	cnt,36	; length of BST chain 0

0297	023B 05		nshfcp	; last test pattern for short faults, BS chain 0

0298	023C 000000000000	int16	.db	$00,$00,$00,$00,$00,$00,$08,$08,$03,$fc,$a4,$fc,$03,$02,$0f

0299	024B 06 04 01		jpe	theend	; stop the test, if a short fault is found

0300	024E 1B		seltap1	; switch to TAP 1

0301	024F 02 00 12		ld	cnt,18	; length of BST chain 1

0302	0252 05		nshfcp	; last TP for short faults, BST chain 1

0303	0253 999900030203	int17	.db	$99,$99,$00,$03,$02,$03,$00,$00,$03

0304	025C 06 04 01		jpe	theend	; stop the test, if a short fault is found

0305	025F 1A		seltap0	; switch to TAP 0

0306	0260

0307	0260	; Sequence for testing the cluster interconnects

0308	0260

0309	0260 0D		serflg5	; select error flag 5 (cluster interconnect faults)

0310	0261 01		tms1		; >> Update DR, BS chain 0

0311	0262 01		tms1		; >> Select DR scan, BS chain 0

0312	0263 00		tms0		; >> Capture DR, BS chain 0

0313	0264 00		tms0		; >> Shift DR, BS chain 0

0314	0265 1B		seltap1	; switch to TAP 1

0315	0266 01		tms1		; >> Update DR, BS chain 1

0316	0267 01		tms1		; >> Select DR scan, BS chain 1

0317	0268 00		tms0		; >> Capture DR, BS chain 1

0318	0269 00		tms0		; >> Shift DR, BS chain 1

0319	026A 1A		seltap0	; switch to TAP 0

0320	026B 02 00 24		ld	cnt,36	; length of BST chain 0

0321	026E 04		nshf		; test pattern for cluster interconnect testing

0322	026F FAFF8C0300	clu0	.db	$fa,$ff,$8c,$03,$00

0323	0274 01		tms1		; BST chain 0, >> Update DR

0324	0275 01		tms1		; BST chain 0, >> Sel. DR scan

0325	0276 1B		seltap1	; switch to TAP 1

0326	0277 02 00 12		ld	cnt,18	; length of BST chain 1

0327	027A 04		nshf		; test pattern for cluster interconnect testing, BS chain 1

0328	027B FF F0 00	clu1	.db	$ff,$f0,$00

0329	027E 01		tms1		; BST chain 1, >> Update DR

0330	027F 01		tms1		; BST chain 1, >> Sel. DR scan

0331	0280 13		ssa1		; SyncOutA goes high (BST test data applied)

0332	0281 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0333	0282 00		tms0		; BST chain 1, >> Capture DR

0334	0283 00		tms0		; BST chain 1, >> Shift DR

0335	0284 1A		seltap0	; switch to TAP 0

0336	0285 00		tms0		; BST chain 0, >> Capture DR

0337	0286 00		tms0		; BST chain 0, >> Shift DR

0338	0287 12		ssa0		; SyncOutA goes low (BST test results captured)

0339	0288 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0340	0289 02 00 24		ld	cnt,36	; length of BST chain 0

0341	028C 05		nshfcp	; test pattern for cluster interconnect testing

0342	028D 11FF00FFF1FF	clu2	.db	$11,$ff,$00,$ff,$f1,$ff,$4c,$ff,$00,$03,$ff,$fc,$00,$0f,$03

0343	029C 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0344	029F 01		tms1		; BST chain 0, >> Update DR

0345	02A0 01		tms1		; BST chain 0, >> Sel. DR scan

0346	02A1 1B		seltap1	; switch to TAP 1

0347	02A2 02 00 12		ld	cnt,18	; length of BST chain 1

0348	02A5 05		nshfcp	; test pattern for cluster interconnect testing, BS chain 1

0349	02A6 FFFF00F0F3F3	clu3	.db	$ff,$ff,$00,$f0,$f3,$f3,$00,$00,$03

0350	02AF 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0351	02B2 01		tms1		; BST chain 1, >> Update DR

0352	02B3 01		tms1		; BST chain 1, >> Sel. DR scan

0353	02B4 13		ssa1		; SyncOutA goes high (BST test data applied)

0354	02B5 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0355	02B6 00		tms0		; BST chain 1, >> Capture DR

0356	02B7 00		tms0		; BST chain 1, >> Shift DR

0357	02B8 1A		seltap0	; switch to TAP 0

0358	02B9 00		tms0		; BST chain 0, >> Capture DR

0359	02BA 00		tms0		; BST chain 0, >> Shift DR

0360	02BB 12		ssa0		; SyncOutA goes low (BST test results captured)

0361	02BC 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0362	02BD 02 00 24		ld	cnt,36	; length of BST chain 0

0363	02C0 05		nshfcp	; test pattern for cluster interconnect testing

0364	02C1 37FF00FFE0FF	clu4	.db	$37,$ff,$00,$ff,$e0,$ff,$3c,$ff,$00,$00,$ff,$fc,$00,$0f,$03

0365	02D0 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0366	02D3 01		tms1		; BST chain 0, >> Update DR

0367	02D4 01		tms1		; BST chain 0, >> Sel. DR scan

0368	02D5 1B		seltap1	; switch to TAP 1

0369	02D6 02 00 12		ld	cnt,18	; length of BST chain 1

0370	02D9 05		nshfcp	; test pattern for cluster interconnect testing, BS chain 1

0371	02DA FFFF00F003F3	clu5	.db	$ff,$ff,$00,$f0,$03,$f3,$00,$00,$03

0372	02E3 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0373	02E6 01		tms1		; BST chain 1, >> Update DR

0374	02E7 01		tms1		; BST chain 1, >> Sel. DR scan

0375	02E8 13		ssa1		; SyncOutA goes high (BST test data applied)

0376	02E9 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0377	02EA 00		tms0		; BST chain 1, >> Capture DR

0378	02EB 00		tms0		; BST chain 1, >> Shift DR

0379	02EC 1A		seltap0	; switch to TAP 0

0380	02ED 00		tms0		; BST chain 0, >> Capture DR

0381	02EE 00		tms0		; BST chain 0, >> Shift DR

0382	02EF 12		ssa0		; SyncOutA goes low (BST test results captured)

0383	02F0 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0384	02F1 02 00 24		ld	cnt,36	; length of BST chain 0

0385	02F4 05		nshfcp	; test pattern for cluster interconnect testing

0386	02F5 D5FF00FF78FF	clu6	.db	$d5,$ff,$00,$ff,$78,$ff,$2c,$ff,$00,$01,$ff,$fc,$00,$0f,$03

0387	0304 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0388	0307 01		tms1		; BST chain 0, >> Update DR

0389	0308 01		tms1		; BST chain 0, >> Sel. DR scan

0390	0309 1B		seltap1	; switch to TAP 1

0391	030A 02 00 12		ld	cnt,18	; length of BST chain 1

0392	030D 05		nshfcp	; test pattern for cluster interconnect testing, BS chain 1

0393	030E FFFF00F003F3	clu7	.db	$ff,$ff,$00,$f0,$03,$f3,$00,$00,$03

0394	0317 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0395	031A 01		tms1		; BST chain 1, >> Update DR

0396	031B 01		tms1		; BST chain 1, >> Sel. DR scan

0397	031C 13		ssa1		; SyncOutA goes high (BST test data applied)

0398	031D 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0399	031E 00		tms0		; BST chain 1, >> Capture DR

0400	031F 00		tms0		; BST chain 1, >> Shift DR

0401	0320 1A		seltap0	; switch to TAP 0

0402	0321 00		tms0		; BST chain 0, >> Capture DR

0403	0322 00		tms0		; BST chain 0, >> Shift DR

0404	0323 12		ssa0		; SyncOutA goes low (BST test results captured)

0405	0324 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0406	0325 02 00 24		ld	cnt,36	; length of BST chain 0

0407	0328 05		nshfcp	; test pattern for cluster interconnect testing

0408	0329 22FF00FFD2FF	clu8	.db	$22,$ff,$00,$ff,$d2,$ff,$2c,$ff,$00,$02,$ff,$fc,$00,$0f,$03

0409	0338 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0410	033B 01		tms1		; BST chain 0, >> Update DR

0411	033C 01		tms1		; BST chain 0, >> Sel. DR scan

0412	033D 1B		seltap1	; switch to TAP 1

0413	033E 02 00 12		ld	cnt,18	; length of BST chain 1

0414	0341 05		nshfcp	; test pattern for cluster interconnect testing, BS chain 1

0415	0342 FFFF00F0F3F3	clu9	.db	$ff,$ff,$00,$f0,$f3,$f3,$00,$00,$03

0416	034B 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0417	034E 01		tms1		; BST chain 1, >> Update DR

0418	034F 01		tms1		; BST chain 1, >> Sel. DR scan

0419	0350 13		ssa1		; SyncOutA goes high (BST test data applied)

0420	0351 15		wsa1		; wait for SyncInpA to go high (external test data applied)

0421	0352 00		tms0		; BST chain 1, >> Capture DR

0422	0353 00		tms0		; BST chain 1, >> Shift DR

0423	0354 1A		seltap0	; switch to TAP 0

0424	0355 00		tms0		; BST chain 0, >> Capture DR

0425	0356 00		tms0		; BST chain 0, >> Shift DR

0426	0357 12		ssa0		; SyncOutA goes low (BST test results captured)

0427	0358 14		wsa0		; wait for SyncInpA to go low (ext. test results captured)

0428	0359 02 00 24		ld	cnt,36	; length of BST chain 0

0429	035C 05		nshfcp	; last test pattern for cluster testing, BS chain 0

0430	035D 22FF00FFB4FF	clu10	.db	$22,$ff,$00,$ff,$b4,$ff,$2c,$ff,$00,$02,$ff,$fc,$00,$0f,$03

0431	036C 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0432	036F 1B		seltap1	; switch to TAP 1

0433	0370 02 00 12		ld	cnt,18	; length of BST chain 1

0434	0373 05		nshfcp	; last TP for cluster interconnect testing, BST chain 1

0435	0374 FFFF00F003F3	clu11	.db	$ff,$ff,$00,$f0,$03,$f3,$00,$00,$03

0436	037D 06 04 01		jpe	theend	; stop the test, if a cluster fault is detected

0437	0380 1A		seltap0	; switch to TAP 0

0438	0381

0439	0381	; Sequence for internal test of components

0440	0381

0441	0381 0F		serflg7	; select error flag 7 (faults detected during internal test)

0442	0382 1A		seltap0	; switch to BS chain 0

0443	0383 01		tms1		; >> Update DR, BS chain 0

0444	0384 01		tms1		; >> Select DR scan, BS chain 0

0445	0385 01		tms1		; >> Select IR scan, BS chain 0

0446	0386 00		tms0		; >> Capture IR, BS chain 0

0447	0387 00		tms0		; >> Shift IR, BS chain 0

0448	0388 02 00 10		ld	cnt,16	; length of the IRs in scan chain 0

0449	038B 04		nshf		; shift in the intest/bypass opcodes

0450	038C 00 00	com0	.db	$00,$00

0451	038E 01		tms1		; >> Update IR, BS chain 0

0452	038F 01		tms1		; >> Select DR scan, BS chain 0

0453	0390 00		tms0		; >> Capture DR, BS chain 0

0454	0391 00		tms0		; >> Shift DR, BS chain 0

0455	0392 02 00 24		ld	cnt,36	; length of scan chain 0

0456	0395 04		nshf		; shift the 1st TP for internal test (no compare)

0457	0396 FF55FF570D	com1	.db	$ff,$55,$ff,$57,$0d

0458	039B 01		tms1		; >> Update DR, BS chain 0

0459	039C 01		tms1		; >> Select DR scan, BS chain 0

0460	039D 00		tms0		; >> Capture DR, BS chain 0

0461	039E 00		tms0		; >> Shift DR, BS chain 0

0462	039F 02 00 24		ld	cnt,36	; length of scan chain 0

0463	03A2 05		nshfcp	; test pattern for internal test of components

0464	03A3 0055FFAAFF00	com2	.db	$00,$55,$ff,$aa,$ff,$00,$03,$57,$fc,$a8,$fd,$03,$0e,$0f,$00

0465	03B2 01		tms1		; >> Update DR, BS chain 0

0466	03B3 01		tms1		; >> Select DR scan, BS chain 0

0467	03B4 00		tms0		; >> Capture DR, BS chain 0

0468	03B5 00		tms0		; >> Shift DR, BS chain 0

0469	03B6 02 00 24		ld	cnt,36	; length of scan chain 0

0470	03B9 05		nshfcp	; last test pattern for internal test of components

0471	03BA 00AAFFAA0000	com3	.db	$00,$aa,$ff,$aa,$00,$00,$03,$a8,$fc,$a8,$02,$03,$0e,$00,$00

0472	03C9 1B		seltap1	; switch to BS chain 1

0473	03CA 01		tms1		; >> Update DR, BS chain 1

0474	03CB 01		tms1		; >> Select DR scan, BS chain 1

0475	03CC 01		tms1		; >> Select IR scan, BS chain 1

0476	03CD 00		tms0		; >> Capture IR, BS chain 1

0477	03CE 00		tms0		; >> Shift IR, BS chain 1

0478	03CF 02 00 08		ld	cnt,8	; length of the IRs in scan chain 1

0479	03D2 04		nshf		; shift in the intest/bypass opcodes

0480	03D3 00	com4	.db	$00

0481	03D4 01		tms1		; >> Update IR, BS chain 1

0482	03D5 01		tms1		; >> Select DR scan, BS chain 1

0483	03D6 00		tms0		; >> Capture DR, BS chain 1

0484	03D7 00		tms0		; >> Shift DR, BS chain 1

0485	03D8 02 00 12		ld	cnt,18	; length of scan chain 1

0486	03DB 04		nshf		; shift the 1st TP for internal test (no compare)

0487	03DC FF 55 03	com5	.db	$ff,$55,$03

0488	03DF 01		tms1		; >> Update DR, BS chain 1

0489	03E0 01		tms1		; >> Select DR scan, BS chain 1

0490	03E1 00		tms0		; >> Capture DR, BS chain 1

0491	03E2 00		tms0		; >> Shift DR, BS chain 1

0492	03E3 02 00 12		ld	cnt,18	; length of scan chain 1

0493	03E6 05		nshfcp	; test pattern for internal test of components

0494	03E7 0055FFAAFF00	com6	.db	$00,$55,$ff,$aa,$ff,$00,$03,$03,$00

0495	03F0 01		tms1		; >> Update DR, BS chain 1

0496	03F1 01		tms1		; >> Select DR scan, BS chain 1

0497	03F2 00		tms0		; >> Capture DR, BS chain 1

0498	03F3 00		tms0		; >> Shift DR, BS chain 1

0499	03F4 02 00 12		ld	cnt,18	; length of scan chain 1

0500	03F7 05		nshfcp	; last test pattern for internal test of components

0501	03F8 00AAFFAA0000	com7	.db	$00,$aa,$ff,$aa,$00,$00,$03,$00,$00

0502	0401

0503	0401	;	End of test program.

0504	0401

0505	0401 1A	theend	seltap0	; (switch to) TAP 0

0506	0402 01		tms1

0507	0403 01		tms1

0508	0404 01		tms1

0509	0405 01		tms1

0510	0406 01		tms1		; reset sequence for TAP 0

0511	0407 1B		seltap1	; switch to TAP 1

0512	0408 01		tms1

0513	0409 01		tms1

0514	040A 01		tms1

0515	040B 01		tms1

0516	040C 01		tms1		; reset sequence for TAP 1

0517	040D 10		halt		; this is the end

0518	040E

0519	040E		.end

0520	040E

0521	040E

0522	040E

0523	8000	primary	.org	8000h

0524	8000

0525	8000	; pattern number 0

0526	8000 00	pi0	.db	$00	; primary inputs

0527	8001 FF	cs0	.db	$ff	; control signal

0528	8002 00	po0	.db	$00	; expected values at primary outputs

0529	8003 FF	pom0	.db	$ff	; primary outputs mask

0530	8004	; pattern number 1

0531	8004 FF	pi1	.db	$ff	; primary inputs

0532	8005 FF	cs1	.db	$ff	; control signal

0533	8006 FF	po1	.db	$ff	; expected values at primary outputs

0534	8007 FF	pom1	.db	$ff	; primary outputs mask

0535	8008	; pattern number 2

0536	8008 AA	pi2	.db	$aa	; primary inputs

0537	8009 FF	cs2	.db	$ff	; control signal

0538	800A AA	po2	.db	$aa	; expected values at primary outputs

0539	800B FF	pom2	.db	$ff	; primary outputs mask

0540	800C	; pattern number 3

0541	800C AA	pi3	.db	$aa	; primary inputs

0542	800D FF	cs3	.db	$ff	; control signal

0543	800E 55	po3	.db	$55	; expected values at primary outputs

0544	800F FF	pom3	.db	$ff	; primary outputs mask

0545	8010	; pattern number 4

0546	8010 56	pi4	.db	$56	; primary inputs

0547	8011 FF	cs4	.db	$ff	; control signal

0548	8012 5A	po4	.db	$5a	; expected values at primary outputs

0549	8013 FF	pom4	.db	$ff	; primary outputs mask

0550	8014	; pattern number 5

0551	8014 59	pi5	.db	$59	; primary inputs

0552	8015 FF	cs5	.db	$ff	; control signal

0553	8016 A5	po5	.db	$a5	; expected values at primary outputs

0554	8017 FF	pom5	.db	$ff	; primary outputs mask

0555	8018	; pattern number 6

0556	8018 65	pi6	.db	$65	; primary inputs

0557	8019 FF	cs6	.db	$ff	; control signal

0558	801A 66	po6	.db	$66	; expected values at primary outputs

0559	801B FF	pom6	.db	$ff	; primary outputs mask

0560	801C	; pattern number 7

0561	801C 95	pi7	.db	$95	; primary inputs

0562	801D FF	cs7	.db	$ff	; control signal

0563	801E 99	po7	.db	$99	; expected values at primary outputs

0564	801F FF	pom7	.db	$ff	; primary outputs mask

0565	8020	; pattern number 8

0566	8020 FF	pi8	.db	$ff	; primary inputs

0567	8021 FF	cs8	.db	$ff	; control signal

0568	8022 FF	po8	.db	$ff	; expected values at primary outputs

0569	8023 FF	pom8	.db	$ff	; primary outputs mask

0570	8024	; pattern number 9

0571	8024 FF	pi9	.db	$ff	; primary inputs

0572	8025 FF	cs9	.db	$ff	; control signal

0573	8026 FF	po9	.db	$ff	; expected values at primary outputs

0574	8027 FF	pom9	.db	$ff	; primary outputs mask

0575	8028	; pattern number 10

0576	8028 FF	pi10	.db	$ff	; primary inputs

0577	8029 FF	cs10	.db	$ff	; control signal

0578	802A FF	po10	.db	$ff	; expected values at primary outputs

0579	802B FF	pom10	.db	$ff	; primary outputs mask

0580	802C	; pattern number 11

0581	802C FF	pi11	.db	$ff	; primary inputs

0582	802D FF	cs11	.db	$ff	; control signal

0583	802E FF	po11	.db	$ff	; expected values at primary outputs

0584	802F FF	pom11	.db	$ff	; primary outputs mask

0585	8030	; pattern number 12

0586	8030 FF	pi12	.db	$ff	; primary inputs

0587	8031 FF	cs12	.db	$ff	; control signal

0588	8032 FF	po12	.db	$ff	; expected values at primary outputs

0589	8033 FF	pom12	.db	$ff	; primary outputs mask

0590	8034	.end

0591	8034

tasm: Number of errors = 0

5.	A case study of test program generation

5.1	The demonstration board	5.1

5.2	The information required for test program generation	5.2

5.2.1	Integrity check of the BS infrastructure	5.3

5.2.2	Full-BS interconnects test	5.3

5.2.3	Non-BS clusters test	5.6

5.2.4	Components test	5.9

5.3	The test vectors	5.10

5.3.1	Test vectors for short-circuit detection in full-BS interconnects	5.11

5.3.2	The complete set of serialised test vectors, expected responses and mask data	5.11

5.4	The test program	5.21

†	 Recall that in full-BS interconnects all pins are either BS pins, primary I/O pins or VCC / Gnd connections.

†	 The original form of the self-diagnosis algorithm was published by W. T. Cheng, J. L. Lewandowski and E. Wu in the Proceedings of the IEEE International Test Conference, 1990, pp. 565-571, in a paper entitled “Diagnosis for Wiring Interconnects”.